3GPP TSG-RAN WG2 Meeting #112 e		R2-2010215

Online, Nov 2nd – 13th 2020

Source:	vivo

Title:          Discussion reliability for RRC_CONNECTED UEs

Agenda Item:	8.1.2.1

Document for:	Discussion and Decision

- Introduction

In the RAN#86 meeting, the work item of NR support of Multicast and Broadcast Services has been approved [1]. The followings have been identified as the objectives related to the physical layer of the work item.

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]: - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2] - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception. Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided. [RAN1, RAN2]

In the RAN2#111-e meeting, an email thread was initiated for MBS L2 architecture which includes reliability in connection mode. During the email discussion, most of the companies agree to not support RLC AM for PTM. Also in RAN1 102 e, the following agreement was made for HARQ feedback that it should be supported for MBS, but the details are FFS[2].

Agreements: For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this. - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based. - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

In this contribution, we will make discussions on the mechanisms to improve reliability for RRC_CONNECTED UEs.

- Discussion

  - L2 reliability

There’re many methodologies for L2 reliability, for example, PDCP retransmission, duplication, and supporting RLC AM mode which being discussed in email Post111-e904[3]. The mechanisms of all these methods are used for different use cases.

    - PTP only mode

- PDCP

Since HARQ feedback was agreed to introduce, in-order delivery function in a higher layer is needed. When PTP transmission mode, the need of reordering and in-order delivery function in PDCP for MBS RB is the same as legacy unicast based transmission. In the email discussion of RAN2 111e, a total of 23 companies have given their views and they all agreed to support reordering and in-order delivery for PDCP.

Furthermore, PDCP status reporting and retransmission can help satisfy the reliability requirement. And 17 companies were agreed that PDCP status reporting and retransmission are needed for NR MBS at least in the case of mobility.

Therefore, we propose that the reordering and in-order delivery function in PDCP is supported for NR MBS. In this way, PDCP status reporting and retransmission are needed for NR MBS at least in the case of mobility.

Proposal 1: For NR MBS in PTP, reordering and in-order delivery function in PDCP is supported, and PDCP status report and retransmission are supported only for handover/bearer type change of AM bearer.

- RLC

RLC transmission mode is also discussed in the email discussion, and the conclusion is suggested to the next meeting.

Table 1 support of RLC mode for PTP and PTM

| Support of RLC mode | PTP | PTM |
| --- | --- | --- |
| RLC-AM | Y | N |
| RLC-UM | Y | Y |
| RLC-TM | N | N |

Since PTP is more like a legacy unicast DRB,it is natural that RLC-AM and RLC-UM are supported for PTP. Firstly PTP transmission should be the same as the current unicast from the RLC point of view. Secondly, it is necessary for reliability transmission. But for TM mode because there is no segmentation with TM mode and is not efficient for scheduling. The use case is unclear, and we agree not to support TM mode for PTP.

The behavior of PDCP when RLC mode is chosen for AM or UM should be the same for unicast. Whether to choose AM or UM mode is based on the reliability requirement.

Proposal 2: For NR MBS in PTP, RLC-AM and RLC-UM mode are supported and RLC-TM is not supported.

    - PTM only mode

- PDCP

For PTM only mode, as it is difficult to design a feedback mechanism for a PTM bearer, so it is not necessary to support PDCP status report and retransmission. We also have discussed to take PDCP as an anchor when mobility and PTP/PTM switching and to support the maintenance of PDCP SNs and duplication detection in the last meeting. Means if we want to support status report and retransmission, another PTP leg is needed.

Proposal 3: For NR MBS in PTM only mode, reordering and in-order delivery function in PDCP is supported. And PDCP status report and retransmission are not supported.

- RLC

Take the result of table1 that most of the companies prefer to only support RLC-UM mode for PTM. The reason why TM mode is not supported has been discussed in PTP mode, so we only give our analysis on why AM mode is not needed for PTM.

Firstly, the reason of supporting AM mode is only for reliability. But PDCP functions can provide reliability for MBS and we also have HARQ feedback. Furthermore, if status report and retransmission is needed, a AM PTP leg is preferable than a AM PTM leg.

Secondly, to support AM mode for PTM is very complicated. About 17 companies share the same view to not supported for PTM AM of NR MBS in the email discussion. Because if RLC AM is used for a PTM RLC bearer, how to transmit the RLC status report and retransmission data is complicated. One possible way is to transmit the RLC status report and retransmit data through another leg as below.

Some companies argued that in NR MBS, some use cases require high reliability and relaxed latency such as V2X applications which require 50 ms E2E latency and up to 99.9999% reliability [3]. So not support RLC AM couldn’t match the requirement. But In Rel-15/16 even for URLLC services such as V2X, the physical layer itself which relies on retransmission and repetition can already meet the requirement of ultra-high reliability. It is the same for MBS. We don’t need to do a special enhancement on reliability than the URLLC case for multicast service.

Base on the above analysis, we already agree to introduce PDCP and HARQ feedback in R17 MBS to help reliability and RLC AM PTM can make little benefit to improve the performance and make it even triple complicated and lost the benefit of PTM. So we decide not to support RLC AM for PTM of NR MBS.

Proposal 4: For NR MBS in PTM, only RLC-UM mode is supported.

In this way, retransmission is only considered by L1 HARQ when PTM only mode is configured.

    - PTP+PTM mode

It has been discussed that what kind of PDCP and RLC functions should be supported for PTP and PTM individually. And it is said status report and retransmission are at least needed when PTP and PTM switching and for mobility use cases.

Figure 1 The case of PTM+PTP

Figure 1 gives an implementation example of PTM plus PTP. When PTM/PTP switching or mobility happens, PTP leg can be used to send PDCP status report and receive retransmission for re-ordering and in-order delivery based on continuous PDCP SN.

Because we have two legs for transmission, it is possible to support common RLC mode and potential different RLC mode for PTP leg and PTM leg.

Proposal 5: PDCP can be taken as an anchor to support both PTP leg and PTM leg configured simultaneously within the same MRB.

PTP and PTM can be configured simultaneously, the two legs of the transmission mode can be configured for different MRB or for the same MRB. If they are configured with different MRB, then two sessions with different QoS can be transmitted simultaneously. This makes the service diversity. If the same MRB is configured for both PTM and PTP, then PTP can be a supplementary transmission for PTM to gain a higher reliability. For example, PTP can be used to send status report and receive retransmission data in poor signal receiving coverage and to support continuity during PTP/PTM switch or HO cases.

From the previous analysis, we could see for PTP, both RLC UM and RLC AM can be chosen. And for PTM, only UM mode is supported. Then two types of combinations are supported.

Table 2 Combination of RLC mode for PTM and PTP leg configured for the same MRB

| PTP leg | PTM leg |
| --- | --- |
| RLC-AM | RLC-UM |
| RLC-UM | RLC-UM |

If both RLC-UM mode is chosen for PTM and PTP, then RLC mode is the same for the two legs, UE doesn’t need RLC status report and doesn’t need RLC retransmission. The PTP leg is only used for double insurance for data transmission, when mobility happens and PDCP retransmission is needed, the PTP leg can be used for PDCP retransmission.

If PTM is configured with RLC-UM and PTM with AM, PTM has no RLC retransmission. PTP can support PDCP retransmission as above. In addition. PTP AM mode can support the RLC status report and provide RLC retransmission to UE for higher reliability requirements.

Proposal 6: The RLC mode of PTP leg and PTM leg in a same MRB can be configured to both UM or potential PTP AM and PTM UM.

Proposal 7: The PTP leg can be used to send status report and receive retransmission data during switching or HO case.

One example can be:

- UE is receiving MBS through PTM. (One leg)

- The measurement report is sent and the network decides to trigger handover.

- PTP leg is configured to UE with RLC AM mode, then the HO command is sent to UE.

- After handover complete, PDCP checks the SN continuity and sends the PDCP status report to the network for retransmission.

- RLC of PTP leg will send the RLC status report to gNB and request RLC retransmission.

- PDCP does re-ordering for MBS data receiving.

- After all the missing data are transmitted, the PTP leg can be released.

  - L1 reliability

- Repetition without HARQ ACK/NACK

It is network implementation that the network can choose to repeat data transmission to improve reliability. In this way, the same data would be transmitted multiple times and the redundancy data should be discarded in the MAC layer.

Proposal 8: It is gNB implementation for MBS data repetition.

- Multiplexing

If multiple QoS flows of an MBS session can be mapped to different RBs and only one G-RNTI is allocated for an MBS session, multiplexing among logical channels should be supported.

Proposal 9: Multiplexing of MAC should be supported by NR MBS if multiple QoS flow of an MBS session mapped to different RBs is supported.

- ACK/NACK

There are two supported HARQ-ACK feedback modes that can be considered for MBS service, i.e., ACK/NACK based feedback and NACK only based feedback.

- Option 1: dedicated resource for ACK/NACK based feedback

  - For the ACK/NACK based feedback, a UE will generate ACK or NACK for a TB based on the TB decoding result, and the PUCCH resources for HARQ-ACK feedback needs to be orthogonal among different UEs. gNB can fully get the PDSCH transmission states for each UE. Therefore, it is possible that MBS PDSCH retransmission can be based on unicast PDSCH per UE basis.

- Option 2: common resource for NACK only based feedback

  - For the NACK only mode, only when a TB is decoded fail, a UE needs to transmit a NACK. For this feedback mode, the PUCCH resources for HARQ-ACK feedback among different UEs can be group common. Thus, it can be a benefit from PUCCH resource consumption. However, gNB is unable to know which UEs or how many UEs are transmitting NACK due to the group common resource. Thus, it is not possible to support MBS PDSCH retransmission through unicast PDSCH. gNB can’t tell it is ACK or DTX even when the feedback resource is UE-specific. In addition, it will bring some additional issues when considering multiplexing between NACK for groupcast PDSCH and other UCI, such as HARQ-ACK for unicast PDSCH, CSI, SR, or multiplexing with PUSCH. Anyway NACK only feedback is a good method for multicast implementation and it is necessary for PTM.

Proposal 10: Both ACK/NACK feedback and NACK only feedback may be supported by PTM. And legacy unicast HARQ mechanism can be baseline for PTP leg. Details are up to RAN1.

It is left for gNB implementation that when NACK receiving is high or retransmission payload is heavy, to improve reliability and efficiency, gNB can decide to change PTM to PTP. Therefore, HARQ feedback can be a considering element to help the network decide PTM/PTP switching. And HARQ feedback should be semi-static configured.

CSI feedback is another effective way to improve the reliability of downlink transmission. In NR, different CSI feedback types are supported, i.e., periodic CSI report on PUCCH, semi-persistent CSI report on PUCCH, semi-persistent CSI report on PUSCH, and a-periodic CSI report on PUSCH. The configuration of CSI measurement and report resource is very flexible. From our perspective, we think whether the current CSI mechanism can be applied to MBS directly for connection UE should be discussed by RAN1

Proposal 11: Whether CSI-RS configuration and CSI feedback schemes can be applied for MBS should be discussed by RAN1.

- Conclusion

Based on the discussion, we observe and propose the following:

Proposal 1: For NR MBS in PTP, reordering and in-order delivery function in PDCP is supported, and PDCP status report and retransmission are supported only for handover/bearer type change of AM bearer.

Proposal 2: For NR MBS in PTP, RLC-AM and RLC-UM mode are supported and RLC-TM is not supported.

Proposal 3: For NR MBS in PTM only mode, reordering and in-order delivery function in PDCP is supported. And PDCP status report and retransmission are not supported.

Proposal 4: For NR MBS in PTM, only RLC-UM mode is supported.

Proposal 5: PDCP can be taken as an anchor to support both PTP leg and PTM leg configured simultaneously within the same MRB.

Proposal 6: The RLC mode of PTP leg and PTM leg in a same MRB can be configured to both UM or potential PTP AM and PTM UM.

Proposal 7: The PTP leg can be used to send status report and receive retransmission data during switching or HO case.

Proposal 8: It is gNB implementation for MBS data repetition.

Proposal 9: Multiplexing of MAC should be supported by NR MBS if multiple QoS flow of an MBS session mapped to different RBs is supported.

Proposal 10: Both ACK/NACK feedback and NACK only feedback may be supported by PTM. And legacy unicast HARQ mechanism can be baseline for PTP leg. Details are up to RAN1.

Proposal 11: Whether CSI-RS configuration and CSI feedback schemes can be applied for MBS should be discussed by RAN1.

- Reference

- P-193248, ‘New Work Item on NR support of Multicast and Broadcast Services’, Huawei

- Minutes_report_RAN1#102-e_v020

- Summary of Email discussion Post111-e904 MBS L2 Architecture

- R1-1809720 LS on Prioritised Use Cases and Requirements for consideration in Rel-16 NR-V2X, 5GAA WG1