3GPP TSG-RAN WG2 Meeting #112 electronic							    	R2-2010219

E-Meeting, 2nd Nov – 13th Nov, 2020

Source: 	vivo

Title:  	Discussion on Idle and Inactive mode UEs

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# 1. Introduction

In the RAN2#111e meeting [1], the observation related to RRC_IDLE/ RRC_INACTIVE state is listed below,

- Chair observations: Many proposals to reuse (to significant extent or even 100%) LTE SC-PTM for Idle/Inactive for NR. Some companies suggest to do control etc in connected also for Idle/Inactive delivery.

In this contribution, we would like to provide our consideration on idle and inactive mode UEs in terms of MBS configuration and mobility issues.

# 2. Discussion

  - 2.1 MBS Configuration

According to RAN2#111e post 906 email discussion [2], the majority of companies has reached the following views(proposals and observations),

- broadcast service and some multicast services (e.g., multicast services with the low-reliability requirement) can be received by UE while it is in RRC Idle/Inactive mode.

- Solution B (or B-variant) is selected for the reception of broadcast services and some multicast services (e.g., multicast services with the low-reliability requirement) while UE is in RRC Idle/Inactive mode.

- FFS on the down selection between B and B-variant.

- Solution B: Use the SC-PTM solution as the baseline, including the following characteristics,

- A limited amount of MBS control information is provided on e.g. BCCH, to indicate how to acquire the MBS control channel, e.g. MCCH;

- Most MBS control information is provided on the MBS control channel, e.g. MCCH;

- The MBS control channel carries a message to indicate the MBMS related information;

- MBS radio bearers are transmitted on respective MBS traffic channel, e.g. MTCH(s);

- A notification mechanism is used to announce the change of MBS control information.

- Solution B-variant: Use the variant of SC-PTM solution as the baseline, including the following characteristics,

- MBS control information is provided on the broadcast channel, e.g. BCCH;

- MBS radio bearers are transmitted on respective MBS traffic channel, e.g. MTCH(s);

- A notification mechanism is used to announce the change of MBS control information.

Besides, regarding the enhancements of solution B for NR MBS, many companies have provided some issues and corresponding solutions, and the majority of companies reached the view that enhancements could be considered after solution B is selected. Some examples of the issues are listed below,

- UE may only be interested in some of the MBS services, but in the solution B reusing LTE, UE in RRC_IDLE/RRC_INACTIVE state will need to wake up and receive the updated SC-MCCH control information blindly. Therefore, the enhancement is that UE can find out whether the interested MBS service has changed to save UE’s power consumption.

- SIB and MCCH signalling is periodically broadcasted, besides it will be broadcasted even though there are not UEs interested in the MBS service. Hence, it will lead to signalling overhead.

- SIB change is notified by the short message associated with P-RNTI in NR to all UEs, so the legacy UEs not interested in MBS may be impacted by MBS-SI(e.g., SIB20) change notifications.

- There are diverse MBS services with different requirements in NR. The modification cycle and repetition cycle of solution B reusing LTE is the same, so it is not flexible enough to support diverse NR MBS application transmission.

Concerning the specific details and enhancements of solution B-variant, considering the only difference between solution B and solution B-variant is that MCCH is no longer used and the content in MCCH is moved to SIB. Therefore, in our views, all issues in solution B above also exist in solution B-variant. Besides, some issues may be severer. For example, the SIB carrying all MBS control information in solution B-variant changes more frequently than SIB (e.g., SIB 20) only carrying the scheduling information of MCCH in solution B, so the legacy UEs not interested in MBS will be more severely impacted by MBS-SI change notifications. Besides, we need to design more new technical schemes(e.g., absolutely new notification mechanisms used to announce the change of MBS control information), which increases more discussions and workloads than following the existed LTE mechanism in solution B. Hence, we propose the following observation and proposal,

Observation 1: Solution B-variant (i.e. all MBS control information is provided by broadcast channel) may have more complexity and workloads.

Proposal 1: Solution B (i.e. most MBS control information is provided by MBS control channel) is suggested as the baseline for MBS service reception by UE while it is in RRC Idle/Inactive mode.

  - 2.2 Mobility

Mobility is a quite vital issue for idle and inactive mode UEs all the time. Concerning the MBS services, to achieve a better user experience, the service continuity for idle/inactive UEs should be supported in NR as that in LTE. Therefore, we have the following proposal,

Proposal 2: The service continuity for RRC_IDLE/ RRC_INACTIVE UEs should be supported in NR MBS.

Regarding the specific service continuity information, the table below summarizes LTE’s scheme.

|  | MBMS | SC-PTM |
| --- | --- | --- |
| Frequency level | 1) Condition 1 SIB15+USD: SAI matching; or The frequency is included in the USD. ; and 2) Condition 2 SIB13 is broadcasted in the reselected cell. | 1) Condition 1 SIB15+USD: SAI matching; or The frequency is included in the USD. ; and 2) Condition 2 SIB20 is broadcasted in the reselected cell. |
| Cell level | \ | SC-MCCH:scptmNeighbourCellList +sc-mtch-neighbourCell in the SC-MTCH-InfoList |

Concerning the NR MBS service continuity information for idle and inactive mode UEs, in our opinions, the main ideas in LTE can be followed with some slight modifications originated from the difference between LTE MBMS/SC-PTM and NR MBS below.

To begin with, the multicast area for the specific multicast service in LTE is fixed and projected in advance while that in NR MBS may be dynamic. Therefore, the SAI concept may no longer exist and the MBS service identifier (e.g. TMGI) can be directly used in the service continuity information.

Next, according to RAN2#111e post 906 email discussion, NR MBS service can be deployed on a frequency basis and/or on a cell basis. In our view, it can include two practical situations. In the first situation, the MBS service is relevant with the frequency (i.e. the MBS service is provided only per frequency), but not all cells belonging to this frequency will provide this MBS service and different cells can have different scheduling configurations. In the second situation, the MBS service is only relevant with the cells (i.e. the MBS service is provided only per cell, for example, cells belonging to different frequencies), and different cells can also have different scheduling configurations. Therefore, the specific granularity of the service continuity information content can be different due to the different deployed granularity analyzed above.

Therefore, based on two analyses above, we have the following proposal,

Proposal 3: The network can provide the MBS service information (e.g. TMGI) of the neighbor frequencies as the service continuity information, and FFS for the MBS service information of the neighbor cells.

As for the cell reselection procedure, although the NR MBS service may be deployed per a cell basis and not relevant with the frequency, the legacy reselection frequency priority handling and the strongest cell belonging to the highest priority frequency should still be adopted to guarantee the unicast reception quality of non-MBS service(e.g. the downlink call) which may be more important than MBS service. Besides, to achieve as good as possible MBS service reception, the frequencies providing the MBS service should be set to the highest priority as that in LTE. Therefore, we have the proposal below,

Proposal 4: The RRC_IDLE/ RRC_INACTIVE UEs should be able to prioritize the frequency providing its interested MBS service during the cell reselection procedure.

# 3. Conclusions

Based on the analysis given above, we have the following observations and proposals:

Observation 1: Solution B-variant (i.e. all MBS control information is provided by broadcast channel) may have more complexity and workloads.

Proposal 1: Solution B (i.e. most MBS control information is provided by MBS control channel) is suggested as the baseline for MBS service reception by UE while it is in RRC Idle/Inactive mode.

Proposal 2: The service continuity for RRC_IDLE/ RRC_INACTIVE UEs should be supported in NR MBS.

Proposal 3: The network can provide the MBS service information (e.g.,TMGI) of the neighbor frequencies as the service continuity information, and FFS for the MBS service information of the neighbor cells.

Proposal 4: The RRC_IDLE/ RRC_INACTIVE UEs should be able to prioritize the frequency providing its interested MBS service during the cell reselection procedure.

# 4. Reference

[1] 3GPP, “Report of 3GPP TSG RAN2#111-e meeting, Online”.

[2] 3GPP RAN2#112e, “Summary of Email Discussion [Post111-e][906][MBS] Idle mode support”, CATT.