3GPP TSG-RAN WG2 Meeting #112e	Tdoc R2-2010247

Electronic meeting, November 2nd – 13th 2020

Agenda Item:	9.2.2

Source:	Ericsson

Title:	Applicability of NR NTN to NB-IoT/LTE-M UEs that support NTN

Document for:	Discussion, Decision

# 1	Introduction

A SI to determine and evaluate the minimum necessary specifications to introduce NB-IoT/LTE-M support for  non-terrestrial networks (NTN) was approved at RAN#86 [1]. The following objectives should be considered:

- to identify scenarios applicable to NB-IoT/LTE-M

to study and recommend necessary changes to support NB-IoT and LTE-M over NTN for the scenarios identified with the intention to use the outcome of the study performed for NR NTN captured in [2].

In this contribution we highlight the differences between NR and NB-IoT/eMTC with respect to a set of key aspects captured in [3], and determine the need for recommendations that may be different with respect to what has been captured for NR..

# 2	User Plane

## 2.1	Random Access

The random access in LTE works in a similar way as 4-step RA in NR. In LTE there is no 2-step RA that is useful for avoiding one round trip time (RTT) during RA procedure.

The same adaptations recommended for NTN regarding 4-step RA in NR can be considered for 4-step RA in NB-IoT/eMTC, that is

- The UE estimates the RTT and uses as timing advance (TA) for the transmission of Msg1

- When Msg2 is received, the UE updates the timing advance using the TAC with respect to the TA used for the transmission of Msg1.

- The UE informs eNB of the current TA in Msg3 or in later messages

The last point is for adapting the scheduling of Msg3 in the eNB; it may not be necessary if the delay can be considered as acceptable if the eNB assumes that each UE has the worst case RTT.

As the TA is much larger than in TNs, there may be a need to add an offset to when a grant for Msg3 is valid, similar to the case for scheduling Msg3 in NR NTN. This shall be investigated by RAN1.

In the TR it is recommended that changes required for 4-step random access in NR NTN can be adopted for LTE-M and NB-IoT:
* UE estimates the RTT and uses as Timing Advance for Msg1 transmission
* In Msg2 TAC is provided relative to the TA used for Msg1.

RAN2 to discuss whether the UE needs to inform the eNB about the used TA during random access.

## 2.2	HARQ Operations

In the same way as in NR, HARQ RTT will be larger than the number of available HARQ processes. Using the N stop-and-wait HARQ scheme may lead to HARQ stalling, that is when all HARQ process have been used and one HARQ RTT has yet not elapsed from the first transmission. HARQ stalling can be avoided by adding more HARQ processes to fill the RTT, but that shall be avoided for NB-IoT/eMTC and is anyway not possible for GEO RTT of >500 ms.

Considering that NB-IoT/LTE-M traffic is delay tolerant and it is acceptable to have a relatively low data rate, there is no need to increase the number of HARQ processes or disable HARQ feedback. Instead it is enough to adjust the HARQ drx timer according to the long RTTs.

In the TR it is recommended to adopt the changes required for HARQ timers for HARQ processes with HARQ feedback in NR NTN.

## 2.2	RLC and PDCP

For RLC and PDCP, same changes required in NR NTN, e.g., extending the value range of the RLC t-Reassembly timer but not changing the RLC SN space, the PDCP discardTimer or the PDCP SN space.

In the TR it is recommended to extend RLC t-Reassembly timer value range with larger values.

# 3	Control Plane

## 3.1	Idle mode mobility

The cell selection/reselection procedures for LTE are quite similar to the procedures in NR; thus, it seems reasonable, as a baseline, to adopt the same measures and concepts described in [3]; nevertheless, there are a few differences.

First, LTE-M and NB-IoT have a lower resolution in terms of RSRP and RSRQ measurements due to lower complexity and sparse measurement behavior. Due to the propagation characteristics of the satellite channel the UE may have difficulties to determine the best cell to camp based on RSRP/RSRQ measurements. If we consider the reduced precision, a UE may have problems in performing cell (re)selection precisely.

The recommendation to address this issue in NR, according to [3], is to use UE location information and provide further assistance information to assist cell (re)selection. For NTN IOT, the same recommendation can be captured as the issue, although more severe, is of the same nature, provided that the new information introduced in NR has sufficient precision to work as expected for NB-IoT and LTE-M.

In the TR, it is recommended that same cell (re)selection enhancements as in NR NTN are adopted ensuring they provide sufficient precision for LTE-M and NB-IoT

Moreover, LTE-M and NB-IoT use further RSRP/RSRQ threshold to determine the Coverage Enhancement (CE) mode to be used within the cell. This feature is not present in NR, so there is no specific recommendation in [3].

Another aspect to consider is the definition of Tracking Areas (TA). Regardless of whether earth-fixed or earth-moving cells are considered, it can happen that cells that belong to a certain tracking area change over time due to the mobile characteristics of the satellites. If a tracking area is considered as “moving”, this would mean that a very large number of UEs have to perform Tracking Area Update (TAU), even though they may be stationary, causing excessive signaling.

According to [3], the recommendation to address this issue is to define earth-fixed tracking areas, and that a UE uses its position to determine if it moves to a new TA and trigger a TAU only if the UE actually moved to a new geographical area.

There is no reason to adopt a different behavior for NB-IoT or LTE-M UEs as this mechanism is the same used for NR UEs.

In the TR it is recommended that Tracking Area enhancements defined for NR NTN are adopted for NB-IoT and LTE-M UEs.

## 3.2	Connected mode mobility

NB-IoT does not support connected mode mobility, so the discussion in this section is not applicable. LTE-M, on the other hand, supports connected mode mobility, and it relies on the LTE/NR framework for measurements and handover. So, as in the case of idle mode mobility, it seems reasonable to use the recommendations from [3] as baseline.

Concerning connected mode mobility, [3] recommends specifying the following enhancements:

-	Definition of schemes to reduce service interruption during Hand-Over due to large propagation delay (especially in the case of GEO transparent)

-	Definition of schemes to tackle frequent handover and high handover rate due to satellite movement (e.g. LEO NTN)

-	Definition of schemes to improve handover robustness due to small signal strength variation in regions of beam overlap

-	Definition of schemes to compensate for propagation delay differences in the UE measurement window between cells originating from different satellites, especially for LEO NTN

The first three items are equally applicable to NTN IoT, so it seems reasonable to monitor progress in the NTN NR WI and adopt enhancements specified for NTN IoT.

RAN2 to wait until changes regarding service interruption during handover, high handover rate, and handover robustness are agreed in the NTN NR, and adopt those for NTN IoT, if applicable.

Regarding the last item, while the SMTC window is inherent only to NR, the problem of propagation delay differences while measuring cells from different satellites exists also in LTE. Since the nature of the issue is very similar, a solution to be specified for NTN NR could be taken as baseline for NTN IoT.

In the TR it is recommended that any changes concerning differences in propagation delays while measuring cells from different satellites are considered as baseline for NTN IoT. FFS if adaptations are needed for NTN IoT.

Another recommendation in [3] concerns enhancements of the triggering conditions for conditional handover (CHO), which was introduced to both LTE and NR in Rel-16:

-	Additional CHO triggering conditions (e.g. location/time based), and adaptation of measurement-based thresholds and events to the NTN environment.

Wait for specification of changes regarding conditional handover in NTN NR, and adopt those for NTN IoT.

Finally, some recommendations regarding the enhancements of mobility and measurement configurations are listed below, and regarding service continuity between TN and NTN:

-	Possible enhancements to mobility configuration (e.g. to support broadcast configuration)

-	Enhancements to measurement configuration/reporting (e.g. pre-triggering based solutions)

-	Service continuity for mobility from TN to NTN and from NTN to TN systems

It is FFS if any of these would provide substantial additional value for NTN IoT.

For NTN IoT it is FFS if any enhancement of mobility and measurement configurations are adopted for service continuity between TN and NTN.

# 4	Conclusion

Based on the discussion in the sections above, we propose the following:

Proposal 1	In the TR it is recommended that changes required for 4-step random access in NR NTN can be adopted for LTE-M and NB-IoT: * UE estimates the RTT and uses as Timing Advance for Msg1 transmission * In Msg2 TAC is provided relative to the TA used for Msg1.

Proposal 2	RAN2 to discuss whether the UE needs to inform the eNB about the used TA during random access.

Proposal 3	In the TR it is recommended to adopt the changes required for HARQ timers for HARQ processes with HARQ feedback in NR NTN.

Proposal 4	In the TR it is recommended to extend RLC t-Reassembly timer value range with larger values.

Proposal 5	In the TR, it is recommended that same cell (re)selection enhancements as in NR NTN are adopted ensuring they provide sufficient precision for LTE-M and NB-IoT

Proposal 6	In the TR it is recommended that Tracking Area enhancements defined for NR NTN are adopted for NB-IoT and LTE-M UEs.

Proposal 7	RAN2 to wait until changes regarding service interruption during handover, high handover rate, and handover robustness are agreed in the NTN NR, and adopt those for NTN IoT, if applicable.

Proposal 8	In the TR it is recommended that any changes concerning differences in propagation delays while measuring cells from different satellites are considered as baseline for NTN IoT. FFS if adaptations are needed for NTN IoT.

Proposal 9	Wait for specification of changes regarding conditional handover in NTN NR, and adopt those for NTN IoT.

Proposal 10	For NTN IoT it is FFS if any enhancement of mobility and measurement configurations are adopted for service continuity between TN and NTN.

# References

RP-193235: New Study WID on NB-IoT/eMTC support for NTN, Sitges, Spain, RAN#86, December 9-31, 2019.

R2-2007558, Idle mode aspects for NTN, Ericsson, RAN#111e, August 2020

TR 38.821, “Solutions for NR to support Non-Terrestrial Networks (NTN),” V16.0.0.