3GPP TSG-RAN WG2#112 e-meeting                                   R2-2010255

Nov 2nd – Nov 13th, 2020

Source: 		Sharp

Title:	UE information transmission in LTE fast MCG recovery case

Agenda Item:		6.8.4

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the issue that the transmission of latest UE information (i.e. MBMSInterestIndication and SidelinkUEInformation messages) may be lost in case that fast MCG recovery is performed after an RLF. Possible change to the RRC specification is provided in [2].

Discussion

In current LTE RRC specification [2], a UE may send an MBMSInterestIndication or a SidelinkUEInformation message (UE information message for short) to report the newest UE information to the PCell before a radio link failure declaration. Due to the bad channel quality in the radio link failure evaluation phase (i.e. during a running T310), such UE information may not reach the PCell. After the UE re-connects to the network via RRC connection reestablishment procedure, the UE can re-initiate transmission of MBMSInterestIndication/SidelinkUEInformation message after reception the RRCConnectionReestablishment, so that the reestablishment cell can acquire the latest UE information.

#### 5.3.7.5	Reception of the RRCConnectionReestablishment by the UE

NOTE 1:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

<OMIT>

2>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission;

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	if the UE has transmitted an MBMSInterestIndication message during the last 1 second preceding detection of radio link failure:

4>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4>	determine the set of MBMS services of interest in accordance with 5.8.5.3a;

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	if SystemInformationBlockType18 is broadcast by the PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink communication related parameters relevant in PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the last 1 second preceding detection of radio link failure; or

2>	if SystemInformationBlockType19 is broadcast by the PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink discovery related parameters relevant in PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the last 1 second preceding detection of radio link failure; or

2>	if SystemInformationBlockType21 including sl-V2X-ConfigCommon is broadcast by the PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of V2X sidelink communication related parameters relevant in PCell (i.e. change of v2x-CommRxInterestedFreqList or v2x-CommTxResourceReq) during the last 1 second preceding detection of radio link failure:

3>	initiate transmission of the SidelinkUEInformation message in accordance with 5.10.2.3;

<OMIT>

2>	submit the RRCConnectionReestablishmentComplete message to lower layers for transmission;

1>	the procedure ends;

Observation 1: the UE may initiate transmission of an MBMSInterestIndication/SidelinkUEInformation message during a reestablishment procedure, if it has transmitted an MBMSInterestIndication/SidelinkUEInformation message within 1 second before RLF detection.

Similar thing applies for handover case. A UE may send RRC messages including UE information (i.e. InDeviceCoexIndication/UEAssistanceInformation/MBMSInterestIndication/SidelinkUEInformation) to report the newest UE information to the source cell in the handover preparation phase. As this may be done after the source cell has sent the Handover Request message to the target cell, such UE information may not reach the target cell. In order to let the target cell acquire the latest UE information, the UE can re-initiate transmission of the UE information messages at the end of the handover procedure.

#### 5.3.5.4	Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover)

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

<OMIT>

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure; or

1>	if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured:

2>	stop timer T304;

<OMIT>

2>	if the UE is configured to provide IDC indications:

3>	if the UE has transmitted an InDeviceCoexIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

2>	if the UE is configured to provide power preference indications, overheating assistance information, SPS assistance information, delay budget report or maximum bandwidth preference indications:

3>	if the UE has transmitted a UEAssistanceInformation message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	if the UE has transmitted a MBMSInterestIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4>	determine the set of MBMS services of interest in accordance with 5.8.5.3a;

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	if SystemInformationBlockType18 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink communication related parameters relevant in target PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if SystemInformationBlockType19 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of sidelink discovery related parameters relevant in target PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if SystemInformationBlockType21 is broadcast by the target PCell; and the UE transmitted a SidelinkUEInformation message indicating a change of V2X sidelink communication related parameters relevant in target PCell (i.e. change of v2x-CommRxInterestedFreqList or v2x-CommTxResourceReq) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

3>	initiate transmission of the SidelinkUEInformation message in accordance with 5.10.2.3;

<OMIT>

2>	the procedure ends;

Observation 2: the UE may initiate transmission of a UE Information RRC message(s) at the end of a handover procedure, if it has transmitted such a RRC message(s) within 1 second before reception of the handover command.

With the introduction of fast MCG recovery, a UE encounters RLF can perform the recovery via MCG failure information procedure instead of RRC reestablishment. In fast MCG procedure, after transmitting the MCGFailureInformation message, UE will wait for a response e.g. HO command within a duration of T316 time. As shown in figure 1, UE sends a UE information message in T0 which is within 1 second before RLF detection at T2, and it may not be received by the PCell as in observation 1. After reporting MCGfailureinformation at T2, UE receives a HO command as response at T3. In case that the time from T0 to T3 is beyond 1 second, the UE information transmission cannot be re-initiated according to observation 2. This will lead to the latest UE information to be lost in the target cell. Given that the maximum value of waiting for a response of the MCGfailureinformation (i.e. max value of T316) is 2000ms, it is very likely that this can happen.

Figure 1 UE information transmission in fast MCG recovery case

Observation 3: in fast recovery case, if a HO command is signalled to the UE for link recovery, the target cell may not get the latest MBMSInterestIndication/SidelinkUEInformation message that has been transmitted within 1 second before the RLF detection.

To deal with this issue, the specification needs to be clarified to allow the UE to initiate the transmission of MBMSInterestIndication/SidelinkUEInformation message in a handover procedure if it has been transmitted within 1 second before the RLF detection in fast MCG recovery case.

Proposal: the UE may initiate the transmission of MBMSInterestIndication/SidelinkUEInformation message in a handover procedure if it has been transmitted within 1 second before the RLF detection in fast MCG recovery case.

Conclusion

In this contribution, we discuss the UE information transmission in fast recovery case, the observations and proposals include:

Observation 1: the UE may initiate transmission of an MBMSInterestIndication/SidelinkUEInformation message in a reestablishment procedure, if it has transmitted an MBMSInterestIndication/SidelinkUEInformation message within 1 second before RLF detection.

Observation 2: the UE can initiate transmission of a UE Information RRC message(s) at the end of a handover procedure, if it has transmitted such a RRC message(s) within 1 second before reception of the handover command.

Observation 3: in fast recovery case, if a HO command is signalled to the UE for link recovery, the target cell may not get the latest MBMSInterestIndication/SidelinkUEInformation message that has been transmitted within 1 second before the RLF detection.

Proposal: the UE may initiate the transmission of MBMSInterestIndication/SidelinkUEInformation message in a handover procedure if it has been transmitted within 1 second before the RLF detection in fast MCG recovery case.

Reference

- TS 36.331-g21

- R2-2010256, Clarification on UE information transmission in fast MCG recovery case (36.331),      Sharp