3GPP TSG-RAN WG2 Meeting #112-e	R2-2010306

Online, 02 – 13 Nov 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0974 | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the UE behaviour before the next period of SL CG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2 #110 meeting, the following agreement was made: 5: No configuredGrantTimer is introduced for SL CG operation. TX UE flushes a TB for the buffer of the Sidelink process associated to a HARQ Process ID before the next CG resource associated to the HARQ Process ID.. However, the UE behaviour of flushing buffer has not been captured in MAC specification. In stead, it is specified that if the UE has received a dynamic sidelink grant before the first PSSCH duration within a SL configured grant period, it would clear the dynamic sidelink grant. But this is not aligned with what we have agreed before. The above agreements implies that initial transmission along with potential retransmissions are required to be finished before the first transmission opportunity of the next period associated with the same HARQ process ID. That is why the Tx UE shall flush the buffer. Therefore, NW implementation will not schedule such a dynamic grant that locates in the next SL CG period, because it knows that the UE will flush the buffer after the end of last period. Thus it is redundant to specify the procedure for this case.Therefore, sticking to our agreements achieved in the previous meeting (i.e. Tx-UE flushes buffer of the Sidelink process associated to a HARQ Process ID before the next period associated to the same HARQ Process ID.) is sufficient. |
| Summary of change: |  | Clarify that the UE shall flush the HARQ buffer of Sidelink process associated with the HARQ process ID if it is the first PSSCH duration of the next SL CG period with the same HARQ process ID, and delete the description about clearing dynamic grant. Impact analysis Impacted functionality Sidelink configured grant Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue. The reason why there is no inter-operability issue is that the change only involves in UE’s internal operation, without impacts on the Inter-operability between UE and the network or that between UE and UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may perform sidelink retransmission in CG resource associated to the HARQ Process ID in next CG period. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.22.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE BEGIN

5.22.1.1	SL Grant reception and SCI transmission

================== Unchanged parts omitted ===============

The MAC entity shall for each PSSCH duration:

1>	for each sidelink grant occurring in this PSSCH duration:

2>	if the MAC entity has been configured with Sidelink resource allocation mode 1:

3>	select a MCS which is, if configured, within the range that is configured by RRC between sl-MinMCS-PSSCH and sl-MaxMCS-PSSCH included in sl-ConfigDedicatedNR;

3>	set the resource reservation interval to 0ms.

2>	else:

3>	select a MCS which is, if configured, within the range that is configured by RRC between sl-MinMCS-PSSCH and sl-MaxMCS-PSSCH included in sl-PSSCH-TxConfigList and, if configured by RRC, overlapped between sl-MinMCS-PSSCH and sl-MaxMCS-PSSCH indicated in sl-CBR-PSSCH-TxConfigList for the highest priority of the sidelink logical channel(s) in the MAC PDU and the CBR measured by lower layers according to clause 5.1.27 of TS 38.215 [24] if CBR measurement results are available or the corresponding sl-defaultTxConfigIndex configured by RRC if CBR measurement results are not available;

3>	if the MAC entity decides not to use the selected sidelink grant for the next PSSCH duration:

4>	set the resource reservation interval to 0ms.

3>	else:

4>	set the resource reservation interval to the selected value.

NOTE 5:	MCS selection is up to UE implementation if the MCS or the corresponding range is not configured by RRC.

2>	if the configured sidelink grant has been activated and this PSSCH duration corresponds to the first PSSCH transmission opportunity within this sl-PeriodCG of the configured sidelink grant:

3>	set the HARQ Process ID to the HARQ Process ID associated with this PSSCH duration and, if available, all subsequent PSSCH duration(s) occuring in this sl-PeriodCG for the configured sidelink grant;

3> flush the HARQ buffer of Sidelink process associated with the HARQ Process ID;

3>	determine that this PSSCH duration is used for initial transmission;

2>	deliver the sidelink grant, the selected MCS, and the associated HARQ information to the Sidelink HARQ Entity for this PSSCH duration.

================== Unchanged parts omitted ===============

END