3GPP TSG-RAN WG2 Meeting #112 electronic	R2-2010331

Online, November, 2020

Agenda Item:	8.7.4

Source:	MediaTek Inc.

Title:	On relay discovery

Document for:     Discussion and Decision

# Introduction

In last meeting, we have the following agreement for relay discovery mechanism below.

[Easy]Proposal6: Solution is needed to differentiate discovery message in AS layer from existing SL signalling or traffic

In this paper, we focus on the method how a UE differentiate discovery message in AS layer from existing SL signalling or traffic.

# Discussion

As well-elaborated in the email discussion paper [1], the solution to differentiate discovery message in AS layer could be

- Option 1: separate resource pool for discovery message

- Option 2: share resource pool for discovery message and other SL traffic/signaling

  - Apply other PHY and/or MAC solution to distinguish discovery message

## Benefit of separate resource pool

In our view, we think separate resource pool indeed has power saving benefit, i.e., a UE only needs to transmit or receive discovery message in specific resource pools, rather than urged to monitor PSCCH all the time for possible reception of discovery message. This means if a UE has no active SL traffic, it can perform SL DRX operation and wakes up only during the period of discovery resource pool (so that this UE can be discovered by other UEs to perform relay operation).

Secondly, separate resource pool is a way to prioritize the transmission of discovery message. That is, within a UE, the discovery message needs not compete with other SL signaling/traffic during SL LCP procedure. Besides, since discovery message has its own resource pool, the impact of transmission collision or half-duplex effect can be reduced, which in some extent enhances the transmission reliability. Furthermore, discovery resource pool enables a remote UE to access multiple relay UEs by sending a single discovery message in discovery resource pool even when multiple relay UEs apply different SL DRX configuration. In contrast, with a shared resource pool, a remote UE needs to send multiple discovery message at different time because a relay UE within the SL-DRX off duration may loss the discovery message.

Further, as well-explained by the rapporteur in the email discussion, the resource pool for discovery message can apply different resource-pool specific configuration. For example, applying different power control parameters is useful for a remote UE to measure SL-RSRP of a candidate relay UE.

As for the drawback of resource fragmentation, we tend to think the impact is not significant and cannot be controlled by NW, e.g. the resource pool for discovery message is configured with a small size or a longer periodicity.

Based on the discussion above, we think the benefit of separate resource pool can overcome the price of resource fragmentation.

Observation 1: Separate resource pool for discovery message has the following benefit:

- Power saving gain

- Enhance transmission reliability (i.e. reduce the possibility of collision and impact of half-duplex )

- Reduce the need to distribute duplicated discovery message at different time

- Flexible resource pool configuration suitable for discovery purpose (i.e. measurement and power control)

Proposal 1: Support separate resource pool for discovery message transmission/reception.

## AS layer handling for shared resource pool

In case of shared resource pool, additional method is required for a UE to recognize a discovery message.

We think several solutions can be considered below:

- Alt 0. A specific SCI indication for discovery message

  - Alt 0-1: a new SCI field for indication

  - Alt 0-2: a dedicated L1 destination ID for discovery message

- Alt 1. A dedicated L2 destination ID for discovery message

- Alt 2. A dedicated SL LCID for discovery message

For Alt 0-1, it has RAN1 impact and thus is not preferred.

For Alt 0-2, a Rx UE could determine whether it is a discovery message upon receiving SCI. The price of fast indication in SCI is that it consumes more L2 destination ID. Currently, the Destination Layer-1 ID is the 16 LSB of the Destination Layer-2 ID. So, assigning a L1 ID dedicated for discovery purposes will occupy 2^8 = 64 L2 ID, which seems acceptable because it occupies only 1/(2^16) = 0.0015% of total L2 IDs.

Observation 2: Use dedicated L1 destination ID to distinguish discovery message can have quick discovery message detection with negligible L2 ID consumption and without any RAN1 spec change

For Alt 1 and Alt 2, they both are slower than Alt 0 because UE should check the content of MAC PDU. Note that Alt 1 and Alt 2 are exclusive – there is no need to support both indications in MAC level.

Observation 3: there is no need to have duplicated indication (i.e. dedicated L2 destination ID and dedicated SL LCID) for discovery message at the same time.

Proposal 2: If shared resource pool is agreed, RAN2 considers the following solutions:

- Alt 0-2: A dedicated L1 destination ID for discovery message

- Alt 1. A dedicated L2 destination ID for discovery message

- Alt 2. A dedicated SL LCID for discovery message

As indicated by companies in [1], to distinguish the discovery message from legacy PC5-S/PC5-RRC message, a new SL-SRB dedicated for discovery message may probably be needed. A new SL-SRB should be associated with a distinct SL LCH, which needs a specific SL-LCID. If a new SL-LCID for discovery message is supported (Alt 2), then Alt 1 is not needed.

Observation 4: To distinguish discovery message from legacy PC5-S/PC5-RRC message, a new SL-SRB and a new SL LCID is needed.

Proposal 3: Introduce a new SL-SRB and a new SL LCID for discovery message for both shared resource pool and dedicated resource pool.

Proposal 4: In AS layer, discovery message is distinguished by a distinct SL LCID in case of shared resource pool.

Since discovery message is carried by shared resource pool, the LCP procedure should be considered.

- First, assign a high priority to the SL LCH dedicated for discovery message

  - As indicated in the email summary, the SL LCH used to carry discovery message should be prioritized with a high-priority value. And whether the priority is equal to or even higher than other SL-SRB traffic can be postponed to WI phase.

- Second, discovery message is not multiplexed with other SL traffic/signalling.

  - We don't see the need to multiplex other SL data with discovery message. Besides, to guarantee the high reliability, the MAC PDU including discovery message should be as small as possible. So, multiplexing with other SL traffic should be avoided.

Observation 5: The SL LCH associated with discovery message should be configured with a high priority, which may be equal to or higher than other SL-SRB.

Observation 6: There is no use case for multiplexing discovery message with other SL traffic signalling. Besides, the MAC PDU size of discovery message should be kept as small as possible to ensure reliability.

Proposal 5: Discovery message is not multiplexed with other SL traffic/signalling.

# Conclusion

In this paper, we focus on the mechanism to distinguish discovery message in AS layer. The following observations are made:

Observation 1: Separate resource pool for discovery message has the following benefit:

- Power saving gain

- Enhance transmission reliability (i.e. reduce the possibility of collision and impact of half-duplex )

- Reduce the need to distribute duplicated discovery message at different time

- Flexible resource pool configuration suitable for discovery purpose (i.e. measurement and power control)

Observation 2: Use dedicated L1 destination ID to distinguish discovery message can have quick discovery message detection with negligible L2 ID consumption and without any RAN1 spec change

Observation 3: there is no need to have duplicated indication (i.e. dedicated L2 destination ID and dedicated SL LCID) for discovery message at the same time.

Observation 4: To distinguish discovery message from legacy PC5-S/PC5-RRC message, a new SL-SRB and a new SL LCID is needed.

Observation 5: The SL LCH associated with discovery message should be configured with a high priority, which may be equal to or higher than other

Observation 6: There is no use case for multiplexing discovery message with other SL traffic signalling. Besides, the MAC PDU size of discovery message should be kept as small as possible to ensure reliability.

Accordingly, we have proposals below:

Proposal 1: Support separate resource pool for discovery message transmission/reception.

Proposal 2: If shared resource pool is agreed, RAN2 considers the following solutions:

- Alt 0-2: A dedicated L1 destination ID for discovery message

- Alt 1. A dedicated L2 destination ID for discovery message

- Alt 2. A dedicated SL LCID for discovery message

Proposal 3: Introduce a new SL-SRB and a new SL LCID for discovery message for both shared resource pool and dedicated resource pool.

Proposal 4: In AS layer, discovery message is distinguished by a distinct SL LCID in case of shared resource pool.

Proposal 5: Discovery message is not multiplexed with other SL traffic/signalling.

# Reference

- R2-20xxxxx, [Post111-e][623][Relay] Remaining issues on relay discovery (OPPO)