3GPP TSG-RAN WG2 Meeting #112-e                             R2-2010350

Online, 2 November 2020 – 13 November 2020

Agenda item:	8.3.3

Source:	Samsung

Title:	Discussion on switching mechanism for multi-SIM

Document for:	Discussion & Decision

# Introduction

In Rel-17 WI on Multi SIM, one of the objectives in [1] is as follows:

- Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2]:

  - RAT Concurrency: Network A is NR. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx

This contribution discusses how to support switching mechanism for multi-SIM UE.

# Discussion

## Short time switching mechanism

Multi-SIM UE may perform SI update, RRM measurements, TAU/RNAU, or paging monitoring in other NW while it stays in CONNECTED in current NW. In the discussion here, we assume this case as short time activities.

For short time activities, one can argue that it can be handled by UE implementation. But, in the absence of any standard defined short time switching mechanism, it may result in interruptions of the ongoing services/ waste of resources even for short time activities. We think that AS based negotiated short time gaps are needed to support short time switching for multi-SIM purpose.

Proposal 1: RAN2 to agree that AS based negotiated short time gaps are needed to support short time switching for multi-SIM purpose.

NW has no idea what kind of short time gaps are needed. It means that UE needs to request its preferred short time gap configuration to NW, and NW needs to confirm/ configure it accordingly.

Proposal 2: Multi-SIM UE can request its preferred short time gap configuration to current NW for short time activities on other NW, and current NW confirms/ configures it accordingly.

In order to handle any kind of short time activities, multiple short time switching gap configuration may be required by multi-SIM UE. Current measurement gap configuration allows a single periodic measurement gap pattern either per FR or per UE. In addition, its value range is quite limited, meaning that it may not be optimal to support short time switching for multi-SIM purpose. We think that different/ multiple short time switching gap configuration can be introduced for multi-SIM purpose while keeping the principle of existing measurement gap.

Proposal 3: Introduce different and multiple short time switching gap configuration while keeping the principle of existing measurement gap for multi-SIM purpose.

While performing short time activities on other NW within a short time switching gap duration, there seems no benefit to transit out of CONNECTED in current NW as it will involve several signaling exchanges between UE and NW. It makes more sense to keep staying in CONNECTED on current NW for short time activities

Proposal 4: When multi-SIM UE performs short time activities on other NW within a short time switching gap duration, it keeps staying in CONNECTED in current NW.

## Long time switching mechanism

Unlike short time activities, multi-SIM UE may have no idea how long it will leave the current NW i.e. to perform voice call in other NW when it stays in CONNECTED in current NW. In the discussion here, we assume this case as long time activities.

For long time activities, we do not see any value to keep staying in CONNECTED in current NW so multi-SIM UE needs to indicate current NW to leave CONNECTED.

Proposal 5: When multi-SIM UE needs to perform long time activities on other NW, it needs to indicate current NW to leave CONNECTED.

Either RRC or NAS based solution can be considered to indicate to leave CONNECTED for multi-SIM purpose i.e.

- For RRC based solution, we have introduced ReleasePreference in UEAssistanceInformation from which UE can request NW to transit itself from CONNECTED to IDLE/INACTIVE for power saving. It may be feasible to reuse existing ReleasePreference with some enhancement if necessary for multi-SIM purpose

- For NAS based solution [2], SA2 has discussed to include release indication as well as detailed release assistance information (e.g. PDU sessions not to trigger MT service for paging filtering) in Service Request.

Both solutions are actually more or less the same to leave CONNECTED for multi-SIM purpose, except more detailed release assistance information can be provided to NW via NAS signalling. So, it is proposed to discuss which layer to initiate to indicate current NW to leave CONNECTED for long time activities on other NW.

Proposal 6: RAN2 to discuss which layer (RRC or NAS) to indicate current NW to leave CONNECTED for long time activities on other NW.

The next question is when exactly multi-SIM UE leaves CONNECTED in current NW. Three options can be considered:

- Option 1: Autonomous transition out of CONNECTED

- Option 2: MAC CE by NW

- Option 3: RRCRelease by NW

We think that both options 1 and 2 do not allow multi-SIM UE to enter INACTIVE from CONNECTED. As it is up to NW whether to transit to IDLE or INACTIVE from CONNECTED, it makes more sense for multi-SIM UE to transit out of CONNECTED in current NW according to RRCRelease.

Proposal 7: After indicating current NW to leave CONNECTED, multi-SIM transits out of CONNECTED in current NW according to RRCRelease.

# Conclusion

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposals:

Proposal 1: RAN2 to agree that AS based negotiated short time gaps are needed to support short time switching for multi-SIM purpose.

Proposal 2: Multi-SIM UE can request its preferred short time gap configuration to current NW for short time activities on other NW, and current NW confirms/ configure it accordingly.

Proposal 3: Introduce different and multiple short time switching gap configuration while keeping the principle of existing measurement gap for multi-SIM purpose.

Proposal 4: When multi-SIM UE performs short time activities on other NW within a short time switching gap duration, it keeps staying in CONNECTED in current NW.

Proposal 5: When multi-SIM UE needs to perform long time activities on other NW, it needs to indicate current NW to leave CONNECTED.

Proposal 6: RAN2 to discuss which layer (RRC or NAS) to indicate current NW to leave CONNECTED for long time activities on other NW.

Proposal 7: After indicating current NW to leave CONNECTED, multi-SIM transits out of CONNECTED in current NW according to RRCRelease.

# References

[1] RP-201309, New WID: Support for Multi-SIM devices for LTE/NR

[2] 3GPP TR 23.761, "Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM)."