3GPP TSG-RAN WG2 #112e                                                                                      R2-2010376

Online, November 2nd - 13th, 2020

Agenda Item:	8.12.2.1

Source: 	CMCC

Title:  	 Discussion on the definition and constraining of reduced capabilities

WID/SID:	         FS_NR_redcap

Document for:	Discussion and Decision

Introduction

In the RAN2 #111e meeting, the definition and constraining of reduced capabilities have been discussed and the agreements have been achieved as the following items:

Agreements:

- At least for device type identification and access restriction (including initial access), the network needs to know whether the UE is redCap UE or not. FFS on whether based on explicit or implicit signalling.

- The existing UE capabilities framework is used as baseline to indicate the capabilities of a RedCap UE (this does not imply anything on the reporting of the device type, if the need for a device type will be agreed)

- The number of device types should be minimised, to reduce market fragmentation, and introduced only where essential to control UE accesses and differentiate them from legacy R15/R16 and non-Redcap R17 UEs, (e.g. number of Tx/Rx antennas, maximum supportable BW, etc.). The exact composition of the set of L1 capabilities of the device type can be discussed by RAN1

- Discuss in normative phase on whether to signal (and in case how) a Device type and its associated capabilities (the reduced set of capabilities) is captured in specifications, and whether device type is indicated as part of UE capability;

Hence, in this contribution, the discussion on the remaining issue for definition and constraining of reduced is provided in the following sections.

Discussion

The RedCap devices could be understood as the equipments serving the relatively low-end services. As described in [1], the main use cases include the small devices in factories with low requirements, e.g. the sensors or the monitors, and the wearable equipments for personal daily life, e.g. the smart watches or the medical monitors.  Therefore the RedCap UEs could be independently defined as one type of UE differentiated from the non-RedCap UEs. As discussed in email discussion [2], it is proposed to define new signalling fields in UE Capability for the features that are mandatory w/o capability signaling for non-RedCap UEs but are optional for Redcap UEs. The detail of supportive features included in the capability of RedCap UE is still under discussed in RAN1. However, due to the low requirement for RedCap UEs, the RedCap UE should be treated as one independent type of UE differentiated from the non-RedCap UE. Thus the optional capability signaling for RedCap UEs also need to include the part of optional capabilities for non-RedCap UE and additional optional capabilities which are only supportive to Redcap UEs.

The network needs to know whether the UE is RedCap UE or not to provide different access strategies which may impact the performance of network. In the email discussion [2], three options are proposed as below:

Option 1: RedCap device type is indicated as part of the capability signaling

Option 2: Define a new IE specifically for RedCap Ues containing these additional Redcap specific capabilities that is included only by Redcap UEs.

Option 3: The network obtains the RedCap based on identification solution during initial access, and forwards it to target during Handover.

As agreed in last meeting, the number of the RedCap device types should be minimised so it is possible to signal the device type of RedCap UE within the capability signalling. From the maintenance of network, it is beneficial to identify the RedCap UEs as early as possible. If the network can identify the RedCap UE during initial access, e.g. via particular/dedicated BWP or particular/dedicated radio resource or during the RRC connection setup procedure, it is possible for network to perform different access strategies.  Furthermore the network could indicate the device type of RedCap UE to CN for further verification. Therefore the option 3 listed above could also be considered as one option to identify the device type of UE for network.

Observation 1: From the maintenance of network, it is beneficial to identify the RedCap UEs as early as possible. If the network can identify the RedCap UE during initial access, e.g. via particular/dedicated BWP or particular/dedicated radio resource or during the RRC connection setup procedure, it is possible for network to perform different access strategies.

Proposal 1: the identification of the RedCap UEs during initial access also could be considered as one option to identify the device type of UE for network.

Conclusion

In this contribution, the discussion on the remaining issue for definition and constraining of reduced capability is provided.

Observation 1: From the maintenance of network, it is beneficial to identify the RedCap UEs as early as possible. If the network can identify the RedCap UE during initial access, e.g. via particular/dedicated BWP or particular/dedicated radio resource or during the RRC connection setup procedure, it is possible for network to perform different access strategies.

Proposal 1: the identification of the RedCap UEs during initial access also could be considered as one option to identify the device type of UE for network.

Reference

[1] RP-201386 Revised SID on Study on support of reduced capability NR devices, Ericsson

[2] Report of [Post111-e][913][REDCAP] Definition and constraining of reduced capabilities (Intel)

[3] 3GPP TS 38.306 v16.2.0 User Equipment (UE) radio access capabilities (Release 16)