3GPP TSG-RAN WG2 Meeting #110-e			  											R2-2010434

# Electronic, 02 – 13 November 2020

Agenda item:	6.16

Source:	Google Inc.

Title:	Clarification on RRC connection fallback handling

Document for:	Discussion & Decision

- Introduction

This document discusses the RRC connection fallback handling in the RRC connection re-establishment procedure.

# Discussion

When NAS is in 5GMM-IDLE and decides to transmit an initial NAS message (e.g., an REGISTRATION REQUEST message), it requests RRC to establish an RRC connection. Upon indication from the lower layers that the RRC connection has been established, the 5GMM enters 5GMM-CONNECTED.

#### 5.3.1.1	Establishment of the N1 NAS signalling connection

When the UE is in 5GMM-IDLE mode over 3GPP access and needs to transmit an initial NAS message, the UE shall request the lower layer to establish an RRC connection. Upon indication from the lower layers that the RRC connection has been established, the UE shall consider that the N1 NAS signalling connection over 3GPP access is established and enter 5GMM-CONNECTED mode over 3GPP access.

Initial NAS messages are:

a)	REGISTRATION REQUEST message;

b)	DEREGISTRATION REQUEST message;

c)	SERVICE REQUEST message; and

d)	CONTROL PLANE SERVICE REQUEST.

According to current NR RRC standard (i.e., TS 38.331), the UE enters RRC_CONNECTED after establishing the RRC connection. Obviously, the UE will indicate the RRC connection has been established to upper layers after it entering RRC_CONNECTED.

#### 5.3.3.4	Reception of the RRCSetup by the UE

(Omit not related parts)

1>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

2>	if T331 is running:

3>	stop timer T331;

3>	perform the actions as specified in 5.7.8.4;

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

Observation 1: In current NR RRC standard, the “enter RRC_CONNECTED” bullet implies that the RRC layer indicates upper layers the RRC connection has been established.

After entering RRC_CONNECTED, a UE may perform an RRC reestablishment procedure and transmit an RRCReestablishmentRequest message to the network. In response to the RRCReestablishmentRequest message, the network may respond the UE an RRCSetup message. After receiving the RRCSetup message, the RRC layer indicates to the NAS layer fallback of the RRC connection. Since the UE stays in RRC_CONNECTED and there is no “enter RRC_CONNECTED” action, which implies the RRC layer does not indicate the NAS layer that the RRC connection has been established.

#### 5.3.3.4	Reception of the RRCSetup by the UE

(Omit not related parts)

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration and CCCH configuration;

2>	indicate to upper layers fallback of the RRC connection;

2>	stop timer T380, if running;

Observation 2: After the RRC connection fallback in RRC connection re-establishment procedure, RRC only indicates upper layers fallback of the RRC connection (i.e., does not indicate upper layers the RRC connection has been established.)

After receiving the fallback indication from the RRC in 5GMM-CONNECTED, the NAS layer enters 5GMM-IDLE. The NAS layer may then try to transmit a REGISTRATION REQUEST message. According to 5.3.1.1, the NAS layer in 5GMM_IDLE will request the RRC layer to establish an RRC connection under this situation.  However, the RRC layer only initiates the RRC establishment procedure while it is in RRC_IDLE. Since the UE is in RRC_CONNECTED, the UE does not initiate the RRC connection establishment procedure and consequently does not indicate upper layers the RRC connection has been established. This results in the NAS layer unable to perform the registration procedure after the RRC connection fallback in RRC re-esatblishment procedure.

Reference from TS 24.501

#### 5.3.1.2	Re-establishment of the N1 NAS signalling connection

When the UE in 5GMM-CONNECTED mode over 3GPP access receives a fallback indication from lower layers, and the UE has no pending NAS procedure and no pending uplink user data for PDU session(s) with user-plane resources already established, the UE shall:

a)	enter 5GMM-IDLE mode; and

b)	initiate the registration procedure for mobility and periodic registration update and include the Uplink data status IE in the REGISTRATION REQUEST message indicating the PDU session(s) for which user-plane resources were active prior to receiving the fallback indication, if any (see subclause 5.5.1.3 for further details).

Reference from TS 38.331

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE and it has acquired essential system information as described in 5.2.2.1, or for sidelink communication as specified in sub-clause 5.3.3.1a.

Observation 3: After the RRC connection fallback in RRC connection re-establishment procedure, the NAS layer is unable to enter 5GMM-CONNECTED and is unable to perform the error handling procedure (e.g., a registration procedure).

Either the NAS solution or the RRC solution can solve the problem in observation as described below.

In the NAS solution, the NAS layer can either stay in 5GMM-CONNECTED without transitioning to 5GMM-IDLE or transition back to 5GMM-CONNECTED from 5GMM-IDLE after receiving the fallback indication. However, the discussion and decision are not in RAN2 and a LS to CT1 is needed.

In the RRC solution, the RRC layer can indicate the RRC connection has been established to the NAS layer after the fallback indication. After receiving the connection establishment indication, the NAS layer is able to enter 5GMM-CONNECTED mode again perform the error handling procedure as specified in section 5.3.1.2 the NAS specification 24.501.

We kindly request RAN2 to discuss the above problem and solutions. If RAN2 decides to go for the RRC solution, we have an RRC solution text proposal in the end of this document. If RAN2 decides to leave the problem to CT1 to solve, RAN2 should send a liaison to CT1 to indicate the problem.

Proposal 1: We kindly request RAN2 to discuss above problem and solutions.

Proposal 2: Send a Liaison to CT1 to indicate the problem if needed.

# 3 Conclusion

According to above discussions, we have following observations, proposals:

Observation 1: In current NR RRC standard, the “enter RRC_CONNECTED” bullet implies that the RRC layer indicates upper layers the RRC connection has been established.

Observation 2: After RRC connection fallback in RRC connection re-establishment procedure, RRC only indicates upper layers fallback of the RRC connection (i.e., does not indicate upper layers the RRC connection has been established.)

Observation 3: After RRC connection fallback in RRC connection re-establishment procedure, the NAS is unable to enter 5GMM-CONNECTED and is unable to perform the error handling procedure (e.g., an registration procedure).

Proposal 1: We kindly request RAN2 to discuss above problem and solutions.

Proposal 2: Send a Liaison to CT1 to indicate the problem if needed.

If RAN2 decides to go for the RRC solution, we have an RRC solution text proposal in the end of this document. If a liaison to CT1 is needed, we are glad to provide the liaison to CT1 based on RAN2’s decision.

# References

[1] TS 24.501 v16.6.0

[2] TS 38.331 v16.2.0

# Text Proposal

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration and CCCH configuration;

2>	indicate to upper layers fallback of the RRC connection;

2>	stop timer T380, if running;

1>	perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T300, T301 or T319 if running;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	stop timer T320, if running;

1>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

2>	if T331 is running:

3>	stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest:

2>	indicate to upper layers that the RRC connection has been established;

1>	consider the current cell to be the PCell;

1>	set the content of RRCSetupComplete message as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	if the RRCSetup is received in response to an RRCSetupRequest:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI-Part2;

3>	else:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI;

2>	set the selectedPLMN-Identity to the PLMN or SNPN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList or the PLMN(s) or SNPN(s) included in the npn-IdentityInfoList in SIB1;

2>	if upper layers provide the 'Registered AMF':

3>	include and set the registeredAMF as follows:

4>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

5>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

4>	set the amf-Identifier to the value received from upper layers;

3>	include and set the guami-Type to the value provided by the upper layers;

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [21]):

3>	include the s-NSSAI-List and set the content to the values provided by the upper layers;

2>	set the dedicatedNAS-Message to include the information received from upper layers;

2>	if connecting as an IAB-node:

3>	include the iab-NodeIndication;

2>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

2>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

3>	include the idleMeasAvailable;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	if reconnectCellId in VarRLF-Report is not set:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link or handover failure;

4>	set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

3>	include rlf-InfoAvailable in the RRCSetupComplete message;

2>	if the UE supports RLF report for inter-RAT MRO NR as defined in TS 36.306 [62], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10]:

3>	if reconnectCellId in VarRLF-Report of TS 36.331[10] is not set:

4>	set timeUntilReconnection in VarRLF-Report of TS 36.331[10] to the time that elapsed since the last radio link or handover failure in LTE;

4>	set nrReconnectCellId in reconnectCellId in VarRLF-Report of TS 36.331[10] to the global cell identity and the tracking area code of the PCell;

3>	if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]

4>	include rlf-InfoAvailable in the RRCSetupComplete message;

2>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

3>	include the mobilityHistoryAvail in the RRCSetupComplete message;

2>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

3>	if speedStateReselectionPars is configured in the SIB2:

4>	include the mobilityState in the RRCSetupComplete message and set it to the mobility state (as specified in TS 38.304 [20]) of the UE just prior to entering RRC_CONNECTED state;

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.