3GPP TSG-RAN WG2 #112e	Tdoc R2-2010534

Electronic meeting, November 2nd – 12th 2020	Revision of R2-2007603

Agenda Item:	8.3.2

Source:	Ericsson

Title:	Paging collision avoidance

Document for:	Discussion, Decision

# 1	Introduction

MultiSIM devices enable use of multiple subscriptions in one device, e.g. more than one USIM and concurrent registration in more than one network. The USIMs are from same or different operator (PLMN).

A MultiSIM device needs to handle “simultaneous state” situations like:

- RRC_IDLE/INACTIVE + RRC_IDLE/INACTIVE, e.g.

  - check paging in multiple systems

  - receive system information for multiple systems

  - perform measurements on multiple systems

- RRC_CONNECTED+ RRC_INACTIVE/IDLE, e.g.

  - voice + paging/SI/measurements

RAN2 WID RP-201309 [1] defines three objectives for the support for MultiSIM devices in Rel-17, In this contribution the focus is on the objective 1, reported here:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs:

  - RAT Concurrency: Network A can be NR. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx.

This contribution was revised to account for another possible solution to this objective.

# 2	Discussion

​A MultiSIM UE with a single Rx processing unit/chain can receive transmissions from one PLMN at a time. One major issue is the possibility to miss the Paging message sent by a PLMN, if the UE listening to the Paging in the other PLMN, causing performance degradation in terms of paging detection.

This may happen if the Paging Occasions (PO) / Paging Frame (PF) for the different USIMs are overlapping in time domain.

The figure shows an example of Paging collision: the slots in bold indicates where the Multi-SIM UE is listening to Paging (PLMN1 or PLMN2). If the UE is listening to PLMN1 while the Paging message is sent by PLMN2, the message is missed (red slot). Even though the paging is repeated (after CN timer T3513 expires) the message may be missed again.

According to TS 36.304 [2] and TS 38.304 [3] the formulas for calculating Paging Frame and Paging Occasions are the following:

(System Frame Number) SFN for the PF is determined by:

5G:	(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

LTE:	SFN mod T= (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

Where:

UE_ID: 5G-S-TMSI mod 1024		(in the 5GS)

UE_ID: IMSI mod 1024			(in the EPS)

The SFN and PO depends on the UE_ID, which can be reassigned (in case of 5GS) over time at the Mobility Registration procedure, and on other parameters configurable on cell level (e.g. T, N, Ns).

Given that the SFN/PO are based on parameters configurable on cell level and that the UE_ID is reassigned under certain conditions (at least in 5GS case), it is expected that the probability of Paging Occasions collision in the two PLMNs is rather low and not systematic.

- It is expected that the probability of Paging Occasions collision in the two PLMNs is rather low and not systematic.

Considering the low probability of Paging Occasions collision, we think that a solution based on UE implementation is feasible.  Even though such solution does not fully avoid the PO collision, we expect that it is a feasible/effective mean to minimize the impacts on the Paging reception caused by the collision, without introducing any specification change.

- Given the low probability of Paging Occasions collision, a solution based on the UE implementation is expected to be a feasible/effective mean to minimize the impacts on the Paging reception caused by the collision, without introducing any specification change.

Another possible solution is to involve the network in case a PO collision occurs. This solution would be based on the usage of an additional offset (here called “UE_offset”) in the SFN and PO calculation formulas.

The formulas for the SFN and PO calculation are updated as following:

5G:	(SFN + PF_offset) mod T = (T div N)*((UE_ID + UE_offset) mod N)

LTE:	SFN mod T= (T div N)*((UE_ID + UE_offset) mod N)

i_s = floor ((UE_ID + UE_offset) / N) mod Ns

Alternatively, the UE_ID can be modified directly, which leads to a similar outcome after the modulo N operation:

UE_ID: (5G-S-TMSI + UE_offset) mod 1024		(in the 5GS)

UE_ID: (IMSI + UE_offset) mod 1024			(in the EPS)

Since the new offset is added to the UE_ID, which is a CN identifier, it makes sense that also the offset is a CN-centric parameter to maintain the control in the CN. For this reason, this solution is based on a new offset generated by the CN.

Note: 	The addition of the offset to the existing UE_ID corresponds to having a new UE_ID. So, another possibility would be the CN generates a completely new UE_ID, instead the offset. Even though the result of the SFN/PO calculation is the same, the usage of the offset allows to have a common solution applicable to both 5GS and EPS, since in EPS the UE_ID (based on IMSI) cannot be changed.

The procedures to notify the PO collision and assign the UE offset are described in the Appendix.

- An alternative solution is to involve the network, e.g. by introducing an additional offset (generated by the CN and delivered to UE and gNB/eNB) which is used in the SFN and PO calculation formulas, whenever the PO collision occurs.

Given the two approaches described above i.e. UE implementation or network solution, it is proposed that RAN2 selects one of the two solutions to use in case of PO collision.

- RAN2 to agree on one of the two following solutions to cope with the Paging Occasion collision:
1) UE implementation based
2) Network based, by introducing an additional offset used in the SFN and PO calculation

# Conclusion

In the previous sections we made the following observations:

- It is expected that the probability of Paging Occasions collision in the two PLMNs is rather low and not systematic.

- Given the low probability of Paging Occasions collision, a solution based on the UE implementation is expected to be a feasible/effective mean to minimize the impacts on the Paging reception caused by the collision, without introducing any specification change.

- An alternative solution is to involve the network, e.g. by introducing an additional offset (generated by the CN and delivered to UE and gNB/eNB) which is used in the SFN and PO calculation formulas, whenever the PO collision occurs.

- Based on the discussion in the previous sections we propose the following:

- RAN2 to agree on one of the two following solutions to cope with the Paging Occasion collision:
1) UE implementation based
2) Network based, by introducing an additional offset used in the SFN and PO calculation

# References

RP-201309, WID: Support for Multi-SIM devices for LTE/NR.

TS 36.304 v.16.0.0, User Equipment (UE) procedures in idle mode

TS 38.304 v.16.0.0, User Equipment (UE) procedures in idle mode

# Appendix: Procedures to notify the PO collision and assign the UE offset

The procedure to notify the PO collision and assign the new offset is shown in the figure below:

The solution requires the UE to check if there is a collision for the PO between the two PLMNs.

The check is done at cell change and whenever a new 5G-GUTI is assigned.

If the collision is detected, the UE (in RRC_IDLE or RRC_INACTIVE) sets up the connection to one of the PLMNs and notifies the collision to the CN via NAS signaling (e.g. NAS Mobility Registration/TAU Request).

The CN selects the new offset value (it can be a random value or other means can be used) and provides it to the UE via the NAS Mobility Registration/TAU Accept message.

In case of 5GS and NR, the CN signals the UE_offset to gNB by performing the UE Context Modification procedure. The gNB  uses the value  for the SFN/PO calculation for RAN Paging when the UE is in RRC_INACTIVE state.

The following figure shows when the UE is paged:

If the UE is in RRC_IDLE, the CN includes the UE_offset in the NGAP Paging, so the gNB can use it for the SFN/PO calculation.

Note: 	The NGAP Paging only carries the 5G-S-TMSI, then the UE_offset has to be provided in a new Information Element to allow the gNB to generate the new UE_ID. On the other hand, the S1AP Paging carries both the IMSI and the UE_ID, so the CN can directly provide the new UE_ID (that is, (IMSI + UE_offset) mod 1024), without sending the UE_offset. This implies that no change is required to the S1AP Paging.

If the UE is in RRC_INACTIVE, the gNB uses the UE_offset previously received from the CN in the UE Context Modification procedure.

The RRC Paging is sent to the UE according to the updated SFN/PO formulas and received by the UE.