3GPP TSG-RAN WG2 #112e	Tdoc R2-2010535

Electronic meeting, November 2nd – 12th 2020

Agenda Item:	8.3.4

Source:	Ericsson

Title:	Introduction of a Paging cause indication

Document for:	Discussion, Decision

# 1	Introduction

MultiSIM devices enable use of multiple subscriptions in one device, e.g. more than one USIM and concurrent registration in more than one network. The USIMs can be from the same or different operator(s) (PLMN).

A MultiSIM device needs to handle “simultaneous state” situations like:

- RRC_IDLE/INACTIVE + RRC_IDLE/INACTIVE, e.g.

  - check paging in multiple systems

  - receive system information for multiple systems

  - perform measurements on multiple systems

- RRC_CONNECTED+ RRC_INACTIVE/IDLE, e.g.

  - voice + paging/SI/measurements

RAN2 WID RP-201309 [1] defines three objectives for the support for MultiSIM devices in Rel-17.  In this contribution the focus is on the objective 3, reported here:

- Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR.

  - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR.

  - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

# 2	Discussion

​A MultiSIM UE with a single Rx processing unit/chain receives transmissions from one PLMN at a time.

Once the Paging message is received from PLMN2, the MultiSIM UE shall be able to decide if interrupting the ongoing service on the PLMN1 to respond to the paging.

This is possible if a paging cause is included in the Paging message. Depending on the cause, the UE can avoid responding to the Paging if it is for a service with lower priority, thus minimizing the impact on the ongoing service in the current network.

From RAN2 point of view, a paging cause can be added to the NR/LTE Paging message to allow the MultiSIM UE to decide if interrupting the ongoing service on the network where the UE is currently connected and to respond to the paging in the other network.

It is expected that the CN performs mapping from incoming IP packet (e.g, ToS/DSCP) to a paging cause and sends the cause to the gNB/eNB using the NGAP/S1AP Paging message. Then the gNB/eNB forwards transparently the paging cause to the MultiSIM UEs via RRC Paging.

It is up to the CN to set the cause and send it to the gNB/eNB using the NGAP/S1AP Paging message.

RAN2 objective 3) is focusing on a “mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR”.

In this respect, a new optional IE containing the Paging cause would be needed to indicate if the service is VoLTE/VoNR or not.

It requires one bit to encode the cause (e.g. VoLTE/VoNR or Other) and one additional bit to indicate whether paging cause is included for the intended MultiSIM UEs. This new IE is included in the Paging Records carried in the Paging message.

Since not all the paging records will include the cause (only the ones for the MultiSIM UEs), we expect that the 2 additional bits of the cause have neglectable impacts to the RRC Paging message, both regarding the message size and the possible coverage shrinking.

It is expected that a 2 bits cause will have neglectable impacts to the RRC Paging message, both regarding the Paging message size and the possible coverage shrinking.

From RAN2 point of view, a paging cause indicating if the incoming service is “VoLTE/VoNR” or “Other” can be added the Paging message with no expected impacts on the Paging message.

As mentioned previously, the paging cause is created by the CN and sent to the gNB/eNB via NGAP/S1AP Paging message. Hence, no capability exchange is expected to be performed between the CN and gNB/eNB in relation with the support of the cause. That is, the CN can anyway always include the cause in the Paging message for the MultiSIM UE and if the gNB/eNB supports it, it will be forwarded to the UE, otherwise it will be discarded.

No capability exchange is needed between CN and gNB/eNB in relation with the paging cause support. The CN always includes the cause in the Paging message for the MultiSIM UE and if the gNB/eNB supports it, it will be forwarded to the UE, otherwise it will be discarded.

As stated in the RAN2 Objective 3), the Objective itself is valid “Unless SA2 find an alternative solution or decides otherwise”.

SA2 group is currently discussing about the possible causes and their number and there is the possibility that a higher number of causes may be agreed. This might have impacts on the Paging message: increase of its size or, similarly, a reduction of the number of the Paging Records carried by one Paging message. It might also imply a shrinking of the Paging coverage, due to the additional SNR caused by the size increase. This scenario requires further study.

If SA2 agrees on a higher number of causes, the impacts on the Paging message (size increase and coverage shrinking) should be further studied.

# Conclusion

In the previous sections we made the following observations:

- It is up to the CN to set the cause and send it to the gNB/eNB using the NGAP/S1AP Paging message.

- It is expected that a 2 bits cause will have neglectable impacts to the RRC Paging message, both regarding the Paging message size and the possible coverage shrinking.

- No capability exchange is needed between CN and gNB/eNB in relation of the paging cause support. The CN always includes the cause in the Paging message for the MultiSIM UE and if the gNB/eNB supports it, it will be forwarded to the UE, otherwise it will be discarded.

- If SA2 agrees on a higher number of causes, the impacts on the Paging message (size increase and coverage shrinking) should be further studied.

- Based on the discussion in the previous sections we propose the following:

- From RAN2 point of view, a paging cause can be added to the NR/LTE Paging message to allow the MultiSIM UE to decide if interrupting the ongoing service on the network where the UE is currently connected and to respond to the paging in the other network.

- From RAN2 point of view, a paging cause indicating if the incoming service is “VoLTE/VoNR” or “Other” can be added the Paging message with no expected impacts on the Paging message.

# References

RP-201309, WID: Support for Multi-SIM devices for LTE/NR.