3GPP TSG-RAN WG2 Meeting #112e	R2-2010553

Online, November 2nd - 13th, 2020

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0464 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Gap capability with dynamic reporting for NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The NeedForGap capability defined for NR SA to NR target bands in Rel.16 allows the UE to provide its gap requirement information dynamically through the RRC Connection Reconfiguration Complete. However when UE is configured with NR-DC, no gap requirement can be provided to the network and gap will always be configured. Some UE will have the ability to perform measurement on Target NR bands without the need of a gap. Therefore it’s crucial to allow UE to share its capability with network and avoid having gap configured when it’s not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | we propose to add a capability to indicate to network that UE supports dynamic reporting for the NeedForGap while in NR-DC (in a similar manner as in NR SA). Impact Analysis: Impacted 5G architecture options: NR-DC Impacted functionality: Measurement capaiblity. Interoperability issue: - if the network is implemented according to the CR and the UE is not, no interoperability issue is expected as network assumes UE doesn’t support this capability and hence gap will always be configured. - if the UE is implemented according to the CR and the network is not, no interoperability issue is expected, as network ignores the new capability and will never inquiry UE with about its gap requirement using the NeedForGapsConfigNR IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Gap will always be configured for UE in NR-DC mode when performing measurement on NR target bands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR 2240 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 4.2.9a	MeasAndMobParametersMRDC

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| condPSCellChangeFDD-TDD-r16 Indicates whether the UE supports conditional PSCell change between FDD and TDD cells. | UE | No | No | No |
| condPSCellChangeFR1-FR2-r16 Indicates whether the UE supports conditional PSCell change between FR1 and FR2. | UE | No | No | No |
| nrdc-NeedForGap-Reporting-r16 Indicates whether the UE supports reporting the measurement gap requirement information for NR target in the UE response to a network configuration RRC message, while UE is in NR-DC. | UE | No | No | No |
| pscellT312-r16 Indicates whether the UE supports T312 based fast failure recovery for PSCell. | UE | No | No | No |

End of changes