3GPP TSG-RAN WG2 Meeting # 112e	R2-2010570

# Electronic Meeting,  Nov 2-13, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0278 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of A-GNSS Periodical retrival of Assistance Data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R2 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the A-GNSS positionings in LTE, if the positioning server (SMLC in LTE network) is not able to find the serving cell of the UE in the Database, it will use the neighbor cells’ information to build the reference data for UE. It is not always possible to guarantee that the serving cell would be available in LMF database mainly because of below reasons. - 3GPP TS 29.272 V16.3 defines the serving cell NR cell identity shall be included in DetermineLocation Request message. However, when the LMF database does not have same cell identity, it will cause failure in LMF to decode the global cell identity (NR Cell Identity (NCI) from DetermineLocation message) gNBid + cellid. Because the gNBid (NR) does not have fixed positions in NCI. The NR Cell Identity (NCI) is a 36-bit integer which consists of gNBId (22-32 bits) and cellId. So, if only NCI is known, it is not possible to extract gNBId + cell ID to get the neighbor cell in LMF DB, unless the gNBId length is known. In EPC also there is same limitations as eNBId may have different length. - Also, for the case that the gNBid can be determined, some of the cells in one gNB can be distributed over large area and it may not be possible to determine which neighbor cell(s) that are closest to the UE and hence AD may not be tuned to that location. - The already deployed E-SMLC, LMF (location servers) which have been tradionally only implemented to support RAT independent positioning method such as A-GNSS may not have all the cell information in it’s database. Mainly because of OAM based slow update (introduction of new cells which has not been updated by OAM). If the serving cell cannot be found from the LMF database, this limitation has a high possibility to lead to a A-GNSS positioning failure especially during RTK positioning, because UE may have to change the serving cell frequently in RTK A-GNSS positionings. Hence, it is desired that neighbor cell information is also provided to get a hit in LMF database and to be able to guarantee the A-GNSS data generation. In order to ensure successful retrival of periodical assistance data, UE should provide the neighbor cell along with serving cell information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Neighbor cell information have been added and capabilty have been added for peridoical asssiatnce data procedure Impacted 5G architecture options: NR SA, NR-DC, (NG)EN-DC, NE-DC Impacted functionality: A-GNSS Periodic Assistance Data Inter-operability: Inter-operability: If the NW implements this CR but the UE does not: - A-GNSS AD may not be generated If the UE implements this CR but the NW does not: - A-GNSS AD may not be generated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Longer time to deploy A-GNSS. Failure in generating A-GNSS. UE may fallback to standalone GNSS which may consume more battery for UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1b, 6.4.1, 6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Changes

### 5.2.1b	Periodic Assistance Data Transfer with Update procedure

# Figure 5.2.1b-1: LPP Periodic Assistance data transfer with update procedure

1.	Steps 1-2 and optionally steps 3-4 are performed for the Periodic Assistance Data Transfer procedure in clause 5.2.1a with the following exceptions:

-	The RequestAssistanceData message in step 1 indicates the update capabilities of the target device.

-	The ProvideAssistanceData message in step 2 indicates the update capabilities of the target device which are supported by the server.

2.	If the target device changes its primary cell and if the update capabilities of the target device supported by the server in step 1 include update of a primary cell ID, the target device sends a RequestAssistanceData message to the server using some available transactionID T3, which is different from T2 (previously used in step 2). The message contains the periodicSessionID S (previously used in step 1),  the new primary cell ID and the neighbour cell IDs (listed by the signal strength) in the IE CommonIEsRequestAssistanceData.

3.	The server responds with a ProvideAssistanceData message to the target. The message uses the transactionID T3 in step 2 and indicates the end of this transaction. The message contains the periodicSessionID S in IE CommonIEsProvideAssistanceData. Steps 2-3 are repeated each time the target device changes its primary cell.

4.	Steps 4-7 are performed for the Periodic Assistance Data Transfer procedure in clause 5.2.1a.

Next Changes

### 6.4.1	Common Lower-Level IEs

#### –	AccessTypes

<Skip Unmodified Changes>

#### PeriodicAssistanceDataControlParameters

The IE PeriodicAssistanceDataControlParameters is used in a periodic assistance data delivery procedure as described in clauses 5.2.1a and 5.2.2a.

-- ASN1START

PeriodicAssistanceDataControlParameters-r15 ::= SEQUENCE {

periodicSessionID-r15			PeriodicSessionID-r15,

...,

[[

updateCapabilities-r15		UpdateCapabilities-r15		OPTIONAL

]]

}

PeriodicSessionID-r15 ::= SEQUENCE {

periodicSessionInitiator-r15	ENUMERATED { locationServer, targetDevice, ... },

periodicSessionNumber-r15		INTEGER (0..255),

...

}

UpdateCapabilities-r15 ::= BIT STRING {primaryCellID-r15	(0),

neigborCellList-r17	(1)} (SIZE(1..8))

-- ASN1STOP

PeriodicAssistanceDataControlParameters field descriptions

periodicSessionID This field identifies a particular periodic assistance data delivery session and the initiator of the session.

updateCapabilities This field identifies the capabilities of the sending entity to support an update of periodic assistance data. A bit value set to one indicates a capability is supported and a bit value set to zero indicates a capability is not supported.

Next Changes

### 6.4.2	Common Positioning

#### –	CommonIEsRequestCapabilities

The CommonIEsRequestCapabilities carries common IEs for a Request Capabilities LPP message Type.

<Skip Unmodified Changes>

#### –	CommonIEsRequestAssistanceData

The CommonIEsRequestAssistanceData carries common IEs for a Request Assistance Data LPP message Type.

-- ASN1START

CommonIEsRequestAssistanceData ::= SEQUENCE {

primaryCellID		ECGI		OPTIONAL,	-- Cond EUTRA

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Cond Segmentation

]],

[[

periodicAssistanceDataReq-r15

PeriodicAssistanceDataControlParameters-r15

OPTIONAL,	-- Cond PerADreq

primaryCellID-r15			NCGI-r15					OPTIONAL	-- Cond NR

]],

[[

nr-NeighborCellIDList-r17		SEQUENCE (SIZE(1..8)) OF NCGI-r15 OPTIONAL,	-- Cond NR

eutra-NeigborCellIDlist-r17	SEQUENCE (SIZE(1..8)) OF ECGI	  OPTIONAL    -- Cond EUTRA

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EUTRA | The field is mandatory present for E-UTRA or NB-IoT access. The field shall be omitted for non-EUTRA and non-NB-IoT user plane support. |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| PerADreq | The field is mandatory present if the target device requests periodic assistance data delivery. Otherwise it is not present. |
| NR | The field is mandatory present for NR access. The field shall be omitted for non-NR user plane support. |

CommonIEsRequestAssistanceData field descriptions

primaryCellID This parameter identifies the current primary cell for the target device.

segmentationInfo This field indicates whether this RequestAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceDataReq This field indicates a request for periodic assistance data delivery, as specified in clause 5.2.1a.

nr-NeighborCellIDList This field indicates a list of neighbor cell NCGI that the target cell provides in descending order of signal strength.

eutra-NeighborCellIDList This field indicates a list of neighbor cell ECGI that the target cell provides in descending order of signal strength.

End of Changes