3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-2011104

Elbonia, 02 – 13 November 2020

Title:	Reply LS on LS on restricting the rate per UE per network slice

Response to:	S2-2007946/ R2-2010694

Release:	Release 17

Work Item:	FS_eNS_Ph2

Source:	TSG RAN WG2

To:	SA2

Cc:	RAN3

Contact Person:

#### Name:	György Wolfner

E-mail Address:	gyorgy(dot)wolfner(at)nokia(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks SA2 for the LS on restricting the rate per UE per network slice (S2-2007946/ R2-2010694).

RAN2 provides the following feedback on the solutions listed in the LS:

1) Solution #22

In this solution RAN enforces uplink and downlink SMBR of UEs. This is a similar function as UE-AMBR enforcement at slice level. With proper configuration (LCG and LCH restrictions), the RAN is able to obtain and control the UL data volume of a slice. Therefore, many companies think the solution can be supported without changes to RAN2 specifications, but some companies do not agree, so RAN2 has no consensus on the matter and will continue to discuss.

2) Solution #37

In this solution the CN calculates the UE-AMBR considering SMBR and RAN should enforce the UE-AMBR. RAN2 does not see any impacts of this solution to RAN2 specifications.

3) Solution #43

In this solution the RAN can send notifications when UE SMBR reached. This solution requires the RAN to be able to detect when the uplink data volume per slice per UE exceeds a limit. RAN2 does not see any impacts of this solution to RAN2 specifications.

2. Actions:

To SA2 group.

ACTION: 	RAN2 respectfully asks SA2 to take the above feedbacks into account.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#113		25 January – 5 February 2020			Electronic Meeting