3GPP TSG RAN WG2#112-e	R2-2011118

Online meeting, 2nd - 13th November, 2020

3GPP TSG RAN WG1 Meeting #103-e		R1-2009570

# E-meeting, October 26th – November 13th, 2020

Title:	LS reply on cell-grouping UE capability for synchronous NR-DC

Response to:	LS on cell-grouping UE capability for synchronous NR-DC (R2-2008662)

Release:	Release 16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	RAN1

To:	RAN2

Cc:	RAN4

Contact person:	Fred Takeda

ktakeda (at) qti (dot) qualcomm (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1 has discussed the need of PUCCH-grouping capability for NR-CA with two PUCCH groups and agreed to support the new Rel.16 capabilities copied below. From RAN1 point of view, framework of PUCCH-grouping capability for NR-CA can also be used as potential solution for synchronous NR-DC, and decision is up to RAN2.

Agreements: - Whether to support two PUCCH groups is reported per BC for NR-CA with 3 or more bands with at least two carrier types from carrier types {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} - For the BC, the UE reports one or multiple of supported configuration(s) of {primary PUCCH group config, secondary PUCCH group config} where for each supported configuration, - the “primary PUCCH group config” includes following information: - One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} mapped to the primary PUCCH group - One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission in the primary PUCCH group - the “secondary PUCCH group config” includes following information: - One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} mapped to the secondary PUCCH group - One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission in the secondary PUCCH group - Note: for each {primary PUCCH group config, secondary PUCCH group config}, each carrier type of {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} is mapped to either or both of the primary PUCCH group config and the secondary PUCCH group config. - Note: RAN1 will discuss on how to handle the SDL or SUL band, for example as below - SDL overlapping with either TDD or FDD can follow the same principle with TDD or FDD accordingly - SDL having no overlapped TDD or FDD can follow the same principle with FDD - Note: When the carrier type of NUL is indicated for PUCCH transmission location, the SUL in the same cell as in the NUL can also be configured for PUCCH transmission - FFS: how to cover licensed/unlicensed and/or FR1/FR2 differentiations - Note: When the carrier type of NUL is indicated for one PUCCH group config, the SUL in the same cell as in the NUL can also be configured for the PUCCH group - FFS: SUL is counted as number of bands for the condition of this new FG reporting

Agreements:

- Define a new FG for support of up to three different numerologies for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA where UE is not configured with two NR PUCCH groups

| 22. NR Others | 22-x | Support of up to three different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA where UE is not configured with two NR PUCCH groups | Support of up to three different numerologies in the same NR PUCCH group for NR-CA where UE is not configured with two NR PUCCH groups 1) Which NR Carrier type(s) that can transmit NR PUCCH |  | Yes | N/A |  | Per BC | N/A | N/A | N/A | Candidate values 1) One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Define a new FG for support of up to four different numerologies for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA where UE is not configured with two NR PUCCH groups

| 22. NR Others | 22-x | Support of up to four different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA where UE is not configured with two NR PUCCH groups | Support of up to four different numerologies in the same NR PUCCH group for NR-CA where UE is not configured with two NR PUCCH groups 1) Which NR Carrier type(s) that can transmit NR PUCCH |  | Yes | N/A |  | Per BC | N/A | N/A | N/A | Candidate values 1) One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Note: These capabilities are indicated independently for each BC of EN-DC, NGEN-DC, NE-DC and NR-CA

# 2	Actions

To 3GPP RAN2

ACTION: To take into account the RAN1’s feedback above for their specification work of cell-grouping capability for synchronous NR-DC.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #104-e	25 January – 5 February 2021, Electronic