3GPP TSG-RAN WG2 Meeting #112 Electronic	R2-20xxxxx

Online, 2 – 13 November 2020

Agenda item:	8.2.3

Source:	CATT

Title:	[AT112-e][231][eDCCA] Progressing conditional reconfiguration for SN initiated inter-SN CPC (CATT)

WID/SID:	LTE_NR_DC_enh2-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

This is the report for the following email discussion:

[AT112-e][231][eDCCA] Progressing conditional reconfiguration for SN initiated inter-SN CPC (CATT)

Scope: Discuss the option 1 and option 3 details from P4 of email discussion [Post111-e][920][1] to better understand the technical details between the alternatives (e.g. signalling flows, signalling load, etc.)

Intended outcome: Discussion summary in R2-2010734 (by email rapporteur).

Deadlines phase 1: Monday 9th November, UTC 1200. Phase 1 is to gather information for detail of option 1 and option 3 and technical points for comparison.

Deadline phase 2:  email discussion report: 2nd week Thu, UTC 1000

Note that the focus of this email discussion is primarily on proposal 4 of email discussion [1].  The two options listed in Proposal 4 is analysed taken in to account the signalling flow, signalling details in order to understand the options. Also pros and cons of each option are discussed with an intention to select the most appropriate option for SN initiated Inter-SN CPC.

# 2	Discussion

For SN initiated Inter-SN CPC, the SN should provide the CPAC trigger condition.  Same as in Rel-16 CPC, the trigger condition in this case can be defined by a measurement identity, given by a measurement configuration provided by the SN.

For SN initiated inter-SN conditional PSCell change, [1] discussed different options for generating the conditional configuration message. The following options were selected for further discussion.

Option 1: The MN generates CPC. The source SN sets the execution condition and communicates it to the MN. The MN generates the conditional reconfiguration message including the execution condition(s) provided by the source SN and RRCReconfiguration provided by the candidate PSCell(s).

Option 3:	The source SN generates CPC. The source SN sets the execution condition. The source SN communicates with target SN and receives RRCReconfiguration provided by the candidate PSCell(s). The source SN generates the conditional reconfiguration message and provides it to the MN (possibly in a transparent container) for transmission to the UE.

2.1 Discussion of Option 1

Option 1: The MN generates CPC. The source SN sets the execution condition and communicates it to the MN. The MN generates the conditional reconfiguration message including the execution condition(s) provided by the source SN and RRCReconfiguration provided by the candidate PSCell(s).

Figure 1 is an illustration of signaling flow for SN initiated Inter-SN CPC based on Option 1. The figure follows the steps used in a conventional SN initiated SN change procedure as shown in Figure 10.5.1-2 of TS37.340. Note that Figure 1 shows the signaling flow up to the signaling of the conditional configuration for SN initiated Inter-SN CPC to the UE. Signaling upon the execution of CPC is not shown in the figure as the main focus of this discussion is on the generation of conditional reconfiguration for SN initiated Inter-SN CPC.

In this solution, the MN generates a CPC configuration, i.e., the IE ConditionalReconfiguration as an MN configuration based on reconfiguration per target candidate (denoted RRCReconfiguration** in Figure 1) and the execution condition per candidate cell. Reconfiguration** per target candidate is provided by each target candidate cell in response to a conditional SN Addition Request. The execution condition per candidate cell is provided by the S-SN in the conditional SN Change Required.

Figure 1: Configuration of SN-initiated CPC based on option 1.

Steps 1: Based on RRC measurement report received from the UE, source SN decides to initiate the CPC procedure. Source SN determines the set of target SNs for the CPC procedure, and the candidate target PSCells for each target SN. For each candidate target PSCell, source SN determines the CPC execution condition. In the SN Change Required message, source SN provides information relevant to CPC configuration to the MN. In addition to the content of conventional SN Change Required message, CPC execution condition for each candidate target PSCell is included in the SN Change Required message.

Steps 2: MN initiates the SN Addition procedure with the set of target SNs indicated in SN Change Required. As a base line, the content of SN addition Request is similar to the conventional SN Addition Request message.

Step 3:  The target SN generates RRCReconfiguration** for each candidate PSCell and provides it to the MN in SN addition request acknowledgement message. FFS on inclusion of multiple candidate cell configurations.

Step 4:  The MN generates an RRCReconfiguration to be provided to the UE including CPC configuration (as an MN configuration), mapping the execution condition configuration to an RRCReconfiguration** provided by the target SN for candidate PSCell.

Step 5: the UE provides RRCReconfigurationComplete message to the MN upon reception of RRCReconfiguration message.

Question 1: Companies are requested to comment whether Figure 1 is a reasonable representation of signalling flow for SN initiated Inter-SN CPC according to Option 1. For SN initiated CPC based on Option 1;

A:	CPC configuration is generated by the MN.

B:	SN provides the execution conditions to the MN.

C: the message applied upon CPC execution is in MN format.

| Company | Agree/not agree | Comments |
| --- | --- | --- |

2.2 Discussion of Option 3

Option 3:	The source SN generates CPC. The source SN sets the execution condition. The source SN communicates with target SN and receives RRCReconfiguration provided by the candidate PSCell(s). The source SN generates the conditional reconfiguration message and provides it to the MN (possibly in a transparent container) for transmission to the UE.

Option 3 requires communication between the source SN and the target SN. There are two methods for enabling communication between the source SN and the target SN [3].

	the communication between S-SN and T-SN is performed directly

	the communication between S-SN and T-SNs occurs via MN

As discussed in [3], there is no direct communication between the source SN and the target SN is supported today. Enabling direct communication between the source SN and the target SN has a significant specification impact, eg. Xn. Therefore [3] argues that a direct communication between the source SN and the target SN should be avoided.

Question 2: should direct communication be avoided between the source SN and the target SN?

| Company | Yes/no | Comments |
| --- | --- | --- |

Figure 2 shows an illustration of signaling flow for SN initiated Inter-SN CPC based on Option 3. It is assumed that there is no direct communication between the Source SN and target SN. In option 3), the source SN generates CPC. The source SN sets the execution condition. The source SN communicates with target SN and receives RRCReconfiguration provided by the candidate PSCell(s). The source SN generates the conditional reconfiguration message and provides it to the MN (possibly in a transparent container) for transmission to the UE. Some assumptions are made regarding the inter-node messages (step 4) in Figure 2 which need to be finally agreed and discussed in RAN3. The source SN sends an S-Node Change Required message. The MN initiates an SN addition procedure to a target candidate SN. The target SN generates RRCReconfigurtaion** for the candidate cell. The target SN sends S-Node Addition Request Ack message including CG-Config and the target generated RRCReconfigurtaion**. In Step 4, the MN sends the CG-Config, including the target generated RRCReconfigurtaion** to the source SN. Note that S-Node Change confirm message is modified to include CG-Config including the target generated RRCReconfigurtaion** (highlighted in red) in the illustration in Figure 2. This should be discussed and agreed in RAN3. The source SN generates an RRCReconfiguration* including CPC configuration, RRCReconfiguration** generated by the target SN. Steps as in Rel-16 CPC can be used for the delivery of the source SN generated RRCReconfiguration* message to the UE. In the illustration in Figure 2, SN initiated SN modification procedure is used to signal the source SN generated RRCReconfigurtaion* message to the UE via the MN (using SRB1).

Figure 2: Configuration of SN-initiated CPC based on option 3.

Steps 1: Based on RRC measurement report received from the UE, source SN decides to initiate the CPC procedure. Source SN determines the set of target SNs for the CPC procedure, and the candidate target PSCells for each target SN. For each candidate target PSCell, source SN determines the CPC execution condition but the execution condition is not provided to the MN. In the SN Change Required message, source SN provides information relevant to CPC configuration to the MN in a similar way as in conventional SN change required message.

Steps 2: MN initiates the SN Addition procedure with the set of target SNs indicated in SN Change Required. As a base line, the content of SN addition Request is similar to the conventional SN Addition Request message.

Step 3:  the target SN generates RRCReconfiguration** for each candidate PSCell and provides it to the MN in SN addition request acknowledgement message. FFS on inclusion of multiple candidate cell configurations.

Step 4:  The MN provides the CG-Config, including the target SN generated RRCReconfiguration** received from the target SN in step 3 to the source SN in S-Node Change Confirm message. This message and the content should be discussed in RAN3.

Step 5-8: The source SN generates an RRCReconfiguration* including CPC configuration, RRCReconfiguration** generated by the target SN and execution condition. Steps as in Rel-16 CPC can be used for the delivery of the source SN generated RRCReconfiguration* message to the UE under the assumption that MN is not involved with the configuration of CPC.

Question 3: Companies are requested to comment whether Figure 2 is a reasonable representation of signalling flow for SN initiated Inter-SN CPC according to Option 3. For SN initiated CPC based on Option 3;

A:	CPC configuration is generated by the source SN.

B:	SN does not provide the execution conditions to other NW nodes.

C: the message applied upon CPC execution is in source SN format.

| Company | Agree/not agree | Comments |
| --- | --- | --- |

2.3 Comparison of option 1 and option 3

In this section we discuss the main technical points for the operation of SN-Initiated Inter-SN CPC.

MN involvement

MN involvement of the procedure affects the generation and structure of the final CPC configuration message for Inter-SN CPC.

As discussed in [2,6], the UE needs to be provided with the MN generated sk-Counter in order to derive a new secondary KgNB for the operation of SN terminated DRBs and SRB3. Sk-Counter is provided to the UE as in an MN configuration in the conventional SN addition/change procedures. In option 1, MN generates the final conditional configuration, hence it can also provide the sk-Counter in MN configuration following the legacy signalling approach for sk-Counter.

It is questionable how sk-Counter can be provided to the UE in Option 3. Note that provisioning of sk-Counter was not an issue for Rel-16 CPC as security keys do not need to be refreshed for Intra-SN CPC. But for inter-SN CPC, sk-Counter should be provided to the UE.

It is clear how option 1 allows a key refresh upon an SN change triggered by CPC, as Sk counter is an MN/MCG configuration. However it is not clear how to provide sk-Counter for key refresh in Option 3.

Question 4: Companies are requested comment on how to provide sk-Counter for key refresh in Option 1 and Option 3.

| Company | Option 1 | Option 3 | Comments |
| --- | --- | --- | --- |

In conventional SN initiated SN change, the Radio bearer configuration could be changed due to that target SN may not be able to admit some of the SN terminated DRBs. As discussed in [2, 4], DRBs which could not be admitted by the target SN is desirable to be configured as MN terminated DRBs. It would be preferable to perform this change at execution time.

Considering final conditional configuration is performed by the MN, it is clear how the bearer configuration (reflecting the change of SN terminated bearer to MN terminated bearer) be signalled in Option 1. However it is not clear how the CPC configuration affecting MN bearer configuration can be signalled in Option 3.

Question 5: Companies are requested comment on how the bearer configuration (reflecting the change of SN terminated bearer to MN terminated bearer) be signalled in Option 1 and Option 3.

| Company | Option 1 | Option 3 | Comments |
| --- | --- | --- | --- |

[6] discusses another aspect of MN involvement at CPC, requirement on capability coordination to facilitate Inter SN CPC. Upon change of SN, the MN and Target SN interact may share some of the UE capabilities and that the nodes interact for this (capability coordination) purpose. E.g. the band combination and/ or feature set combination used by MN/ for MCG may need to be adjusted to facilitate configuration of the SCG configuration included in the condRRCConfig. This means that, the MN configuration may need modification to ensure that, together with the SCG configuration applied at execution time, it respects the UE capabilities. It would again be preferable to avoid that MN configuration should be restricted prior to conditional reconfiguration execution. I.e. if application of the MN configuration cannot be delayed until execution, MN has to restrict its configuration such that in conjunction with every conditional CPC candidate, UE capabilities are respected. This would result in performance loss. Hence, it should be possible to delay application of MN generated fields until CPC execution.

With Option 1, it is clear how to provide MN configuration, affecting capability coordination which could be applied at the CPC execution, to the UE. However it is not clear how this can be enable in Option 3.

Question 6: Companies are requested comment on how to provide MN configuration (affecting capability coordination which could be applied at the CPC execution) to the UE in Option 1 and Option 3.

| Company | Option 1 | Option 3 | Comments |
| --- | --- | --- | --- |

Signalling load for inter-node messages

Limitation in support of multiple candidate cell configurations in single inter-Node message was discussed in [2,5]. Currently, only one PSCell configuration can be provided in a single inter-Node message (RAN3 scope). If multiple candidate PSCell configurations are to be provided in a single inter-node message, RAN3 should be consulted.

The above issue of multiple candidate PSCell configurations occures if target SN requires providing multiple candidate PSCell in the same target SN. The issue of how to provide multiple candidate PSCell configurations by the target SN is seen in both Option 1 (step 3 in Figure 1) and Option 2 (step 3 in Figure 2).

The number of inter-node messages required for Option 3 is more than that of Option 1 as can be seen in Figure 1 and Figure 2, however the content of the messages are different in option 1 and option 3.

Considering that the target candidate PSCell is chosen by target SN, the MN or Source SN can’t perform the mapping of the execution condition and the candidate target SCG configuration until the MN or source SN received the candidate target SCG configuration in option 1 and option 3 respectively. In order to facilitate mapping of the execution condition and the target SCG configuration, Option 1 requires the provision of a list of execution conditions and candidate PSCells to the MN in step 1 of Figure 1. In option 3, provision of execution condition to the MN or target SN is not required. However, the candidate target SCG configuration should be forwarded to Source SN by the MN (step 4/5) in Figure 2.

Question 6: Considering the number of inter-node messages and signalling content in each message, companies are requested to comment on/compare the overhead of inter-node messages in option 1 and option 3.

| Company | Inter-node signaling in Option 1 (High/low) | Inter-node signaling in Option 3 (High/low) | Comments |
| --- | --- | --- | --- |

RAN3 involvement and Specification impact

The procedure in Option 1 reuses conventional SN change procedure. Even though, additional message content may need to be included in inter-node messages in Option 1 (step 1), option 1 does not require introducing new inter-node messages.

Option 3 uses Rel-16 Intra-SN CPC based signalling. Rel-16 intra-SN CPC only supported scenario where MN is not involved in the CPC configuration. As discussed in section 2.3, the MN involvement is required for Inter-SN CPC in order to provide sk-Counter, affected MN radio bearer configuration, etc. this would require additional modification to the Rel-16 CPC procedure in RAN2. Required modification on inert-node message to support Option 3 should be discussed in RAN3. Especially the message involved in step 4 in Figure 2 needs further discussion in RAN3.

Question 6: Considering the above discussion points (inter-node message content, enabling MN involvement in CPC configuration), companies are requested to comment on/compare the specification impact (in RAN2 and RAN3) for introduction of Option 1 and Option 3.

| Company | Specification impact of Option 1 (High/low) | Specification impact of Option 3 (High/low) | Comments |
| --- | --- | --- | --- |

Any other point to be considered for comparison of Option 1 and Option 3

2.4 Decision on option 1 or option 3 [Phase 2]

# 5	Conclusion

[TBC]

# 6	Reference

[1] R2-2009360	Summary of  [Post111-e][920][eDCCA] Conditional PSCell Change and Addition (CATT)	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

[2] R2-2010088	Progressing conditional configuration for R17	Samsung Telecommunications	discussion	Rel-17	LTE_NR_DC_enh2-Core

[3] R2-2010626	Further consideration for Conditional PSCell addition and change	NTT DOCOMO INC.	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

[4] R2-2010130	Configuration of Conditional PSCell addition/change	Qualcomm Incorporated	discussion	Rel-17

[5] R2-2010529	Regarding inter MN-SN signaling design for Conditional PSCell Addition	Intel Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

[6] R2-2010003	Conditional PSCell Change / Addition	Ericsson	discussion	LTE_NR_DC_enh2-Core

[7] R2-2009771	On Rel-17 Conditional PSCell Addition and Change (CPAC)	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

[8] R2-2010125	Discussion on support of conditional PSCell change/addition	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

[9] R2-2010373	Discussions about CPAC procedures	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

[10]R2-2009379	Discussion on conditional PSCell addition/change	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

[11] R2-2009596	Discussion on conditional PSCell change and addition	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

[12] R2-2009868	Issues on inter-SN CPC	Lenovo, Motorola Mobility	discussion	Rel-17

[13] R2-2009592	Discussion on inter-SN conditional PSCell change (SN initiated)	China Unicom	discussion	LTE_NR_DC_enh2-Core

[14] R2-2009358	Discussion on Further CPAC Enhancements	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

[15] R2-2009816	Framework of Inter-SN Conditional PSCell change	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

[16] R2-2009815	Conditional PSCell addition procedure	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

[17] R2-2009088	Conditional PSCell change / addition	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

[18] R2-2009158	CPC configuration number restriction	Spreadtrum Communications	discussion	Rel-17	LTE_NR_DC_enh2-Core

[19] R2-2009260	Coexistence of CHO and CPC	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core

[20] R2-2009285	CPAC failure handling discussio	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core

[21] R2-2009475	Discussion on conditional PSCell change and addition	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

[22] R2-2010248	Discussion on SN initiated CPC and CPAC Execution	ETRI	discussion	Rel-17	LTE_NR_DC_enh2-Core

[23] R2-2010282	Considerations of CPAC in Rel-17	LG Electronics	discussion	Rel-17	R2-2007985