3GPP TSG-RAN WG2 Meeting #112 electronic	draftR2-2010819

# Online, November 2nd – 13th 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1780 | rev | 3 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing capabilities for eMTC R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - RAN4 related optional UE features without capability signalling, according to R2-2006134, are missing: 1-1 Relaxed RRM measurements - Relaxation of RRM measurements for serving cell for UEs using MWUS for at least low mobility UEs 1-2 RSS-based measurement improvement in RRC_IDLE - Improved DL RSRP measurement accuracy through use of RSS in RRC_IDLE - In RAN4#96e the WF on RRM performance requirements was approved in R4-2012192 in which the following was agreed: - RSS based neighbour cell measurement conditions in IDLE and CONNECTED mode - Introduce capability signaling to indicate whether the UE is able to measure on neighbor cell RSS that is in the same NB that UE monitors. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add two new optional features without UE capability signalling in section 6.17 - Add new UE capability in section 4.3.6 Impact analysis Impacted functionality: eMTC Idle mode Inter-operability: No interoperability issues, since for (1) there is no need to indicate capability, however for (2) the NW will not be aware of the UE capability and may not be able to optimise accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN4 defined optional features are missing from the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.x (new), 6.17.x (new), 6.17.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.331 CR 4483 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.3.6.39	ce-MeasRSS-Dedicated-r16

This field indicates whether the UE supports improved DL RSRP measurement accuracy through use of RSS in RRC_CONNECTED when the UE is operating in coverage enhancement mode A or B as specified in 36.133 [16]. A UE indicating support of ce-MeasRSS-Dedicated-r16 shall also indicate support of ce-ModeA-r13.

#### 4.3.6.39a	ce-MeasRSS-InMonitoredNB-r16

This field indicates whether the UE supports measurement on a neighbour cell RSS that is in the same narrowband as the MPDCCH which the UE monitors, as specified in TS 36.133[16]. A UE indicating support of ce-MeasRSS-Dedicated-r16 shall also indicate support of ce-MeasRSS-Dedicated-r16.

NEXT CHANGE

### 6.17.10	DL channel quality reporting in Msg3

It is optional for UE to support DL channel quality reporting of the serving cell in Msg3, as specified in TS 36.321 [4]. This feature is only applicable if the UE supports ce-ModeA-r13.

### 6.17.x	Relaxed RRM measurements

It is optional for UE to support relaxation of RRM measurements for serving cell while using WUS, as specified in TS 36.133 [16]. This feature is only applicable if the UE supports ce-ModeA-r13 and (wakeUpSignal-r15 or groupWakeUpSignal-r16 or wakeUpSignal-TDD-r15 or groupWakeUpSignalTDD-r16).

### 6.17.y	RSS based measurement improvement

It is optional for UE to support improved DL RSRP measurement accuracy through use of RSS in RRC_IDLE as specified in TS 36.133 [16]. This feature is only applicable if the UE supports ce-ModeA-r13 and resynchronization signals.

END