3GPP TSG-RAN WG2 Meeting #113 electronic 	R2-2100172

Online, Jan 25 - Feb 5, 2021

Agenda Item:	8.1.2.1

Source: 	MediaTek Inc.

Title:  	HARQ operation to improve reliability for PTM transmission

Document for:	Discussion and decision

# Introduction

HARQ feedback based NR MBS reliability improvement for RRC_CONNECTED UEs was discussed in RAN1 #103 e-meeting. The following agreements were reached:

Agreements: For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following: - ACK/NACK based HARQ-ACK feedback for multicast, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, - FFS: PUCCH resource configuration. - NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From UEs within the group perspective, - FFS: PUCCH resource configuration. - To decide in RAN1#104-e whether or not to support only one or both of the above schemes For ACK/NACK based HARQ-ACK, PUCCH resource for HARQ-ACK from per UE perspective is, down-select one of the following options: - Option 1: shared PUCCH resource for unicast - Option 2: separate PUCCH resource for unicast For NACK-only based HARQ-ACK, PUCCH resource for HARQ-ACK from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast. - FFS PUCCH format Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between: - Option 1: DCI - Option 2: RRC configures enabling/disabling - Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling For slot-level repetition for group-common PDSCH, for indicating the repetition number, further down-select among: - Opt 1: by DCI - Opt 2: by RRC - Opt 3: by RRC+DCI

In this contribution, we will further discuss the detailed design about the NR multicast HARQ operation.

# HARQ operation to improve reliability for PTM

## Allowed HARQ Retransmission for PTM

It should be noted that the discussion of HARQ operation in the paper focuses on the PTM based HARQ transmission, as PTP HARQ operation can simply follow legacy principle.

At first, within PTM based HARQ transmission should we allow PTM retransmission at HARQ, or we just allow retransmission at PTP leg. Supporting HARQ based PTM retransmission may serve multiple UEs in one shot if multiple UEs did not decode the TB (i.e. Transport Block) successfully based on the initial HARQ transmission via PTM. It seems beneficial in terms of radio resource utilization efficiency. Then we suggest to allow PTM retransmission at HARQ.

Proposal 1: Support PTM retransmission at HARQ as a method to improve reliability of the transmission of MBS service.

When PTM retransmission at HARQ is confirmed for MBS service, we need to clarify which type(s) of HARQ retransmission should be allowed in order to improve the reliability at physical layer to support the transmission of MBS service.

During HARQ operation, HARQ retransmission can be performed blindly or based on HARQ feedback. There is also a possibility mix those two types of HARQ retransmissions together. Blind HARQ retransmission may have the benefit of ensuring low transmission latency but may sacrifice some radio resource utilization efficiency, since the current TB (i.e. Transport Block) may be successfully decoded by some UEs based on the initial HARQ transmission.

HARQ feedback based PTM retransmission at HARQ is a bit complex subtopic, since it means to collect all of the HARQ feedback from the UEs and then a rational needs to be defined to decide when to start PTM retransmission at HARQ. For example, if there are 25 UEs participating the reception of the MBS service, a threshold (e.g. 10 UEs replying NACK) can be defined for the start of PTM retransmission at HARQ. It is obvious that starting PTM retransmission at HARQ targeting only for a few UEs is not economic from radio resource utilization efficiency perspective. As can be seen, allowing HARQ feedback based PTM retransmission at HARQ can enable precise usage of radio resource at the cost of complexity.

According to the discussion above, both blind HARQ retransmission for PTM and HARQ feedback based PTM retransmission at HARQ have some merits. However, the combination of the above two approaches may present high complexity as there is a need of coordination between two types of HARQ retransmission for PTM, and then it is not preferred.

Proposal 2: Blind HARQ retransmission and HARQ feedback based HARQ retransmission are allowed for PTM transmission of MBS service.

Proposal 2a: The combination of blind HARQ retransmission and HARQ feedback based HARQ retransmission is not allowed for PTM transmission of MBS service.

Proposal 2b: Discuss the rational to enable PTM based HARQ retransmission according to HARQ feedback.

Following the same example as discussed above, if there are 25 UEs participating the reception of the MBS service, PTP retransmission at HARQ may be the best choice if only a few UEs reply HARQ NACK after initial PTM transmission or PTM retransmission(s). This is for purpose of radio resource utilization efficiency. Hence PTP based HARQ retransmission should be allowed as well. However the rational to enable PTP based HARQ retransmission (e.g. according to HARQ feedback.) deserves further study.

Proposal 3: PTP based HARQ retransmission should be allowed after initial PTM transmission or PTM retransmission(s).

Proposal 3a: Further study the rational to enable PTP based HARQ retransmission according to HARQ feedback.

## Coordinated HARQ transmission between PTM and PTP

In general, the HARQ entity of the MAC entity at gNB can assign a specific HARQ process for the transmission of a particular Transport Block of MBS data. The HARQ process generates HARQ initial transmission and/or retransmission in PTM mode to multiple UEs and the PTM based HARQ transmission is carried by multicast PDSCH (i.e. PTM PDSCH).  The PTM PDSCH is scrambled by a specific RNTI e.g. G-RNTI.

In addition, a UE specific HARQ process can generate needed HARQ retransmission for the Transport Block in PTP mode to a specific UE. The PTP based HARQ retransmission is carried by legacy unicast PDSCH and the unicast PDSCH is scrambled by a UE specific C-RNTI.

Proposal 4: Independent HARQ process is allocated at gNB to PTM and PTP for downlink multicast transmission.

The DCI used by the gNB to schedule the unicast PDSCH needs to carry some assistance information for the UE to understand the current unicast data is a retransmission of the previously PTM transmitted data.

In case there are multiple UEs that require HARQ retransmission for the Transport Block, multiple unicast HARQ processes need to run simultaneously to generate multiple HARQ retransmissions for the Transport Block in PTP mode to different UEs.

Two HARQ process or a combined HARQ process at UE side

In the UE side, if following the legacy approach, it can establish two independent HARQ processes to receive the PTM HARQ process and PTP process. However, as the UE is expected to correlate the correlate the Transport Blocks received from PTM HARQ process and PTP HARQ process, establishing two independent HARQ processes at the UE side to receive the Transport Blocks may result in difficult for the UE to perform soft combining. More precisely, from network perspective, the PTP HARQ process may transmit a different RV version of the TB following the initial PTM transmission and/or PTM retransmission, and then the UE needs to combine different RV versions for the same TB received from PTM HARQ process and HARQ process. However, traditionally, the HARQ combining is performed per HARQ process at the UE side. This means legacy HARQ reception processing principle will be broken if the UE establish two independent HARQ processes to receive the PTM HARQ process and PTP process.

In reality, the UE can initiate a combined HARQ process within the HARQ entity for the reception of corresponding Transport Block. A single/combined HARQ process at UE can correspond the two independent HARQ processes at gNB (PTM and PTP). In this manner, the UE can perform the HARQ combining based on different RV versions for the same TB received from PTM HARQ process and HARQ process following legacy HARQ reception processing principle, which is preferred.

Proposal 5: A combined HARQ process is allocated at UE to receive the data from both PTM and PTP HARQ process.

The abovementioned combined HARQ process at UE requires that different HARQ identities should be allocated at gNB side and UE side in order to allow this type of two-to-one HARQ process mapping. The gNB may carry the HARQ identity information of HARQ process for the transmission of the data within the DCI (for PTM transmission and/or PTP transmission) in order for the UE to associate the received data from both PTM based HARQ transmission and PTP based HARQ transmission with the same HARQ process at the UE side for such MBS data reception.

Proposal 6: DCI is used to inform the UE to correlate the Transport Blocks received from PTM HARQ process and PTP HARQ process.

The uplink HARQ feedback from the UE is taken by the gNB to decide if PTM based HARQ retransmission is needed for the corresponding TB and to decide if the PTP based HARQ retransmission is needed for the corresponding TB to a specific UE.

# Conclusion

The following proposals are made:

Proposal 1: Support PTM retransmission at HARQ as a method to improve reliability of the transmission of MBS service.

Proposal 2: Blind HARQ retransmission and HARQ feedback based HARQ retransmission are allowed for PTM transmission of MBS service.

Proposal 2a: The combination of blind HARQ retransmission and HARQ feedback based HARQ retransmission is not allowed for PTM transmission of MBS service.

Proposal 2b: Discuss the rational to enable PTM based HARQ retransmission according to HARQ feedback.

Proposal 3: PTP based HARQ retransmission should be allowed after initial PTM transmission or PTM retransmission(s).

Proposal 3a: Further study the rational to enable PTP based HARQ retransmission according to HARQ feedback.

Proposal 4: Independent HARQ process is allocated at gNB to PTM and PTP for downlink multicast transmission.

Proposal 5: A combined HARQ process is allocated at UE to receive the data from both PTM and PTP HARQ process.

Proposal 6: DCI is used to inform the UE to correlate the Transport Blocks received from PTM HARQ process and PTP HARQ process.

# References

[1]	Chairman’s Notes, RAN1 #103-e, Nov 2020.

[2]	R2-2009126 HARQ operation for NR MBS reliable transmission, MediaTek Inc.