3GPP TSG-RAN WG2 Meeting #113 electronic 	R2-2100173

Online, Jan 25 – Feb 5 2021

Agenda Item:	8.1.2.2

Source: 	MediaTek Inc.

Title:  	Dynamic PTM/PTP Switch

Document for:	Discussion and decision

# Introduction

During last RAN2 meeting (RAN#112e), there were discussions on MBS L2 Architecture with regard to the anchor layer for the dynamic switch of PTP and PTM dynamic switch. The majority companies suggest PDCP based PTP/PTM dynamic switch.

Chair: it seems there are two proposals on the table

1) P16P17 with PDCP as the anchor

2) To have also a Common PTP PTM RLC entity to easier support RLC AM for PTM.

Will come back to this discussion.

This paper discusses the further details of PTP and PTM dynamic switch mechanism based on the discussion at RAN2#112e.

# Triggers for PTM/PTP Switch

3GPP specified the basic eMBMS support of LTE since Rel-9 with the focus on MBSFN. At Rel-13, the Single-Cell Point to Multipoint (i.e. SC-PTM) was specified for LTE. Both MBSFN and SC-PTM transmission is supported based on the dedicated MBMS system architecture, where an MCE entity is located between RAN and CN. MCE is responsible for the determination of the transmission mode MBSFN or SC-PTM.

As a difference between NR multicast/broadcast and LTE multicast/broadcast, the delivery mode switching between PTP and PTM mode can only be performed in NR RAN system while supporting service continuity, even though the transmission mode over N2 interface (i.e. the adoption of IP multicast mode or IP unicast mode) is controlled by 5GC.

Without the MCE-like entity, the transmission mode decision is made within gNB for NR Multicast and Broadcast. According to the objective described within the WID, dynamic delivery mode switching between PTM and PTP needs to be supported. It should be necessary to support reliable and efficient delivery mode switching between PTM and PTP depending on the number of active devices receiving a specific content, their location, etc. The base station (i.e. gNB) can dynamically transfer a PTP bearer to PTM delivery bearer and vice versa and all the concerned UEs follow the same transmission method i.e. either PTM or PTP. It should be noted that the discussion is per NR multicast/broadcast service. In general, the triggers for delivery mode switch at cell level can be network implementation.

Proposal 1: As the baseline, the triggers for cell specific delivery mode switch between PTM and PTP can be network implementation.

One important aspect for NR multicast is to support reliable transmission of multicast services. When the gNB detects that there is UE(s) that did not successfully receive the data packets, there are two options for the handling at gNB side. At first option, the gNB can decide to initiate the retransmission via PTP to the UE. In this case, UE still receives the PTM transmission as other UEs do.  Alternatively, the gNB can move the UE out of the reception of PTM transmission and perform dedicated PTP transmission to the UE. In the latter case, the PTM transmission continues for other UEs.  We believe the former case presents a better resource utilization efficiency for the system if the radio of the packets that are subject to PTP retransmission is low. However, if the radio of the packets that require retransmission is high, which means the link adaption for the multicast transmission is too aggressive for this UE. The network should have the freedom to adjust the link adaption to improve the successful probability of the initial reception for this UE, or maintain the current link adaption for the majority UEs and remove the UE from this PTM transmission (then a PTP based initial transmission should be performed from the network to the UE). Both cases are network operation, but it may impact the UE reception behavior.

According to the description above, it is observed that PTM->PTP switch can be performed per UE and this type of PTM->PTP switch is triggered by the need of reliable multicast transmission. As a consequence, the PTM and PTP transmission may run simultaneously within a particular cell, since there may be other UEs receiving PTM transmission, when the concerned UE is moved from PTM to PTP.

Proposal 2: PTM->PTP switch can be triggered by the need of reliable multicast transmission.

# Service Continuity requirements for PTM/PTP switch

As the expectation of reliable MBS transmission is to achieve unicast alike performance, the data loss should be avoided during the switch (both PTM -> PTP switch and PTP -> PTM switch).

Proposal 3: lossless switch should be ensured during PTM/PTP switch in order to ensure service continuity for reliable MBS transmission.

In order to achieve lossless switch, the UE is expected to receive the unsuccessfully received data packets after the switch from the new transmission leg. This further requires the UE to enable data combining between the data received before PTM/PTP switch and the data received after PTM/PTP switch. That is to say, the UE needs to support redundant packets detection, packet reordering, packet discard when the PTM/PTP switch is implemented at L2.

Proposal 4: To support service continuity during dynamic PTM/PTP switch, the UE is required to receive the non-successfully received data packets (from previous leg) after the switch to the new leg.

Proposal 5: To support service continuity during dynamic PTM/PTP switch, the UE is required to enable data combining between the data received before PTM/PTP switch and the data received after PTM/PTP switch.

## PTM -> PTP switch

During the PTM->PTP switch, the data packets to be delivered by PTP mode needs to be consistent with the data packets as previously delivered by PTM mode, in order to ensure the service continuity.

Specifically, when the PTM->PTP switch is implemented at HARQ, the next multicast TB from MAC layer needs to be delivered to PTP HARQ process. The unsuccessfully delivered multicast TB via PTM HARQ process may need to be retransmitted by PTP HARQ process to the UE. The UE needs to monitor C-RNTI scrambled PDCCH and receive the data coming from the PTP HARQ process following the PTM->PTP switch order from network.

When the PTM->PTP switch is implemented at L2 (i.e. at PDCP), the next L2 packets transmitted to the UE via PTP leg needs to inherit the Sequence Numbering from PTM leg.

## PTP -> PTM switch

During the PTP->PTM switch, the data packets to be delivered by the PTM mode needs to be consistent with the  data packets as previously delivered by UE specific PTP transmission, in order to ensure the service continuity. The challenge here is that it is hard for the network to ensure the service continuity for multiple UEs simultaneously from PTP to PTM.

If the RAN node has already established PTM leg transmission other UEs, the precondition for service continuity for the current UE is that the PTM HARQ process and the PTP HARQ process always transmit the same TB when the PTP->PTM switch is implemented at HARQ, or PTM leg and PTP leg share the a common Sequence Numbering at PDCP when the PTP->PTM switch is supported at L2.

In summary, according to the discussion in both section 3.1 and section 3.2, in order to enable the service continuity during dynamic PTM/PTP (PTP->PTM switch and PTM->PTP switch) delivery mode switch, the following proposals are made on the requirements:

Proposal 6: To support service continuity during dynamic PTM/PTP switch, it is required to ensure consistent multicast TB delivery after PTM/PTP switch when the PTM/PTP switch is supported at HARQ.

Proposal 7: To support service continuity during dynamic PTM/PTP switch, it is required to enable consistent data packet sequence numbering after PTM/PTP switch when the PTM/PTP switch is supported at PDCP

# PDCP based PTM/PTP switch

## UE behaviour during PDCP based PTM/PTP switch

The dynamic PTM/PTP switch can be handled by PDCP layer. In this case, we need take the advantage of the L2 based uplink feedback. For example, specific to PTM->PTP Switch, the UE specific PTP transmission leg can be established as a L2 transmission leg. During PTM->PTP Switch, the PTM L2 functions at the RAN node can be simply disabled and then the common PDCP SN allocation function delivers the new data packets to each UE specific PDCP function established. From UE perspective, the protocol stack for data reception is kept but the monitoring on the PTM leg is not needed. The RAN node needs to notify this switch to the UE to perform such adaption from UE side.  There may be a case that the multicast RB is kept at RAN node after the switch from PTM->PTP, in order to support the PTM transmission to other UEs.

Proposal 8: The UE is notified to switch his monitoring between PTM transmission leg and PTP transmission leg during dynamic PTM/PTP switch.

## Transmission of Switch Order

In order to direct the UE to switch the reception mode from PTM to PTP, or vice versa, there is a need to transmit a switch order from network to the UE over Uu interface. Typically, the UE and other UEs are receiving the DL MBS transmission. So then a general distribution of the switch indication (or Switch Order) from the network to the UEs can be performed over the PTM transmission leg. As an alternative way, the switch indication can be sent to each UE via dedicated PTP signalling. The approach of PTP signalling for switch indication may be applicable to the case when the intention is to move the PTM to PTP for a particular UE or part of the UEs. And in this case, the other UEs should not be impacted. When moving the UE from PTP to PTM mode, it is natural to direct the UE via PTP signalling. MAC CE or High layer signalling may be used to carry the Switch Order over Uu from network to the UE, as L1 control signaling may be subject to the size limitation.

Proposal 9: Discuss if the Switch Order for delivery mode switch is over PTM transmission, PTP transmission or both (depending on the cases).

Proposal 10: Discuss if the Switch Order for delivery mode switch is based on MAC CE or high layer signalling.

# Conclusion

The following proposals are made:

Proposal 1: As the baseline, the triggers for cell specific delivery mode switch between PTM and PTP can be network implementation.

Proposal 2: PTM->PTP switch can be triggered by the need of reliable multicast transmission.

Proposal 3: lossless switch should be ensured during PTM/PTP switch in order to ensure service continuity for reliable MBS transmission.

Proposal 4: To support service continuity during dynamic PTM/PTP switch, the UE is required to receive the non-successfully received data packets (from previous leg) after the switch to the new leg.

Proposal 5: To support service continuity during dynamic PTM/PTP switch, the UE is required to enable data combining between the data received before PTM/PTP switch and the data received after PTM/PTP switch.

Proposal 6: To support service continuity during dynamic PTM/PTP switch, it is required to ensure consistent multicast TB delivery after PTM/PTP switch when the PTM/PTP switch is supported at HARQ.

Proposal 7: To support service continuity during dynamic PTM/PTP switch, it is required to enable consistent data packet sequence numbering after PTM/PTP switch when the PTM/PTP switch is supported at PDCP

Proposal 8: The UE is notified to switch his monitoring between PTM transmission leg and PTP transmission leg during dynamic PTM/PTP switch.

Proposal 9: Discuss if the Switch Order for delivery mode switch is over PTM transmission, PTP transmission or both (depending on the cases).

Proposal 10: Discuss if the Switch Order for delivery mode switch is based on MAC CE or high layer signalling.

# References

[1]	3GPP TS36.331

[2]	3GPP TS36.300

[3]	R2-2006575 UE Reception Model of MBS Radio Bearer and its Dynamic PTM/PTP switch