3GPP TSG-RAN WG2 Meeting #113-e 	R2-2100199

# Online, Jan 25 – Feb 5, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4547 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on inter-RAT MRO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1/ According to 38.306, the feature of inter-RAT MRO from NR to E-UTRA is defined as “Radio Link Failure Report for inter-RAT MRO EUTRA”. Same term can be used in 36.331. 2/ Accroding to 36.306, two features of inter-RAT MRO are defined i.e. UTRA and NR. 6.10.1 Radio Link Failure Report for inter-RAT MRO It is optional for UE to include previousUTRA-CellId and selectedUTRA-CellId in RLF-Report upon request from the network as specified in TS 36.331 [5]. 6.10.3 Radio Link Failure Report for inter-RAT MRO NR It is optional for UE to include previousNR-PCellId, failedNR-PCellId and nrReconnectCellId in RLF-Report upon request from the network as specified in TS 36.331 [5]. They are independent features and its distinction comes from the existence of the word ‘NR’. Such distinction should be clearly specified in 36.331 to avoid any ambiguity. 3/ For inter-RAT HO from E-UTRA to NR, MobilityFromEUTRACommand will not be embedded in RRCConnectionReconfiguration. 4/ Both intra-RAT HO failure and inter-RAT HO failure are specified in 5.3.10.6. For each HO failure case, the fields previousPCellId and timeConnFailure should be included according to the relevant RRC message i.e. RRCConnectionReconfiguration message for intra-RAT HO failure or MobilityFromEUTRACommand for inter-RAT HO failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Align the term of feature in 38.331 to in 36.331. 2/ Distinguish two independent features of RLF report for inter-RAT MRO in 36.331 as defined in 36.306. 3/ Delete “last RRCConnectionReconfiguration message including” in 5.4.3.5 for failed inter-RAT HO from E-UTRA to NR. 4/ Add MobilityFromEUTRACommand to the procedural text in 5.3.5.6 so that the fields previousPCellId and timeConnFailure are determined according to the MobilityFromEUTRACommand for failed inter-RAT HO from E-UTRA to NR. Impact analysis Architecture options LTE SA, (NG)EN-DC Impacted functionality Storage of handover failure information upon mobility from E-UTRA failure Inter-operability 1/ If the UE is implemented according to the CR while the network is not, there is no inter-operability issue 2/ If the network is implemented according to the CR while the UE is not, the network may detect wrong inter-RAT HO failure problems so inter-RAT MRO may not work properly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong source PCell Id and time elapsed since the last inter-RAT HO initialization until handover failure may be included in VarRLF-Report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.4, 5.3.11.3, 5.4.3.5, 5.3.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

START OF CHANGE

5.3.3.4	Reception of the RRCConnectionSetup by the UE

NOTE 1:	Prior to this, lower layer signalling is used to allocate a C-RNTI. For further details see TS 36.321 [6];

The UE shall:

1>	except when the UE connected to 5GC is a BL UE or UE in CE, if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from a suspended RRC connection:

2>	if the UE is resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.18:

3>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity for all established or suspended RBs, except for SRB0;

2>	discard the stored UE AS context and resumeIdentity;

2>	if stored, discard the stored nextHopChainingCount;

2>	if stored, discard the stored drb-ContinueROHC;

2>	indicate to upper layers fallback of the RRC connection;

1>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from RRC_INACTIVE:

2>	stop T380 if running;

2>	discard the stored UE Inactive AS context;

2> release rrc-InactiveConfig, if configured;

1>	if the UE connected to 5GC is a BL UE or UE in CE, and the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from a suspended RRC connection:

2>	discard the stored UE AS context and resumeIdentity;

2>	if stored, discard the stored nextHopChainingCount;

2>	if stored, discard the stored drb-ContinueROHC;

1>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from RRC_INACTIVE; or

1>	if the UE connected to 5GC is a BL UE or UE in CE, and the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest from a suspended RRC connection:

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0, including release of the RLC entities, of the associated PDCP entities and of SDAP entities;

2>	release the RRC configuration except for the default L1 parameter values, default MAC main configuration and CCCH;

2>	apply the default NR PDCP configuration as specified in TS 38.331 [82], clause 9.2.1.1 for SRB1;

2>	use NR PDCP for all subsequent messages received and sent by the UE via SRB1;

2>	indicate to upper layers fallback of the RRC connection;

1>	if the RRCConnectionSetup is received in response to an RRCEarlyDataRequest or RRCConnectionResumeRequest for transmission using PUR:

2>	instruct the associated MAC entity to start timeAlignmentTimer;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	if stored, discard the altFreqPriorities provided by the RRCConnectionRelease;

1>	if stored, discard the dedicated offset provided by the redirectedCarrierOffsetDedicated;

1>	stop timer T300;

1>	if T302 is running:

2>	stop timer T302;

2>	if the UE is connected to 5GC:

3>	perform the actions as specified in 5.3.16.4;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	stop timer T306, if running;

1>	stop timer T308, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	stop timer T350, if running;

1>	perform the actions as specified in 5.6.12.4;

1>	release rclwi-Configuration, if configured, as specified in 5.6.16.2;

1>	stop timer T360, if running;

1>	stop timer T322, if running;

1>	if timer T331 is running:

2>	stop timer T331;

2>	perform the actions as specified in 5.6.20.3;

1>	stop timer T323, if running;

1>	forward the dedicatedInfoNAS, if received, to the upper layers;

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	enter RRC_CONNECTED;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCConnectionSetupComplete message as follows:

2>	if the RRCConnectionSetup is received in response to an RRCConnectionResumeRequest:

3>	if upper layers provide an S-TMSI:

4>	set the s-TMSI to the value received from upper layers;

3>	else if upper layers provide a 5G-S-TMSI:

4>	if the UE is a NB-IoT UE:

5>	set the ng-5G-S-TMSI to the value received from upper layers;

4>	else:

5>	set the ng-5G-S-TMSI-Bits to ng-5G-S-TMSI with the value received from upper layers;

2>	else if upper layers provide a 5G-S-TMSI:

3>	except for NB-IoT, set the ng-5G-S-TMSI-Bits to ng-5G-S-TMSI-Part2 to the leftmost 8 bits of 5G-S-TMSI received from upper layers;

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35] for E-UTRA/EPC and TS 24.501 [95] for E-UTRA/5GC) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1 (or SystemInformationBlockType1-NB in NB-IoT);

2>	if upper layers provide the 'Registered MME', include and set the registeredMME as follows:

3>	if the PLMN identity of the 'Registered MME' is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredMME and set it to the value of the PLMN identity in the 'Registered MME' received from upper layers;

3>	set the mmegi and the mmec to the value received from upper layers;

2>	if upper layers provided the 'Registered MME':

3>	include and set the gummei-Type to the value provided by the upper layers;

2>	if upper layers provide the 'Registered AMF', include and set the registeredAMF as follows:

3>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

4>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

3>	set the amf-Identifier to AMF Identifier of the 'Registered AMF' received from upper layers;

2>	if upper layers provided the 'Registered AMF':

3>	include and set the guami-Type to the value provided by the upper layers;

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [27]):

3>	include the s-NSSAI-list and set the content to the values provided by the upper layers;

2>	if the UE supports CIoT EPS optimisation(s):

3>	include attachWithoutPDN-Connectivity if received from upper layers;

3>	include up-CIoT-EPS-Optimisation if received from upper layers;

3>	except for NB-IoT, include cp-CIoT-EPS-Optimisation if received from upper layers;

2>	if the UE supports CIoT 5GS optimisation(s):

3>	for NB-IoT, include ng-U-DataTransfer if received from upper layers;

3>	except for NB-IoT, include cp-CIoT-5GS-Optimisatoin if received from upper layers;

2>	if connecting as an RN:

3>	include the rn-SubframeConfigReq;

2>	if the RRCConnectionSetup is received in response to RRCEarlyDataRequest:

3>	set the dedicatedInfoNAS to a zero-length octet string;

2>	else:

3>	set the dedicatedInfoNAS to include the information received from upper layers;

2>	if the RRCConnectionSetup is not in response to transmission using PUR and the UE has a stored pur-Config including pur-ConfigID:

3>	include the stored pur-ConfigID;

2>	if the UE is connected to EPC:

3>	except for NB-IoT:

4>	if the UE supports RLF report for inter-RAT MRO EUTRA as defined in TS 38.306 [87], and if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 38.331 [82] and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 38.331 [82]:

5>	if reconnectCellId in VarRLF-Report of TS 38.331 [82] is not set:

6>	set timeUntilReconnection in VarRLF-Report of TS 38.331 [82] to the time that elapsed since the last radio link failure or handover failure;

6>	set eutraReconnectCellId in reconnectCellId in VarRLF-Report of TS 38.331 [82] to the global cell identity and the tracking area code of the PCell;

4>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

5>	if reconnectCellId in VarRLF-Report is not set:

6>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure;

6>	set eutraReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

5>	include rlf-InfoAvailable;

4>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableMBSFN;

4>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailable;

4>	if Bluetooth measurement results are included in the logged measurements the UE has available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableBT;

4>	if WLAN measurement results are included in the logged measurements the UE has available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableWLAN;

4>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

5>	include connEstFailInfoAvailable;

4>	include the mobilityState and set it to the mobility state (as specified in TS 36.304 [4]) of the UE just prior to entering RRC_CONNECTED state;

4>	if the UE has flight path information available:

5>	include flightPathInfoAvailable;

3>	for NB-IoT:

4>	if the UE has radio link failure information available in VarRLF-Report-NB and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report-NB:

5>	include rlf-InfoAvailable;

4>	if the UE has ANR measurements information available in VarANR-MeasReport-NB and if the RPLMN is included in plmn-IdentityList stored in VarANR-MeasReport-NB:

5>	include anr-InfoAvailable;

3>	include dcn-ID if a DCN-ID value (see TS 23.401 [41]) is received from upper layers;

2>	else (i.e. the UE is connected to 5GC):

3>	if the UE is a BL UE:

4>	include lte-M;

2>	except for NB-IoT:

3>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

4>	include the mobilityHistoryAvail;

3>	if the SIB2 contains idleModeMeasurements and the UE has E-UTRA idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

3>	if the SIB2 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information available in VarMeasIdleReport:

4>	include the idleMeasAvailable;

3>	if upper layers indicate that access to RLOS is initiated (see TS 23.401 [41] subclause 4.3.8.3):

4>	set rlos-Request to true;

2>	if UE needs UL gaps during continuous uplink transmission:

3>	include ue-CE-NeedULGaps;

2>	for NB-IoT:

3>	if the UE supports serving cell idle mode measurements reporting and servingCellMeasInfo is present in SystemInformationBlockType2-NB:

4>	set the measResultServCell to include the measurements of the serving cell;

NOTE 2:	The UE includes the latest results of the serving cell measurements as used for cell selection/ reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	if connecting as an IAB-node:

3>	include iab-NodeIndication;

1>	submit the RRCConnectionSetupComplete message to lower layers for transmission;

1>	the procedure ends.

NEXT CHANGE

5.3.11.3	Detection of radio link failure

The UE shall:

1>	in case any DAPS bearer is configured, only the target PCell is considered in the following;

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304 nor T311 is running; or

1>	upon indication from MCG RLC, which is allowed to be send on PCell, that the maximum number of retransmissions has been reached for an SRB or DRB:

2>	consider radio link failure to be detected for the MCG i.e. RLF;

2>	discard any segments of segmented RRC messages received;

2>	if the UE is configured with (NG)EN-DC; and

2>	if T316 is configured; and

2>	if SCG transmission is not suspended; and

2>	if NR PSCell change is not ongoing (i.e. T304 for the NR PSCell is not running as specified in TS 38.331 [82], clause 5.3.5.5.2, in (NG)EN-DC):

3>	initiate the MCG failure information procedure as specified in 5.6.26 to report MCG radio link failure;

2>	else:

3>	store the following radio link failure information in the VarRLF-Report (VarRLF-Report-NB in NB-IoT) by setting its fields as follows:

4>	clear the information included in VarRLF-Report (VarRLF-Report-NB in NB-IoT), if any;

4>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

4>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected radio link failure;

4>	except for NB-IoT, set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected radio link failure, and set its fields as follows;

5>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

5>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

5>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

5>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

4>	except for NB-IoT, if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	except for NB-IoT, if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if detailed location information is available, set the content of the locationInfo as follows:

5>	include the locationCoordinates;

5>	include the horizontalVelocity, if available;

4>	set the failedPCellId to the global cell identity, if available, and otherwise , except for NB-IoT, to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

4>	except for NB-IoT, set the tac-FailedPCell to the tracking area code, if available, of the PCell where radio link failure is detected;

4>	except for NB-IoT, if an RRCConnectionReconfiguration message including the mobilityControlInfo was received before the connection failure:

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned an intra E-UTRA handover:

6>	include the previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO NR:

6>	include the previousUTRA-CellId and set it to the physical cell identity, the carrier frequency and the global cell identity, if available, of the UTRA Cell in which the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

5>	if the last RRCConnectionReconfiguration message including the mobilityControlInfo concerned a handover to E-UTRA from NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO:

6>	include the previousNR-PCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3;

6>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo embedded in NR RRC message MobilityFromNRCommand message as specified in TS 38.331 [82] clause 5.4.3.3.

4>	except for NB-IoT, if the UE supports QCI1 indication in Radio Link Failure Report and has a DRB for which QCI is 1:

5>	include the drb-EstablishedWithQCI-1;

4>	except for NB-IoT, set the connectionFailureType to rlf;

4>	except for NB-IoT, set the c-RNTI to the C-RNTI used in the PCell;

4>	except for NB-IoT, set the rlf-Cause to the trigger for detecting radio link failure;

3>	if AS security has not been activated:

4>	if the UE is a NB-IoT UE:

5>	if the UE is connected to EPC and the UE supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation; or

5>	if the UE is connected to 5GC, the UE supports RRC connection re-establishment for the Control Plane CIoT 5GS optimisation and the UE is configured with a truncated 5G-S-TMSI:

6>	initiate the RRC connection re-establishment procedure as specified in 5.3.7;

5>	else:

6>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

4>	else:

5>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7;

In case of DC or NE-DC, the UE shall:

1>	upon T313 expiry; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC, which is allowed to be sent on PSCell, that the maximum number of retransmissions has been reached for an SCG, for a split DRB or for a split SRB:

2>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

2>	if the UE is configured with DC; or

2>	if the UE is configured with NE-DC and MCG transmission is not suspended:

3>	initiate the SCG failure information procedure as specified in 5.6.13 to report SCG radio link failure;

2>	else:

3>	initiate the connection re-establishment procedure as specified in TS 38.331 [82], clause 5.3.7.

In case of CA PDCP duplication, the UE shall:

1>	upon indication from an RLC entity, which is restricted to be sent on SCell only, that the maximum number of retransmissions has been reached:

2>	initiate the failure information procedure as specified in 5.6.21 to report RLC failure of type duplication;

If any DAPS bearer is configured and T304 is running, the UE shall:

1>	upon T310 expiry; or

1>	upon T312 expiry; or

1>	upon random access problem indication from source MCG MAC; or

1>	upon indication from source MCG RLC, which is allowed to be sent on source PCell, that the maximum number of retransmissions has been reached for an DRB:

2>	consider radio link failure to be detected for the source MCG;

2>	suspend the transmission of all DRBs in the source MCG;

2>	reset MAC for the source MCG;

2>	release the source connection;

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report (VarRLF-Report-NB in NB-IoT), 48 hours after the radio link failure is detected, upon power off or upon detach, and for NB-IoT, upon entering another RAT.

NEXT CHANGE

5.4.3.5	Mobility from E-UTRA failure

The UE shall:

1>	if T304 configured in the MobilityFromEUTRACommand message expires (mobility from E-UTRA failure); or

1>	if the UE does not succeed in establishing the connection to the target radio access technology; or

1>	if the UE is unable to comply with (part of) the configuration included in the MobilityFromEUTRACommand message; or

1>	if there is a protocol error in the inter RAT information included in the MobilityFromEUTRACommand message, causing the UE to fail the procedure according to the specifications applicable for the target RAT (i.e. according to subclause 5.3.5.6 if the targetRAT-Type in the received MobilityFromEUTRACommand is set to eutra):

2>	stop T304, if running;

2>	if the cs-FallbackIndicator in the MobilityFromEUTRACommand message was set to TRUE or e-CSFB was present:

3>	indicate to upper layers that the CS fallback procedure has failed;

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, mac-MainConfig and sps-Config;

2>	if MobilityFromEUTRACommand concerned a failed inter-RAT handover from E-UTRA to NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO NR:

3>	store handover failure information in VarRLF-Report according to 5.3.5.6;

2>	initiate the connection re-establishment procedure as specified in 5.3.7;

NOTE:	For enhanced CS fallback to CDMA2000 1xRTT, the above UE behavior applies only when the UE is attempting the enhanced 1xRTT CS fallback and connection to the target radio access technology fails or if the UE is attempting enhanced 1xRTT CS fallback along with concurrent mobility to CDMA2000 HRPD and connection to both the target radio access technologies fails.

NEXT CHANGE

5.3.5.6	T304 expiry (handover failure)

If T304 expires (handover failure), the UE shall:

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

1>	if no DAPS bearer is configured; or

1>	if any DAPS bearer is configured and radio link failure has been detected for the source MCG in accordance with 5.3.11.3:

2>	if attemptCondReconf is not configured:

3>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	else:

3>	revert back to the configuration used in the source PCell;

NOTE 1a:	In the context above, "the configuration" includes state variables and parameters of each radio bearer. PDCP entities associtated with RLC UM and SRB bearers are reset after the successful RRC connection re-establishment procedure according to clause 5.2 in TS 36.323 [8]. In the above, "the configuration" includes the RB configuration using NR PDCP, if configured (i.e. by nr-RadioBearerConfig1 and nr-RadioBearerConfig2).

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the source PCell based on measurements collected up to the moment the UE detected handover failure and in accordance with the following;

4>	if the UE includes rsrqResult, include the lastServCellRSRQ-Type;

3>	set the measResultNeighCells to include the best measured cells, other than the source PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE includes rsrqResult, include the rsrq-Type;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	if the UE was configured to perform measurement reporting, not related to NR sidelink communication, for one or more neighbouring NR frequencies, include the measResultListNR;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

3>	if available, set the logMeasResultListWLAN to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

3>	if available, set the logMeasResultListBT to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	if last RRCConnectionReconfiguration message including mobilityControlInfo concerned a failed intra-RAT handover (E-UTRA to E-UTRA):

4>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	else if last MobilityFromEUTRACommand concerned a failed inter-RAT handover from E-UTRA to NR and if the UE supports Radio Link Failure Report for Inter-RAT MRO NR:

4>	set the failedNR-PCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo or MobilityFromEUTRACommand was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo or MobilityFromEUTRACommand;

3>	set the connectionFailureType to 'hof';

3>	set the c-RNTI to the C-RNTI used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

1>	else (any DAPS bearer is configured and radio link failure has not been detected for the source MCG):

2>	release the MAC entity for the target PCell;

2>	for each DAPS bearer:

3>	re-establish the RLC entity for the target PCell;

3>	release the RLC entity or entities and the associated DTCH logical channel for the target PCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 36.323 [8];

2>	for each non-DAPS bearer:

3>	revert back to the configuration used for the DRB in the source PCell, including PDCP and RLC states and the security configuration;

2>	for each SRB:

3>	discard any PDCP SDUs along with the PDCP data PDUs for the source PCell;

3>	re-establish the RLC entity for the source PCell;

3>	release the PDCP entity for the target PCell;

3>	release the RLC entity and the associated DCCH logical channel for the target PCell;

2>	release the physical channel configuration for the target PCell;

2>	resume the SRBs for the source PCell;

2>	initiate the failure information procedure as specified in 5.6.21 to report a DAPS HO failure.

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

END OF CHANGE