3GPP TSG-RAN WG2 Meeting #113-e                                                     R2-2100204

Electronic, 25th Jan - 5th Feb, 2021‎‎

Source:	CATT

Title:	Miscellaneous Issus on Relay Discovery

Agenda Item:	8.7.3‎

Document for:	Discussion and Decision

Introduction

In email discussion [Post112-e][615][Relay] Update TR 38.836 (OPPO), companies have expressed their views on the draft TP.

[Post112-e][615][Relay] Update TR 38.836 (OPPO)

Scope: Update TR 38.836 with decisions of RAN2#112-e.  Rapporteurs of email discussions [Post111-e][627], [Post111-e][621], [Post111-e][622], [Post111-e][623], [AT112-e][610], [AT112-e][611], and [AT112-e][612] are asked to provide input text.

Intended outcome: Endorsed TP

Deadline:  Short

However, there are several remaining issues for sidelink relay discovery:

- Protocol stack for relay discovery;

- Issues from Editor Notes;

- Issues for Uu signal strength threshold(s).

In this contribution, the above issues will be discussed one by one.

Discussion

Protocol stack for relay discovery

In the latest SA2 spec TR 23.752[3], according to the high layer discovery signaling, there is a note as below:

- 8 Conclusions Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities. - 8.1 Key Issue #1: ProSe Direct discovery For Key Issue #1 (ProSe Direct discovery), the following aspects are concluded: - For discovery procedure over PC5 for commercial services and public safety, both model A and model B as defined in TS 23.303 [9] are recommended to be standardized. NOTE 1: Mechanism for discovering a UE-to-Network Relay and UE-to-UE Relay can be concluded in KI#3 and KI#4. - PC5 communication channel is used to carry the discovery message over PC5 and discovery message over PC5 is differentiated with other PC5 messages by AS layer. NOTE 2: Whether PC5-S signalling or any other new signalling in upper layer is used will be decided during the normative phase based on the protocol stack and the message structures/formats to be defined for PC5 direct discovery.

In RAN2#112-e, RAN2 had send LS to SA2 to consult whether discovery message could be taken as PC5-S signaling or other new signaling in upper layer. In the reply LS, SA2 provided feedback on the question, which is listed as below:

SA2 thanks RAN2 for the reply LS (S2-2008500/R2-2010883) to S2-2006587. SA2 would like to provide feedback on the question and progress as below: 1) Feedback on the question Q1: Whether discovery message could be taken as PC5-S signalling or other new signalling in upper layer? A1: SA2 agreed that direct discovery message will be taken as new signalling in ProSe layer separately from PC5-S signalling. ProSe layer will indicate to AS layer whether the signalling is discovery message or PC5-S signalling.

According to the reply LS and the latest SA2 spec TR 23.752, there are two different interpretations:

- Interpretataion1: the direct discovery message is not PC5-S signaling, it is a new signaling, ProSe layer will indicate to AS layer whether the signaling is a discovery message or PC5-S signaling.

- Interpretataion2: the direct discovery message is still a PC5-S signaling, but a new one PC5-S signaling separately from the existing PC5-S signaling.

The signaling type of sidelink discovery will affect the AS layer protocol stack design, e.g., whether a new protocol stack different from PC5-S should be introduced. But in the current RAN TR 38.836[2], the description about the protocol stack is as below:

The protocol stack of discovery message is similar or identical to PC5-S signalling as illustrated in Figure 16.9.2.1-2 of 38.300 [4].

The above description is not proper if the direct discovery message is not a PC5-S signaling. Taking the deadline for this SI into account, it had better send LS to SA2 to check which interpretation above is the precise understanding of SA2, and in the meanwhile, capture that the protocol stack for sidelink relay discovery can be left to WI in TR38.836 instead of the current description.

Proposal 1: Capture in TR38.836 that the protocol stack design for sidelink relay discovery can be left to WI phase.

Proposal 2：Send LS to SA2 to check which interpretation below is the precise understanding of SA2:

- Interpretataion1: the direct discovery message is not PC5-S signaling, it is a new signaling. ProSe layer will indicate to AS layer whether the signaling is a discovery message or PC5-S signaling.

- Interpretataion2: the direct discovery message is still a PC5-S signaling, but a new PC5-S signaling separately from the existing PC5-S signaling. ProSe layer will indicate to AS layer the detailed PC5-S signaling type to AS.

Issues from Editor Notes

In section 4.2 of TR 38.836[2], there are two editor notes:

Editor note: For Remote UE out of coverage, it is FFS whether transmission of discovery message is based on configuration from network if the Remote UE is already connected with network through a Relay UE. Editor note: For Remote UE in RRC_CONNECTED, the detail of configuration provided by serving gNB is FFS.

For the first editor note, notice that for L3 relay solution, it was already made agreement on the configuration for remote UE out of coverage in RAN2#112-e:

Proposal 14: for L3 solution, it is not feasible for serving gNB to configure an out of coverage remote UE with radio configuration for transmission of discovery message

For L2 relay solution, although it can work for serving gNB to configure an out of coverage remote UE with radio configuration for transmission of discovery message via relay. But there seems no benefit. the remote UE seeking for a sidelink relay may  experience bad Uu channel, under this condition, the configuration from network is not always possible, while pre-configuration can always be feasible. Hence, from the robust, it is proposed that pre-configuration is also used for L2 solution. The description of TR38.836 should be updated to reflect this preference.

Proposal 3: For L2 sidelink relay, OOC remote UE will use pre-configured resource for sidelink relay discovery message transmission instead of acquiring it from gNB.

Proposal 4: Capture the above agreement in the TR38.836.

For the second editor note, remote UE in RRC_CONNECTION may receive the configuration via dedicated RRC signaling and/or SIB. Reference to the LTE legacy, the configuration may include:

  - The thresholds, which indicate when a sidelink remote UE that is in network coverage mayperform sidelink relay discovery;

  - Offset, indicates an offset below the threshold；

  - The sidelink relay discovery resources and so on.

The detail of configuration provided by serving gNB has no impact on selection of L2 or L3 relay takes the deadline of this SI into account, it is proposed to remove the editor note and leave the detail of configuration to WI phase.

Proposal 5: Leave the detail of configuration for remote UE in RRC_CONNECTED to WI phase, and remove the editor note in TR38.836.

Issue for Uu signal strength threshold(s)

In section 4.2 of TR 38.836[2], the Relay UE needs to be within a minimum and a maximum Uu signal strength threshold(s) if provided by gNB before it can transmit discovery message when in RRC_IDLE or in RRC_INACTIVE state.

- 4.2 Discovery Model A and model B discovery model as defined in clause 5.3.1.2 of TS 23.303 [3] are taken as a working assumption for both UE-to-Network Relay and UE-to-UE Relay. The protocol stack of discovery message is similar or identical to PC5-S signalling as illustrated in Figure 16.9.2.1-2 of 38.300 [4]. For relay UE of UE-to-Network Relay, - The Relay UE needs to be within a minimum and a maximum Uu signal strength threshold(s) if provided by gNB before it can transmit discovery message when in RRC_IDLE or in RRC_INACTIVE state.

In RAN2#111-e, the following agreement is made:

[Easy]Proposal11: For U2N relay, LTE principle i.e. one lower threshold and one upper threshold can be reused for relay UE in IDLE/INACTIVE state to decide whether it is allowed to transmit/receive discovery message

In legacy LTE protocol, the two thresholds are both optional, as can be seen in the TS36.331, which is shown below:

TS36.331 SystemInformationBlockType19-r12 ::= SEQUENCE { discConfig-r12 SEQUENCE { discRxPool-r12 SL-DiscRxPoolList-r12, discTxPoolCommon-r12 SL-DiscTxPoolList-r12 OPTIONAL, -- Need OR discTxPowerInfo-r12 SL-DiscTxPowerInfoList-r12 OPTIONAL, -- Cond Tx discSyncConfig-r12 SL-SyncConfigList-r12 OPTIONAL -- Need OR } OPTIONAL, -- Need OR discInterFreqList-r12 SL-CarrierFreqInfoList-r12 OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ discConfig-v1310 SEQUENCE { discInterFreqList-v1310 SL-CarrierFreqInfoList-v1310 OPTIONAL, -- Need OR gapRequestsAllowedCommon ENUMERATED {true} OPTIONAL -- Need OR } OPTIONAL, -- Need OR discConfigRelay-r13 SEQUENCE { relayUE-Config-r13 SL-DiscConfigRelayUE-r13, remoteUE-Config-r13 SL-DiscConfigRemoteUE-r13 } OPTIONAL, -- Need OR discConfigPS-13 SEQUENCE { discRxPoolPS-r13 SL-DiscRxPoolList-r12, discTxPoolPS-Common-r13 SL-DiscTxPoolList-r12 OPTIONAL -- Need OR } OPTIONAL -- Need OR ]] } SL-CarrierFreqInfoList-r12 ::= SEQUENCE (SIZE (1..maxFreq)) OF SL-CarrierFreqInfo-r12 SL-CarrierFreqInfoList-v1310 ::= SEQUENCE (SIZE (1..maxFreq)) OF SL-CarrierFreqInfo-v1310 SL-CarrierFreqInfo-r12::= SEQUENCE { carrierFreq-r12 ARFCN-ValueEUTRA-r9, plmn-IdentityList-r12 PLMN-IdentityList4-r12 OPTIONAL -- Need OP } SL-DiscConfigRelayUE-r13 ::= SEQUENCE { threshHigh-r13 RSRP-RangeSL4-r13 OPTIONAL, -- Need OR threshLow-r13 RSRP-RangeSL4-r13 OPTIONAL, -- Need OR hystMax-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12, dBinf} OPTIONAL, -- Cond ThreshHigh hystMin-r13 ENUMERATED {dB0, dB3, dB6, dB9, dB12} OPTIONAL -- Cond ThreshLow }

It is proposed to modify the TR38.836 to correctly reflect the RAN2 agreement reached on RAN2#111-e meeting.

Proposal 6: Modify the TR 38.836 to correctly reflect the agreement reached on RAN2#111-e meeting.

Conclusion

According to the above discussion, the observations and proposals are as follows:

Proposal 1: Capture in TR38.836 that the protocol stack design for sidelink relay discovery can be left to WI phase.

Proposal 2：Send LS to SA2 to check which interpretation below is the precise understanding of SA2:

- Interpretataion1: the direct discovery message is not PC5-S signaling, it is a new signaling. ProSe layer will indicate to AS layer whether the signaling is a discovery message or PC5-S signaling.

- Interpretataion2: the direct discovery message is still a PC5-S signaling, but a new PC5-S signaling separately from the existing PC5-S signaling. ProSe layer will indicate to AS layer the detailed PC5-S signaling type to AS.

Proposal 3: For L2 sidelink relay, OOC remote UE will use pre-configured resource for sidelink relay discovery message transmission instead of acquiring it from gNB.

Proposal 4: Capture the above agreement in the TR38.836.

Proposal 5: Leave the detail of configuration for remote UE in RRC_CONNECTED to WI phase, and remove the editor note in TR38.836.

Proposal 6: Modify the TR 38.836 to correctly reflect the agreement reached on RAN2#111-e meeting.

TP corresponding to the above proposals are provided in section 5.

Reference

- R2-2100070_S2-2009229 Reply LS to Reply LS on Direct Discovery and Relay SA2

- R2-201xxxx  TR 38.836 V1.0.0

- 3GPP TR 23.752 V0.7.0

Text proposal

* * * Start of changes * * *

- 4.2	Discovery

Model A and model B discovery model as defined in clause 5.3.1.2 of TS 23.303 [3] are taken as a working assumption for both UE-to-Network Relay and UE-to-UE Relay. The protocol stack of discovery message can be left to WI phase.

For relay UE of UE-to-Network Relay,

-	the Relay UE needs to respect a minimum and/or a maximum Uu signal strength threshold(s) if provided by gNB before it can transmit discovery message when in RRC_IDLE or in RRC_INACTIVE state.

-	NR sidelink communication configuration provided by gNB is necessary for a Relay UE to transmit discovery message in all RRC states.

-	Relay UE supporting L3 UE-to-Network Relay is allowed to transmit discovery message based on at least pre-configuration when it is connected to a gNB which is not capable for sidelink relay operation, in case its serving carrier is not shared with carrier for sidelink operation.

-	Relay UE supporting L2 UE-to-Network Relay should be always connected to a gNBwhich is capable for sidelink relay operation for relay operation.

For remote UE of UE-to-Network Relay,

-	The Remote UE in RRC_IDLE and RRC_INACTIVE state is allowed to transmit discovery message if measured signal strength of serving cell is lower than a configured threshold.

-	Whether Remote UE in RRC_CONNECTED is allowed to transmit discovery is based on configuration provided by serving gNB.

-	Remote UE out of coverage is always allowed to transmit discovery message based on pre-configuration.

-	Remote UE supporting L2 or L3 UE-to-Network Relay is allowed to transmit discovery message based on at least pre-configuration when it is directly connected to a gNB which is not capable for sidelink relay operation, in case its serving carrier is not shared with SL carrier.

-	For Remote UE supporting L3 UE-to-Network Relay which is connected to a gNB indirectly, it is not feasible for the serving gNB to provide radio configuration to transmit discovery message.

A gNB which is capable for sidelink relay operation is a gNB which provides configuration for relay operation e.g. at least for transmission of discovery message. A gNB which is not capable for sidelink relay operation is a gNB which doesn’t provide any configuration for relay operation.

Further details on the configuration for remote UE in RRC_CONNECTED can be discussed in WI phase.

Resource pool to transmit discovery message can be either shared with or separated from resource pool for data transmission.

-	In case of shared resource pool, a new LCID is introduced for discovery message, i.e., discovery message is carried by a new SL SRB.

-	Within separated resource pool, discovery messages are treated equally with each other during LCP procedure.

* * * End of changes * * *