3GPP TSG-RAN WG2 Meeting #113-e	                                                    R2-2100225

Online, Jan 25 – Feb 5, 2021

Source:	CATT

Title:	Consideration on topology-wide fairness, multi-hop latency and congestion mitigation

Agenda Item:	8.4.2

Document for:	Discussion and Decision

Introduction

One of the objectives in eIAB WID is to consider topology-wide fairness, multi-hop latency and congestion mitigation [1].

After the last RAN2#112-e meeting, there is an email discussion#065 to identify the technical issues to address the objective in RAN2 scope. Based on the email discussion, we will further discuss the following issues in this contribution.

Topology-wide fairness:

- Issue 1: The scheduler of an IAB node does not have all the information needed (e.g. link quality across multiple hops) to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality.

Multi-hop latency:

- Issue 2: one hop latency consideration and reliably packet discarding by IAB node.

Congestion mitigation:

- Issue 3: Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets.(DL flow control enhancement)

- Issue 4: Child node keeps requesting UL resources and/or allocating UL resources to its descendant nodes, even if the parent node is experiencing upstream congestion, as there is no UL HbH flow control in Rel-16 (apart from uplink scheduling which serves this purpose, but up to a point). (UL Flow control)

Discussion

Topology-wide fairness

Based on the email discussion#065 [2], the first issue is that the scheduler of an IAB node does not have all the information needed to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality.

What information IAB node needs is based on unified understanding of fairness. In our point, Topology-wide fairness should be distinguished with load balancing. Load balancing focus on IAB-node that should schedule the child IAB-node and UE fairly. Topology-wide fairness requests that UE with different hops have the same QoS for the same service regardless of where a UE attaches to the IAB network. To achieve the fairness in R17, IAB node should maintain related topology information, such as remaining hops in the upstream or downstream.

The parameter PDB for BH RLC channel defines the upper bound for the time that a packet may be delayed between the gNB-DU and its child IAB-MT [3]. Generally speaking, when CU calculates PDB for each hop according E2E PDB, it should leave margin value for each hop to avoid excessively local packet discarding. That is, in IAB architecture, for same service, sum of PDBs of multi-hop in the path should be larger than the PDB of UE accessed to donor directly. It means with same PDB in one hop, the service with smaller remaining hop number has less remaining time for E2E PDB actually. Therefore, with same PDB and other QoS parameters (such as same priority), the BH RLC channel with smaller remaining hop number should be scheduled prioritized over the BH RLC channel with larger remaining hop number.

Proposal 1: Topology-wide fairness requires that UE experience the same QoS for the same service regardless of the number of hops between the UE and the Donor CU.

Proposal 2: IAB-node should maintain remaining hops of BH RLC channel in the upstream or downstream, to achieve the topology-wide fairness in R17.

Multi-hop latency

As mentioned in section 2.1, PDB for BH RLC channel is configured by CU to the IAB DU for current hop. In DL, IAB node can discard the packet(s) exceeding PDB locally. In UL, packet discard Timer at BAP entity was suggested in [4] to let IAB node MT discard packet(s) locally. However, considering PDB is configured per BH RLC channel. The UL packet discard timer should be configured per BH RLC and operated in BH RLC entity. The BH RLC discard timer is similar to PDCP discardTimer. When a RLC SDU is delivered to BH RLC entity, the BH RLC discard timer is started. If the packet has not been delivered to lower layer when the BH RLC discard timer is expired, the packet should be discarded by the BH RLC entity. The benefit of BH RLC discard timer is to save BH UL resource.

Proposal 3: BH RLC discard timer operated in BH RLC entity should be introduced to save BH UL resource.

Congestion mitigation

In R16 DL HbH flow control mechanism, parent IAB-node will reduce the data transmission of child IAB-node after receiving the DL HbH flow control message from child IAB-node. The DL HbH flow control mechanism in R16 may cause the long-term congestion of parent IAB-node, because the data transmission of egress link is reduced and the data transmission of ingress link isn’t reduced. If ancestor IAB-node can know the congestion of child IAB-node to reduce the transmission between ancestor and parent IAB-node, the long term congestion will be reduced.

It is supported that parent IAB-node forwarding the DL HbH flow control message from child IAB-node to ancestor IAB-node, which can make ancestor IAB-node reduce the data transmission of parent IAB-node to avoid the congestion of parent IAB-node. It can reduce the DL long-term congestion of parent IAB-node.

Proposal 4: RAN2 support parent IAB-node forwarding the DL HbH flow control message from child IAB-node to ancestor IAB-node.

In R16, UL HbH flow control is not specified. UL congestion mitigation depends on the implementation. Parent IAB-node will reduce the UL grant of child IAB-node if parent IAB-node is congested.

As shown in the figure below, if IAB-node A (or PATH 1) is congested, IAB-node A will reduce the UL grant of child IAB-node C. That would cause the long-term congestion of child IAB-node C, because the data transmission of egress link is reduced and the data transmission of ingress link isn’t reduced. And then PATH 2 is going to be affected due to the PATH 1. In R17, it can be supported the UL flow control feedback from IAB-node A(parent IAB-node) to IAB-node C(child IAB-node) and IAB-node C forwarding the UL flow control feedback to IAB-node D(descent IAB-node). When IAB-node C and IAB-node D receive the UL flow control feedback, they will reduce the transmission of PATH 1(congestion path).

In R17, it should be supported that child IAB-node also reduced the UL grant of descendant IAB-node when it receives the UL HbH flow control feedback. This requires parent IAB-node should indicate the UL HbH Flow control message to child IAB-node. It can reduce the UL long-term congestion of child IAB-node, due to reducing the data transmission of descendant IAB-node.

It is supported that child IAB-node forwarding the UL HbH flow control message from parent IAB-node to descent IAB-node, which can reduce the congestion path correctly and avoid affecting other paths.

Proposal 5: In R17, UL HbH Flow Control can be supported.

Proposal 6: RAN2 support child IAB-node forwarding the UL HbH flow control message from parent IAB-node to descent IAB-node.

R16 DL HbH Flow control is a local mechanism, which can mitigate the congestion to the parent IAB-node. Since it can be applied in a hop-by-hop manner, the congestion will finally reach the IAB donor-DU. If local re-routing can be performed after IAB-node receiving the DL HbH Flow control indication, IAB-node will select another path for the data transmission. Local re-routing reduces the ingress link data transmission of child IAB-node and it doesn’t cause the congestion of parent IAB-node. Local re-routing will mitigate the long-term congestion of parent IAB-node if parent IAB-node can perform the local re-routing. In R16, local re-routing is only performed when BH link is RLF. Thus, the enhancements on local re-routing mechanism‎ can be supported in R17.

If UL HbH Flow control is supported in R17, the local re-routing triggered by HbH Flow control can also be performed to mitigate the UL congestion.

Proposal 7: Upon receiving HbH Flow control indication, the local re-routing can be performed to mitigate the congestion.

Conclusion

Based on the discussion in section 2, we propose:

Proposal 1: Topology-wide fairness requires that UE experience the same QoS for the same service regardless of the number of hops between the UE and the Donor CU.

Proposal 2: IAB-node should maintain remaining hops of BH RLC channel in the upstream or downstream, to achieve the topology-wide fairness in R17.

Proposal 3: BH RLC discard timer operated in BH RLC entity should be introduced to save BH UL resource.

Proposal 4: RAN2 support parent IAB-node forwarding the DL HbH flow control message from child IAB-node to ancestor IAB-node.

Proposal 5: In R17, UL HbH Flow Control can be supported.

Proposal 6: RAN2 support child IAB-node forwarding the UL HbH flow control message from parent IAB-node to descent IAB-node.

Proposal 7: Upon receiving HbH Flow control indication, the local re-routing can be performed to mitigate the congestion.

Reference

- RP-201293, New WID on Enhancements to Integrated Access and Backhaul

- [Post112-e][065][eIAB] Fairness Latency Congestion (Samsung)

- TS38.473, F1 application protocol (F1AP), V16.4.0

- R2-2009388, Discussion on topology-wide fairness, multi-hop latency and congestion mitigation, ZTE