3GPP TSG-RAN WG2 Meeting #113                                                       R2-2100236

Electronic, 25th Jan - 5th  Feb, 2021

Source:	CATT

Title:	Sidelink DRX Timer Maintenance and Active Time Definition

Agenda Item:	8.15.2.1

Document for:	Discussion and Decision

Introduction

In RAN2#112-e meeting, the sidelink DRX was initially discussed and reached the following agreements:

Agreements on SL DRX:

1: 	Sidelink DRX needs to support sidelink communications for both in and out of network’s coverage scenarios.

2:	RAN2 will prioritize normal use case without consideration of relay UE use case in Rel-17.

3:	Support SL DRX for all casting types.

4:	If a UE is in SL active time, UE should monitor PSCCH. FFS on PSSCH. FFS for sensing impacts.

5:	RAN2 is not going to introduce SL paging and SL PO for SL DRX.

6:	As baseline, for Sidelink DRX for SL unicast, it is proposed to inherit and use timers similar to what are used in Uu DRX. FFS for SL broadcast/groupcast. FFS on detailed timers.

7:	Working assumption: SL DRX should take PSCCH monitoring also for sensing (in addition to data reception) into account if SL DRX is used.

8:	Support of long DRX cycle for SL unicast should be assumed as a baseline. FFS on the need of short DRX cycle.

9:	Deprioritize SL WUS from RAN2 point of view in Rel-17.

In this contribution, the sidelink DRX timer maintenance and active time definition will be further discussed.

Discussion

DRX timer maintainence for sidelink unicast

In Uu, DRX only controls the DL PDCCH monitoring. UE UL transmission is not restricted by DRX since gNB is always on. That is to say, for Uu DRX, either the gNB or the UE only needs to maintain one set of DRX timers which is only related to UE PDCCH monitoring.

But for sidelink unicast, the communication peers are both UEs and power saving are both need to be considered for the peer UEs. In addition, as proposed in [4] Tx UE centric DRX configuration is preferred. Hence, each direction of the sidelink connection may have different active time. In order to determine the active time of the UE for sidelink unicast, one UE should maintain two sets of sidelink DRX timers for each sidelink unicast connection, and each set is corresponding to one direction of the sidelink unicast connection. E.g., one set for the sidelink transmission and another set for the sidelink reception.

Proposal 1: One set of sidelink DRX timers should be maintained for each direction of the sidelink unicast connection.

In the following, we will further analyse which DRX timers should be contained in each set of DRX timers. For sidelink unicast, in the last RAN2 meeting, it was agreed to inherit and use timers similar to what are used in Uu DRX. In NR Uu, the following DRX timers were introduced:

- drx-onDurationTimer

- drx-InactivityTimer

- drx-HARQ-RTT-TimerDL

- drx-HARQ-RTT-TimerDL

- drx-RetransmissionTimerDL

- drx-RetransmissionTimerUL

- drx-ShortCycleTimer.

In the following, we will further analyse whether the above Uu DRX timers can be reused in sidelink unicast one by one:

drx-onDurationTimer

Regarding to the drx-onDurationTimer, in Uu, it represents the duration at the beginning of a DRX cycle and its length unit is ms.

For NR sidelink, in order to determine the onduration period, it is reasonable to inherit it from Uu to sidelink, and its maintenance can be same as Uu.

Proposal 2: For sidelink unicast, drx-ondurationtimer should be introduced and its maintenance should be same as Uu.

drx-InactivityTimer

Regarding to the drx-InactivityTimer, in Uu, it represents the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity. It is started or restarted in the first symbol after the end of the PDCCH reception. The length unit of this timer is ms.

For NR sidelink, although SCI can indicate the resource reserved for initial transmission and retransmission, but considering re-evaluation or pre-emption may happen, the resource actually used may be changed. Hence, in order to ensure Rx UE can still receive the SCI in these exceptional cases, it had better introduce drx-InactivityTimer in sidelink. Its length unit can also be in ms.

Proposal 3: For sidelink unicast, drx-InactivityTimer should be introduced.

If drx-InactivityTimer is introduced in sidelink, how to maintain it should be further discussed. Based on Proposal 1, once SCI is received, the drx-InactivityTimer in Rx UE side should be started; and once SCI is transmitted, the drx-InactivityTimer  in Tx UE side should be started.

Proposal 4: once SCI is received, the drx-InactivityTimer in Rx UE side should be started; and once SCI is transmitted, the drx-InactivityTimer in Tx UE side should be started.

drx-HARQ-RTT-Timer

In Uu, both drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL are introduced. The drx-HARQ-RTT-TimerDL represents the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity; and the drx-HARQ-RTT-TimerUL represents the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity. Its length unit is symbol.

In NR sidelink, considering resource reservation for retransmission are supported for both mode 1 and mode 2 resource allocation, hence it needs to discuss whether it is necessary to introduce the HARQ RTT timer. At most cases, if resource for retransmission is reserved in SCI, UE does not need to maintain the HARQ related timer since Rx UE can know when to awake for retransmission reception through decoding SCI. But it should also be noted that in NR V2X, resource re-evaluation and pre-emption are also supported, in this case, the sidelink retransmission resource can be changed. If resource re-evaluation and pre-emption happens and there is no HARQ related timers, UE may miss the SCI reception for retransmission. Hence, it had better maintain the drx-HARQ-RTT-Timer in sidelink.

Proposal 5: For sidelink unicast, drx-HARQ-RTT-Timer should be introduced.

If drx-HARQ-RTT-Timer is introduced in sidelink, how to maintain it should be further discussed. Different from Uu, in NR sidelink, HARQ feedback can be enabled or disabled. If HARQ feedback is disabled, there is no need to maintain the sidelink drx-HARQ-RTT-Timer. If HARQ feedback is enabled, based on Proposal 1, it is clearly that for each sidelink direction, Tx UE should maintain the drx-HARQ-RTT-Timer for sidelink transmission, and Rx UE should maintain the drx-HARQ-RTT-Timer for sidelink reception. Similar as Uu, the maintenance can be as follows:

- For drx-HARQ-RTT-Timer in Rx UE side, similar as the drx-HARQ-RTT-TimerDL, it can be started for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the sidelink HARQ feedback.

- For drx-HARQ-RTT-Timer in Tx UE side, it can be started for the corresponding HARQ process in the first symbol after the end of the corresponding reception of sidelink HARQ feedback.

Proposal 6: If sidelink HARQ feeback is disabled, drx-HARQ-RTT-Timer should not be started.

Proposal 7: If sidelink HARQ feedback is enabled, similar as Uu, the drx-HARQ-RTT-Timer in Rx UE side should be started for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the sidelink HARQ feedback.

Proposal 8: If sidelink HARQ feedback is enabled, similar as Uu, the drx-HARQ-RTT-Timer in Tx UE side should be started for the corresponding HARQ process in the first symbol after the end of the corresponding reception of sidelink HARQ feedback.

drx-RetransmissionTimer

Regarding to the drx-RetransmissionTimerDL and drx-RetransmissionTimerUL, in Uu, it represents the maximum duration until a DL retransmission or a grant for UL retransmission is received. It is started in the first symbol after the end of the PDCCH reception. The length unit of this timer is slot.

For NR sidelink, due to the same reason for introducing the sidelink drx-HARQ-RTT-Timer, the sidelink drx-RetransmissionTimer should also be introduced. It can ensure when resource pre-emption happens, UE can still receive the SCI for retransmission.

Proposal 9: For sidelink unicast, drx-RetransmissionTimer should be introduced.

If sidelink HARQ feedback is disabled, as proposed in Proposed 7, it does not need to maintain drx-HARQ-RTT-Timer. But in this case, whether sidelink drx-RetransmissionTimer should be maintained needs further discussion. According to the current spec, even if HARQ feedback is disabled, the UE is still possible to provide HARQ feedback on Uu PUCCH if it wants to request resource for retransmission. In this case, in order to ensure the retransmission can be performed, the sidelink drx-RetransmissionTimer had better be maintained even if there is no sidelink drx-HARQ-RTT-Timer. When to start the sidelink drx-RetransmissionTimer can be further discussed.

In our understanding, two options can be considered:

- Option 1: The sidelink drx-RetransmissionTimer in Rx/Tx UE side can be started after SCI is received/transmitted.

- Option 2: The sidelink drx-RetransmissionTimer in Rx/Tx UE side can be started after SCI is received/transmitted plus a minimum gap. E.g., the minimum gap length can be determined based on the minimum time gap between any two selected resources as defined in section 5.22.1.1 of [3].

Proposal 10: Even if the sidelink HARQ feeback is disabled, the drx-RetransmissionTimer still needs to be maintained at least in the case that UE has PUCCH in Uu. In this case, when to start the drx-RetransmissionTimer can be further studied.

If sidelink HARQ feedback is enabled, based on Proposal 8, drx-HARQ-RTT-Timer should be maintained. Hence it is also reasonable to maintain the sidelink drx-RetransmissionTimer. Similar as Uu, in this case, when the sidelink drx-HARQ-RTT-Timer expiry, the sidelink drx-RetransmissionTimer should be started.

Proposal 11: If sidelink HARQ feedback is enabled, once the drx-HARQ-RTT-Timer expiry, the corresponding drx-RetransmissionTimer should be started.

drx-ShortCycleTimer

For drx-ShortCycleTimer, whether it is necessary depends on whether short DRX cycle should be supported. Although it may introduce complexity on the DRX maintenance, it can provide flexibility. It is especially useful for time-sensitive services.

Proposal 12: For sidelink unicast, short DRX cycle should be supported.

Proposal 13: The maintenance of drx-ShortCycleTimer is same as Uu.

Proposal 14: Sidelink DRX MAC CE can be used to control the switching between long and short DRX cycle.

DRX timer maintainence for sidelink broadcast/groupcast

For sidelink broadcast/groupcast, which DRX timer is needed and how to maintain it are discussed as below:

drx-onDurationTimer

For sidelink broadcast/groupcast, how to implement the sidelink DRX is still FFS, there are two main options:

- Option 1: use dedicated resource pool.

- Option 2: use the DRX timers.

For Option 1, the resource pool should be dedicated for different cast type. Its main disadvantage is that it will impact the sidelink resource efficiency and it will impact both RAN1 and RAN2. Hence, Option 2 is slightly preferred. If Option 2 is adopted, it is no doubt that drx-onDurationTimer should be supported for sidelink broadcast and it maintenance should be same as Uu.

drx-InactivityTimer

In Uu, the essence of the drx-InactivityTimer is to extend the active time.

For sidelink broadcast, it is best-effort. Some UEs cannot always receive the SCI. Hence, in order to align the understanding on active time between Tx UE and Rx UE, it had better not introduce the sidelink drx-InactivityTimer.

For sidelink groupcast, due to the same reason as sidelink broadcast, it had better not introduce the sidelink drx-InactivityTimer.

HARQ-RTT-Timer/ drx-RetransmissionTimer

For sidelink broadcast, there is no HARQ feedback, there is no need to maintain the HARQ related timer.

For sidelink groupcast, HARQ feedback can be supported. But considering sidelink groupcast is 1:M transmission, some Rx UE may receive the TB correctly; while others are not. If HARQ related timer is maintained, it should further consider how to keep Tx and Rx UE has the same understanding on the sidelink active time. Hence, it had better not introduce these timers for sidelink groupcast.

drx-ShortCycleTimer

For sidelink broadcast/groupcast, as suggested in[2], there is no company interested in introducing short DRX cycle. Hence, drx-ShortCycleTimer is not needed.

Proposal 15: For sidelink broadcast/groupcast, only drx-ondurationtimer should be introduced and its maintenance should be same as Uu.

Definition of active time

In NR Uu, the definition of active time is as below:

When a DRX cycle is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL or drx-RetransmissionTimerUL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

For sidelink unicast, the active time should be defined per sidelink unicast direction. For each sidelink unicast direction, the active time should include the time period when any of the drx-onDurationTimer or the drx-InactivityTimer or thedrx-RetransmissionTimer is running.

For sidelink broadcast/groupcast, the active time should only include the time where the drx-onDurationTimer is running.

Proposal 16: For sidelink unicast, the active time should be defined per sidelink unicast direction.

Proposal 17: For sidelink unicast, for each sidelink direction, when a DRX cycle is configured, the Active Time includes the time while the drx-onDurationTimer, or the drx-InactivityTimer, or the drx-RetransmissionTimer is running.

Proposal 18: For sidelink broadcast/groupcast, the sidelink DRX Active Time should only include the time period when the drx-onDurationTimer is running.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: One set of sidelink DRX timers should be maintained for each direction of the sidelink unicast connection.

Proposal 2: For sidelink unicast, drx-ondurationtimer should be introduced and its maintenance should be same as Uu.

Proposal 3: For sidelink unicast, drx-InactivityTimer should be introduced.

Proposal 4: once SCI is received, the drx-InactivityTimer in Rx UE side should be started; and once SCI is transmitted, the drx-InactivityTimer in Tx UE side should be started.

Proposal 5: For sidelink unicast, drx-HARQ-RTT-Timer should be introduced.

Proposal 6: If sidelink HARQ feeback is disabled, drx-HARQ-RTT-Timer should not be started.

Proposal 7: If sidelink HARQ feedback is enabled, similar as Uu, the drx-HARQ-RTT-Timer in Rx UE side should be started for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the sidelink HARQ feedback.

Proposal 8: If sidelink HARQ feedback is enabled, similar as Uu, the drx-HARQ-RTT-Timer in Tx UE side should be started for the corresponding HARQ process in the first symbol after the end of the corresponding reception of sidelink HARQ feedback.

Proposal 9: For sidelink unicast, drx-RetransmissionTimer should be introduced.

Proposal 10: Even if the sidelink HARQ feeback is disabled, the drx-RetransmissionTimer still needs to be maintained at least in the case that UE has PUCCH in Uu. In this case, when to start the drx-RetransmissionTimer can be further studied.

Proposal 11: If sidelink HARQ feedback is enabled, once the drx-HARQ-RTT-Timer expiry, the corresponding drx-RetransmissionTimer should be started.

Proposal 12: For sidelink unicast, short DRX cycle should be supported.

Proposal 13: The maintenance of drx-ShortCycleTimer is same as Uu.

Proposal 14: Sidelink DRX MAC CE can be used to control the switching between long and short DRX cycle.

Proposal 15: For sidelink broadcast/groupcast, only drx-ondurationtimer should be introduced and its maintenance should be same as Uu.

Proposal 16: For sidelink unicast, the active time should be defined per sidelink unicast direction.

Proposal 17: For sidelink unicast, for each sidelink direction, when a DRX cycle is configured, the Active Time includes the time while the drx-onDurationTimer, or the drx-InactivityTimer, or the drx-RetransmissionTimer is running.

Proposal 18: For sidelink broadcast/groupcast, the sidelink DRX Active Time should only include the time period when the drx-onDurationTimer is running.

Reference

- R2-2100235  General principles for sidelink DRX   CATT

- [POST112-e][702][SLe] High-level principles for SL DRX

- TS38.321  Medium Access Control (MAC) protocol specification (Release 16)

- R2-2100237 Sidelink DRX Configuration Procedure for Sidelink Unicast  CATT