3GPP TSG-RAN WG2 Meeting #113-e                                                     R2-2100278

Online, Jan 25 – Feb 5, 2021

Source:	CATT

Title:	Discussion on UE Onboarding and Provisioning for NPN

Agenda Item:	8.16.3

Document for:	Discussion and Decision

Introduction

At RAN#90e meeting, a revised WID [1] was approved for R17 eNPN, and one of the objectives in the WID was “Support UE onboarding and provisioning for NPN”, the details of the objective are as the following:

- Support UE onboarding and provisioning for NPN including: - The UE onboarding relevant parameter broadcast from SIB [RAN2] - The associated cell selection/reselection, cell access control and the connected mode mobility support [RAN2/RAN3] - The necessary modifications over network interfaces (e.g. NG, Xn, F1, E1 etc) [RAN3]

In this contribution, we discuss key aspects to support UE onboarding and provisioning for NPN.

Discussion

In SA2, the corresponding issue for this objective is “Key Issue #4: UE Onboarding and remote provisioning”, some conclusions on key issue #4 has been made by SA2, which can be found in [2], clause 8.4.

In this section, we look into these conclusions to address the potential NG-RAN impacts.

One of the conclusions which has apparent NG-RAN impact is as following,

- The NG-RAN of the Onboarding network includes an indication for Onboarding enabled in the SIB (per O-SNPN, considering that the NG-RAN can be shared) so that the UE can discover and select an appropriate O-SNPN. The UE may or may not be pre-configured with O-SNPN network selection information (e.g. O-SNPN network identifiers).

To facilitate UE to discover and select the onboarding SNPN, NG-RAN needs to indicate whether onboarding is enabled for a specific O-SNPN. This enhancement can be simply realized by adding the related indication in the R16 IE “npn-IdentityInfoList” in SIB1.

Proposal 1: To support O-SNPN, SIB1 can be enhanced to indicate whether onboarding is enabled per SNPN.

Using PLMN as Onboarding Network is also supported according to SA conclusion in [2], clause 8.4 as following,

- Using PLMN credentials for UE onboarding and PLMN as Onboarding Network (ON) is already possible.

And onboarding is performed via the UP connectivity of PLMN.

Usage of a PLMN as Onboarding Network for a UE equipped with a USIM shall be possible. The SO-SNPN credentials can be transmitted to UE via UP connectivity.

It seems that SA2 is assuming any PLMN is able to provide UP connectivity to the provisioning server for credentials provisioning. With this, PLMN as Onboarding Network (ON) is transparent to NG-RAN. Therefore, from NG-RAN point of view, we do not need to indicate to UE whether onboarding is enabled by a specific PLMN, i.e. not like what is needed for onboarding SNPN.

Proposal 2: No enhancement in NG-RAN is needed to support PLMN as onboarding network.

There is another conclusion in SA2 which explicitly requires enhancement in NG-RAN. The conclusion is as following,

- Upon registration to an SNPN for Onboarding, the UE provides an indication at RRC level that the RRC connection is for onboarding. This information will be specified only for SNPN and allows NG-RAN to select an appropriate AMF that supports onboarding procedures.

According to the above description, UE need to indicate that the RRC connection is for onboarding, subsequently this information will be used by NG-RAN to select AMF which supports onboarding procedures.

However, there is already an IE “selectedPLMN-Identity” which is mandatory in MSG5(RRCSetupComplete).

selectedPLMN-Identity Index of the PLMN or SNPN selected by the UE from the plmn-IdentityList or npn-IdentityInfoList fields included in SIB1.

As above, if the selected network is SNPN, then index of the SNPN selected by the UE will be included in the MSG5.This is per Rel16 NPN agreement.

Consequently, if UE selected a SNPN (i.e. O-SNPN) for onboarding, when UE requests RRC connection for onboarding on an O-SNPN, the IE “selectedPLMN-Identity” in MSG5 will be set to the SNPN ID of the selected SNPN (i.e. SNPN support onboarding).

After receiving “selectedPLMN-Identity” in MSG5, the SNPN ID of O-SNPN in “selectedPLMN-Identity” can be used by NG-RAN node for AMF selection. Eventually the AMF supports onboarding will be selected by NG-RAN.

In Figure 1, AMF selection by using the SNPN ID of O-SNPN in MSG5 is illustrated,

Figure 1

Therefore an explicit indication at RRC level that the RRC connection is for onboarding is not necessary.

Proposal 3: An explicit indication at RRC level that the RRC connection is for onboarding is not necessary.

Based on SA2 conclusion，one SNPN can be used only for on-boarding service, or be used for both on-boarding service and normal service.

If the SNPN is only used for on-boarding service, only the UE for onboarding is allowed to camp on it. UE requires normal services should not be allowed to camp on it.

SA2 has concluded that Onboarding network should support functionality to restrict usage to only on-boarding service.

Onboarding network should support functionality to restrict usage to only on-boarding service

The mentioned functionality is located in AMF and be used when UE perform the registration after camp on the cell.

Even through it is not mentioned there is any NG-RAN impacts, we see benefit of a RAN level solution to prevent normal UE from camping on SNPN cell only for Onboarding. To facilitate this, RAN node may need to indicate whether a SNPN is only used for onboarding service or for both onboarding service and normal services.

Proposal 4: Discuss whether NG-RAN needs to indicate whether the SNPN only supports onboarding service, or supports both onboarding service and normal services.

SA2 has concluded that onboarding function could be used to PNI-NPN credentials provisioning and no enhancement to PLMN network selection is needed.

- No enhancement for the UE onboarding (component 1 of KI#4) with PLMN credentials used for primary authentication and PLMN network selection are needed for the case of PNI-NPN credentials provisioning.

Therefore we do not need to consider any enhancement in NG-RAN to support onboarding for PNI-NPN as well.

Proposal 5: No enhancement in NG-RAN is needed to support onboarding for PNI-NPN credentials provisioning.

After UE has camped on an O-SNPN, mobility procedure can be triggered before the remote provisioning is completed. RAN2 should discuss how to ensure UE staying on O-SNPN cells before the completion of the remote provisioning.

Proposal 6: Discuss how to support the Mobility (including cell selection and handover) between O-SNPNs.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: To support O-SNPN, SIB1 can be enhanced to indicate whether onboarding is enabled per SNPN.

Proposal 2: No enhancement in NG-RAN is needed to support PLMN as onboarding network.

Proposal 3: An explicit indication at RRC level that the RRC connection is for onboarding is not necessary.

Proposal 4: Discuss whether NG-RAN needs to indicate whether the SNPN only supports onboarding service, or supports both onboarding service and normal services.

Proposal 5: No enhancement in NG-RAN is needed to support onboarding for PNI-NPN credentials provisioning.

Proposal 6: Discuss how to support the Mobility (including cell selection and handover) between O-SNPNs.

Reference

[1] RP-202363, revision of ePRN WID

[2] TR 23.700-07, Study on enhanced support of non-public networks,v1.2.0