3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2100296

Online, Jan 25 – Feb 5, 2021

Source:	CATT

Title:	Considerations on transition into RRC_CONNECTED during subsequent SDT

Agenda Item:	8.6.4

Document for:	Discussion and Decision

Introduction

We agreed to introduce subsequent SDT without transitioning to RRC_CONNECTED. It is a common understanding that the network can transit the UE into RRC_CONNECTED at any time during subsequent SDT. In this contribution, we analyze the procedure and possible issues if the network wants to transit the UE into RRC_CONNECTED with or without anchor relocation.

Discussion

We agreed context fetch and data forwarding with anchor re-location and without anchor re-location will be considered for SDT with RRC message via RACH based schemes. During subsequent SDT, the network may decide to transit the UE to RRC_CONNECTED. In this case, the gNB needs to send RRCResume message to the UE to transit to RRC_CONNECTED. We will discuss the possible procedure and considerations when the network transits the UE into RRC_CONNECTED with or without anchor relocation.

- With anchor relocation

With anchor relocation, the current serving gNB obtains the UE context and handles small data with updated security key. During subsequent SDT, the current serving gNB can send RRCResume message to the UE directly if the gNB decides to transit UE into RRC_CONNECTED. When the UE receives RRCResume message, it enters RRC_CONNECTED state and sends RRCResumeComplete message to the serving gNB. The legacy procedure can be used as RRCResumeRequest message and RRCResumeComplete message are received by the current serving gNB and RRCResume message as the response message is sent by the same gNB.

Figure 1 Transition to RRC_CONNECTED during subsequent SDT with anchor relocation

Observation 1: With anchor relocation, legacy RRC resume procedure can be reused if the gNB decides to transit UE into RRC_CONNECTED during subsequent SDT.

- Without anchor relocation

Without anchor relocation, the current serving gNB cann’t obtain the whole UE context. Small data is treated by anchor gNB with updated security key.

During subsequent SDT, the network may decide to transit the UE into RRC_CONNECTED. The following issues need to be discussed:

- Which node can decide to transit the UE into RRC_CONNECTED?

Option 1: The current serving gNB decides to transit the UE into RRC_CONNECTED.

Option 2: The anchor gNB decides to transit the UE into RRC_CONNECTED.

As the current serving gNB can be aware of the size of UL data to be transmitted and performance of data transmission via radio interface, it is reasonable for the current serving gNB to decide to transit the UE into RRC_CONNECTED.

On the other hand, the anchor gNB can be aware of DL data arrival. Hence, it is reasonable for the anchor gNB to trigger  the UE transition to RRC_CONNECTED.

As the issue may impact signalling exchange via Xn interface, RAN3 can discuss and decide it further.

Observation 2: From RAN2 point of view, both serving gNB and anchor gNB can trigger the UE transition to RRC_CONNECTED. Due to Xn signaling involvement, RAN3 should make the final decision.

- Which node decides the content of RRCResume message?

Option 1: The current serving gNB decides the content of RRCResume message.

Option 2: The anchor gNB decides the content of RRCResume message.

As the content of RRCResume message is related to configurations of radio interface, it is more reasonable for the current serving gNB to decide the content of RRCResume message. Option 1 is preferred.

Observation 3: The content of RRCResume message should be decided by the serving gNB.

- Which node performs ciphering and integrity protection for RRCResume message?

Option 1: The current serving gNB.

Option 2: The anchor gNB.

When the network decides to transit the UE into RRC_CONNECTED, the UE cann’t be aware this. The UE uses updated security key (marked as KgNB1), which is updated upon initiation of RRCResumeRequest message, to receive DL data and DL signalling (including RRCResume message). Hence, ciphering and integrity protection for RRCResume message need to be performed based on KgNB1. As KgNB1 is used by the anchor gNB for UL/DL data during SDT without anchor relocation, the anchor gNB should perform ciphering and integrity protection for RRCResume message. Option 2 should be adopted.

Observation 4: The anchor gNB should perform ciphering and integrity protection for RRCResume message.

- Which node is RRCResumeComplete message sent to?

Option 1: The current serving gNB.

Option 2: The anchor gNB.

Upon receiving RRCResumeRequest message, the UE enters RRC_CONNECTED mode and sends RRCResumeComplete message for response. When the network decides to transit the UE into RRC_CONNECTED mode, there is no benefit to keep the UE context and handle UL/DL data in anchor gNB. Thus, anchor relocation will be performed. And it is straightforward for the UE to send RRCResumeComplete message to the current serving gNB, which is similar as handover procedure. Option 1 is preferred.

Observation 5: The UE sends RRCResumeComplete message to the current serving gNB, which is similar as handover procedure.

- Security issue

As analyzed above, updated security key (KgNB1) will be applied to ciphering and integrity protection for RRCResume message.

And similar as handover procedure, we propose that the UE sends RRCResumeComplete message to the current serving gNB. Then subsequent UL/DL data is handled in the current serving gNB. Different security keys should be used in different network nodes. But according to current TS, security key cann’t be updated via RRCResume message. Thus, we need to ask SA3 how to ensure security in this case.

The following is the general picture of transition into RRC_CONNECTED during subsequent SDT without anchor relocation.

Figure 2 Transition into RRC_CONNECTED during subsequent SDT without anchor relocation

Observation 6: Without anchor relocation, the issue that how to ensure security separation between two different gNBs after transaction to RRC_CONNECTED needs to be resolved.

If we agreed that subsequent SDT is always performed with anchor relocation, above issues don’t need to be considered. Hence, we propose:

Proposal 1: RAN2 to discuss whether subsequent SDT without anchor relocation is supported.

If we agree to support subsequent SDT without anchor relocation, we further discuss how to support the transaction into RRC_CONNECTED during subsequent SDT.

Proposal 2: If subsequent SDT without anchor relocation is supported, we propose:

- Ask RAN3 to decide which node can decide to transit the UE into RRC_CONNECTED .

- The current serving gNB decides the content of RRCResume message.

- The anchor gNB performs ciphering and integrity protection for RRCResume message.

- The UE sends RRCResumeComplete message to the current serving gNB.

- Ask SA3 how to ensure security key separation if UL/DL data and RRCResume message are treated by anchor gNB with updated security key then RRCResumeComplete message and subsequent UL/DL are treated by the current serving gNB.

Conclusion

In this contribution, we analyze the procedure and possible issues if the network wants to transit the UE into RRC_CONNECTED with or without anchor relocation.

For SDT with anchor relocation, we observe:

Observation 1: With anchor relocation, legacy RRC resume procedure can be reused if the gNB decides to transit UE into RRC_CONNECTED during subsequent SDT.

For SDT without anchor relocation, we observe:

Observation 2: From RAN2 point of view, both serving gNB and anchor gNB can trigger the UE transition to RRC_CONNECTED. Due to Xn signaling involvement, RAN3 should make the final decision.

Observation 3: The content of RRCResume message should be decided by the serving gNB.

Observation 4: The anchor gNB should perform ciphering and integrity protection for RRCResume message.

Observation 5: The UE sends RRCResumeComplete message to the current serving gNB, which is similar as handover procedure.

Observation 6: Without anchor relocation, the issue that how to ensure security separation between two different gNBs after transaction to RRC_CONNECTED needs to be resolved.

If we agreed that subsequent SDT is always performed with anchor relocation, then we don’t need to discuss observation 2-6. Hence, we propose:

Proposal 1: RAN2 to discuss whether subsequent SDT without anchor relocation is supported.

Proposal 2: If proposal 1 is agree, we propose in the case of subsequent SDT without anchor relocation:

- Ask RAN3 to decide which node can decide to transit the UE into RRC_CONNECTED .

- The current serving gNB decides the content of RRCResume message.

- The anchor gNB performs ciphering and integrity protection for RRCResume message.

- The UE sends RRCResumeComplete message to the current serving gNB.

- Ask SA3 how to ensure security key separation if UL/DL data and RRCResume message are treated by anchor gNB with updated security key then RRCResumeComplete message and subsequent UL/DL are treated by the current serving gNB.

Reference

- TS 38.300  v16.4.0