3GPP TSG-RAN WG2 Meeting #113-e	R2-2100312

Online, January 25 – February 5, 2021

Agenda item:	8.12.3

Source:	Qualcomm Incorporated

Title:	Power saving enhancements for RedCap UEs

WID/SID:	FS_NR_redcap

Document for:	Discussion and Decision

# Introduction

This paper discusses three topics related to power saving enhancements for RedCap UEs: eDRX, RRM relaxation for stationary RedCap UEs and C-DRX enhancements for video applications.

# Discussion

## eDRX enhancements

In RAN2#112-e, the following agreements were made:

eDRX cycle extension in RRC_IDLE beyond 10.24s for REDCAP UEs will be studied in this SI/WI. For UE in RRC IDLE and eDRX cycle is equal to 10.24s, among the solution options, we start from the assumption that paging monitoring does not use PTW and PH.

In LTE, if UE is configured with eDRX cycle of 5.12 sec, UE then monitors paging every 5.12 sec, i.e. its eDRX cycle overrides the default paging cycle and RAN paging cycle. We think the same behavior can be extended to RedCap UEs that do not need or do not support eDRX cycles longer than 10.24 sec (e.g. wearables). This simple overriding rule can help keep their eDRX operation simple and minimize their implementation complexity. Moreover, because NR uses a different encoding for eDRX cycles than LTE, 10.24 sec does not require the use of hyper-SFN. And at 10.24 sec, whether to have PTW does not make a big difference in UE’s power consumption. Hence not supporting PTW is a sensible tradeoff with lower complexity, which is an important aspect for RedCap.

Proposal 1. 	RAN2 confirm that if UE is configured with eDRX cycle of 10.24 sec, UE monitors its paging occasions every 10.24 sec and does not use PTW and PH.

## RRM relaxation for stationary UEs

In RAN2#112-e, the following agreements were made

The target REDCAP UE, considering mobility, is not limited to a fixed UE, but can also experience some low mobility, and this, during some “stationary” periods of time. The RRM relaxation of REDCAP UEs is triggered based on measurements, as a baseline. Other triggering conditions for the “level-1” (still device at fixed location) UEs are not excluded, e.g. the possibility to signal their stationary property explicitly.

In our view, it is beneficial for both network and UEs if UEs with fixed location (referred to as “stationary UEs” thereafter) can explicitly signal their stationarity. For example,

- Network can use that information to optimize its paging for stationary UEs, especially if those UEs can provide a list of preferred neighbour cells to network;

- Network can also use that information to configure more power efficient RRM for stationary UEs when they are in RRC Connected.

And in typical deployment scenarios, stationary UEs do not change their locations often. Hence this signaling would not create much overhead.

Proposal 2. 	UE with fixed location explicitly signals network that it is stationary.

We think true stationary UEs should be configured with different triggering conditions as well as different relaxation criteria for RRM relaxation from those that are temporarily stationary. That is because true stationary UEs do not need search period to validate their stationarity. Once their RSRP measurements are complete, they can decide which type of RRM relaxation they can apply (e.g. stop all neighbor cell measurements or apply a scaling factor).

Proposal 3.  	UEs with fixed location can have different triggering conditions and relaxation criteria from other UEs.

Therefore, network can advertise at least the following set of configurations for RRM relaxation in a cell:

- R17 RRM relaxation for UEs with fixed locations;

- R16 RRM relaxation for low mobility for non-RedCap UEs;

- R16 RRM relaxation for not-at-cell-edge for any UEs (i.e. both RedCap and non-RedCap).

It can be studied further whether R16 low mobility criterion can be by for temporarily stationary RedCap UEs too. R16 RRM relaxation for low mobility does include a search period to detect if UE has been stationary. It is not clear if RedCap UEs would need a different (e.g. shorter) search period or different relaxation behavior when they are temporarily stationary.

Proposal 4.	Network can advertise any combination of the following set of configurations for RRM relaxation in a cell:

- R17 RRM relaxation for UEs with fixed locations;

- R16 RRM relaxation for low mobility for any UEs;

- R16 RRM relaxation for not-at-cell-edge for any UEs.

FFS whether temporarily stationary RedCap UEs reuse the R16 RRM relaxation criteria for low mobility or have their own criteria.

If a RedCap UE does not meet the R17 criteria, it should be allowed to check if it may apply R16 low mobility and/or not-at-cell-edge criteria, as R16 criteria were designed for generic scenarios and UEs.

Proposal 5.  	If a RedCap UE does not meet the R17 relaxation criterion for “stationarity”, it can still check if R16 low mobility and/or not-at-cell-edge criteria apply.

## C-DRX enhancements for video applications

In the RedCap SID [1], video surveillance is listed as one of the motivating use cases. Besides that, we see additional video-related use cases that take advantage of what RedCap can offer. For example:

- Some high-end wearables start to offer video calls. We expect this trend will grow and be offered on more devices;

- Drones or vloggers stream real-time videos of live actions or events. A similar type of application is body-cam worn by police, security or military personale which also stream real-time videos when needed.

One commonality among the above applications is that they are all battery powered and hence power sensitive. And RedCap UEs are a good fit (e.g. in term of cost-to-performance ratio) for those applications.

Observation 1. There are growing number of real-time video applications running on battery-powered devices, in market segments targeted by RedCap UEs.

However, the current DRX procedure and configuration have some shortcoming in supporting video applications. We discuss two such issues in the following.

### DRX cycle for video traffic

Video traffic is known to be period, because video frames are generated continuously with a fixed period. Typical frame rates are 30 or 60 frames per second, which correspond to inter-frame interval of 33.33 or 16.66 msec, respectively.

To save power, network needs to configure UE’s DRX cycle to match the inter-frame intervals of video traffic. However, as one may see, none of the above inter-frame intervals are supported in the current configurable values of DRX cycles. If the nearest DRX cycle supported by the current spec is used (e.g. 32ms and 20ms), UE may either wakeup early or later than a new frame starts. That’s not power efficient.

Observation 2. 	There is a significant mismatch between inter-frame interval in video traffic and available values of DRX cycle.

Proposal 6. 	Introduce new DRX cycles to better support inter-frame intervals in video traffic.

If we introduce new values of DRX cycle to the spec by rounding those inter-frame intervals to the nearest symbol, the rounding error can accumulate over time and eventually result in a large gap between the start of a DRX cycle and a video frame. For example, it takes about only 20 frames for the error to add up to one slot. Therefore, the starting offset of a DRX cycle needs to be adjusted overtime to compensate the rounding errors.

Observation 3. 	The discrepancy between starting times of DRX cycle and video frame can increase over time, even if DRX cycle is closely matched to inter-frame interval of video traffic.

Proposal 7. 	Study methods that enable periodic adjustment to UE’s DRX start offset.

## Early termination of DRX active time by UE

When transmitting/receiving streaming video traffic, UE needs to be able to sleep between video frames to save power. Otherwise, UE has to stay DRX active all the time. Given the relatively short inter-frame intervals of video traffic, being able to terminate DRX active time quickly and efficiently between two video frames thus is the key to save power.

In current DRX procedure, there are two ways to terminate DRX active time: either network sends UE a DRX MAC CE or by DRX inactivity timer. However, neither is efficient for video traffic.

First, to send DRX MAC CE, network needs to know when UE has finished a video frame. As one may know, because of compression and adaptive encoding is widely used in video applications, length of each video frame varies in time and is hard to predict by looking at packet-level statistics. Therefore, there is no easy way for gNB to know, with a high confidence, when a video frame ends so that it can send a DRX MAC CE.

Observation 4. 	It is difficult for gNB to know when a video frame ends, for it to send a DRX MAC CE.

How to configure the right length of DRX inactivity timer is also challenging. First, because of the relatively short inter-frame interval of video traffic, DRX inactivity timer has to be short. However, it cannot be shorter than typical scheduling gaps. On a FR1 carrier, this gap can be around 10ms. If inter-frame interval is 33.33 msec (i.e. video is 30 frame per sec), since a video frame on avg takes ~15 msec to finish (actual number varies depend on link throughput and video resolution), a 10 msec DRX inactivity timer does not leave much time for UE to sleep (i.e. sleep time = 33.33-15-10=8.33 msec), and UE can have only shallow sleep between frames.

Observation 5. 	Even with a short DRX inactivity timer, UE would not be able to have much sleep between two video frames.

However, UE can have assistance from application-layer on when a video frame ends. If UE can signal network when a frame ends, its DRX active time can be terminated promptly to save more power. In the example above, if the 10ms DRX inactivity timer is removed and the signaling is done by a MAC CE which requires 3ms processing time, then the amount of time that UE can sleep is 33.33-15-3x2 (see Option 1 below) = 15.33 msec. That is 48% increase in power savings.

Observation 6. 	Significant power savings can be achieved if UE can assist in terminating DRX active time at end of a video frame.

We think an efficient way for this signal can be a UL MAC CE from UE to gNB, to indicate its preference to terminate DRX active time. There can be different ways to implement the idea. For example:

- Option 1. UE waits for a legacy DRX MAC CE from network to terminate DRX active time. This option is simple and no further change to the legacy DRX procedure is required. But it takes more time (2x MAC CE processing time) to enter DRX off time;

- Option 2. UE starts a timer after sending its MAC CE. If UE does not receive any DL assignment for new data when this timer expires, it autonomously terminates DRX active time. Otherwise, it restarts the DRX inactivity timer. This approach potentially can be faster than Option 1 but requires new behaviors.

Proposal 8. 	Study methods for UE to efficiently indicate its preference to terminate DRX active time.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1. 	RAN2 confirm that if UE is configured with eDRX cycle of 10.24 sec, UE monitors its paging occasions every 10.24 sec and does not use PTW and PH.

Proposal 2. 	UE with fixed location explicitly signals network that it is stationary.

Proposal 3.  	UEs with fixed location can have different triggering conditions and relaxation criteria from other UEs.

Proposal 4.	Network can advertise any combination of the following set of configurations for RRM relaxation in a cell:

- R17 RRM relaxation for UEs with fixed locations;

- R16 RRM relaxation for low mobility for any UEs;

- R16 RRM relaxation for not-at-cell-edge for any UEs.

FFS whether temporarily stationary RedCap UEs reuse the R16 RRM relaxation criteria for low mobility or have their own criteria.

Proposal 5.  	If a RedCap UE does not meet the R17 relaxation criterion for “stationarity”, it can still check if R16 low mobility and/or not-at-cell-edge criteria apply.

Observation 1. 	There are growing number of real-time video applications running on battery-powered devices, in market segments targeted by RedCap UEs.

Observation 2. 	There is a significant mismatch between inter-frame interval in video traffic and available values of DRX cycle.

Proposal 6. 	Introduce new DRX cycles to better support inter-frame intervals in video traffic.

Observation 3. 	The discrepancy between starting times of DRX cycle and video frame can increase over time, even if DRX cycle is closely matched to inter-frame interval of video traffic.

Proposal 7. 	Study methods that enable periodic adjustment to UE’s DRX start offset.

Observation 4. 	It is difficult for gNB to know when a video frame ends, for it to send a DRX MAC CE.

Observation 5. 	Even with a short DRX inactivity timer, UE would not be able to have much sleep between two video frames.

Observation 6. 	Significant power savings can be achieved if UE can assist in terminating DRX active time at end of a video frame.

Proposal 8. 	Study methods for UE to efficiently indicate its preference to terminate DRX active time.

# References

- RP-201386, Revised SID on Study on support of reduced capability NR devices, June 2020.