3GPP TSG-RAN WG2 #113-e	Tdoc R2-2100340

Electronic meeting, Jan 25th – Feb 5th, 2021

Agenda Item:	6.1.3

Source:	Ericsson

Title:	UL PUSCH skipping without intra-UE prioritization

Document for:	Discussion, Decision

# Introduction

This paper discusses the two LSes on UL PUSCH skipping [1][2]. UL skipping for dynamic grant is described in [1] and UL skipping for configured grant is described in [2]. In the LS [2], it is clarified that

The discussions on PUSCH skipping with UCI in RAN1, including the agreements made for DG PUSCH in RAN1#102-e, were based on the assumptions where LCH based prioritization is not configured and there is a single PHY priority for UL transmission.

- In this paper, we focus on the case when intra-UE prioritization is not configured, i.e., LCH-based prioritization is not configured and there is a single PHY priority for UL transmission. The other case with intra-UE prioritization configured is discussed in the paper [3].

# Discussion

In both cases of dynamic grant (DG) and configured grant (CG), the essence of the LS[1][2] is that MAC shall not skip building MAC PDUs for a PUSCH with UCI to be multiplexed on it. Which such PUSCH cannot be skipped is an information provided by PHY layer. In RAN2#112e, the case for dynamic grant is discussed with the below agreements:

[016] RAN2 confirms that a new UE capability is introduced for Rel-16 dynamic UL skipping. [016] A new RRC parameter is introduced to enable Rel-16 dynamic UL skipping.

The agreements related with configured grant are indicated in [2]. In our view, this feature is more related with the UL skipping itself and there is no need for a differentiation between configured grant and dynamic grants. The network needs both features to work to alleviate the blind decoding complexity associated with grant skipping that has UCI-multiplexed. From the UE point of view, we don’t see any implementation challenges to support both dynamic grant and configured grant simultaneously. Therefore, we propose to have one set of UE capability and RRC parameter for Rel-16 UL skipping feature, to avoid the feature fragmentation and the complexity that it brings.

Proposal 1	Introduce only one UE capability along with only one RRC parameter to enable Rel-16 UL skipping feature applied for both configured grant and dynamic grant.

An example TP can be found in the Annex 6. The RRC configuration parameter is skipUplinkTx-r16 and the network does not configure skipUplinkTx-r16 and skipUplinkTxDynamic simultaneously. This proposal on configured grants can cover the below two cases in the LS [2]:

| Case 1-2 | Case 1-5 |
| --- | --- |

In the LS [2], further cases due to overlapping between dynamic grants and configured grants are discussed. In the MAC spec, the grant selection between dynamic grant and configured grant is performed at subclause 5.4.1 (i.e. UL grant reception) and before the “multiplexing and assembly” entity (specified in subclause 5.4.3). More precisely, if a configured grant overlaps with a dynamic grant, this configured grant is skipped regardless of the followed-up conditions in “multiplexing and assembly” entity.

Observation 1	In the current MAC spec, a configured grant is always skipped if it overlaps with a dynamic grant in the same serving cell.

In the LS [2], there are three cases in which the DG PUSCH and CG PUSCH overlap.

| Case 1-3 | Case 1-4 | Case 1-6 |
| --- | --- | --- |

In all three cases, RAN1 agreement/working assumptions indicate that the CG PUSCH are dropped and there is no mandate to generate the MAC PDU on this CG PUSCH, including both options for Case 1-6, see details in the Annex 5. Thus, PHY procedure is in align with the existing MAC spec procedure and we don’t see a need to update the spec.  Therefore, we propose to confirm this understanding.

Proposal 2	When LCH-based prioritization is not configured and there is a single PHY priority for UL transmission, RAN2 confirms that a dynamic grant always overrides a configured grant (i.e., configured grant is skipped) if they overlap in the same cell, regardless of whether Rel-16 UL skipping is configured or not.

# Conclusion

In the previous section, we made the following proposals:

Proposal 1	Introduce only one UE capability along with only one RRC parameter to enable Rel-16 UL skipping feature applied for both configured grant and dynamic grant.

Proposal 2	When LCH-based prioritization is not configured and there is a single PHY priority for UL transmission, RAN2 confirms that a dynamic grant always overrides a configured grant (i.e., configured grant is skipped) if they overlap in the same cell, regardless of whether Rel-16 UL skipping is configured or not.

# References

R2-2008623, LS on PUSCH with UL skipping, RAN2#111-e

R1-2009772, LS on PUSCH skipping with UCI in Rel-16, RAN1#103-e

R2-2100341, UL PUSCH skipping with intra-UE prioritization, RAN2#113e

# 5. Cases analysis

For case 1-3 and 1-4, it is written in the below (yellow highlights) that MAC does not need to generate a TB for the CG PUSCH.

Conclusion For the following cases, for CA and non-CA, when DG PUSCH skipping is configured and Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, MAC generates MAC PDU for the DG PUSCH and the UCI is multiplexed on the DG PUSCH. For the case 1-3 and 1-4, MAC does not generate a TB for the CG PUSCH(s) overlapping with the DG PUSCH on the same serving cell. The GG PUSCH(s) is discarded and does not participate in subsequent physical layer procedure. - (Case 1-3) DG PUSCH and CG PUSCH are overlapping and both DG/CG PUSCH are overlapping with PUCCH - (Case 1-4) DG PUSCH and CG PUSCH are overlapping and DG PUSCH is overlapping with PUCCH, and CG PUSCH is non-overlapping with the PUCCH - (Case 1-5) DG PUSCH and CG PUSCH are non-overlapping and both DG/CG PUSCH are overlapping with PUCCH

For case 1-6, both opt-3 and opt-4 do not lead to a generation of the CG PUSCH.

Working Assumption: For the case (Case 1-6) when DG PUSCH and CG PUSCH are overlapping on a serving cell and CG PUSCH is overlapping with PUCCH, and DG PUSCH is non-overlapping with the PUCCH - In Rel.16, for non-CA case, when DG PUSCH skipping is configured and Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, and when PUSCH repetition is not applied, in case of one or more CG PUSCHs overlapping with UCI and there is DG PUSCH overlapping with the CG PUSCHs on a serving cell and not overlapping with the UCI - Opt-3: - If there is data for DG, MAC generates PDU for DG PUSCH - UCI is transmitted on PUCCH. - If there is no data for DG, MAC does not generate PDU for DG or CG PUSCH - UCI is transmitted on PUCCH. - Opt-4: - If there is data for DG, MAC generates PDU for DG PUSCH - UCI is dropped together with CG PUSCH. - If there is no data for DG, MAC does not generate PDU for DG or CG PUSCH. - UCI is dropped together with CG PUSCH. Note: In RAN1#104-e, aim to resolve case 1-6 using above options as a starting point, other options are not precluded.

# 6.  TP for 38.321

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall not generate a MAC PDU for the HARQ entity if the following conditions are satisfied:

-	there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

-	the MAC PDU includes zero MAC SDUs; and

-	the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR; and

-	if either one of the following the conditions is true:

-	the grant indicated to the HARQ entity was addressed to a C-RNTI and the MAC entity is configured with skipUplinkTxDynamic with value true; or

- the grant indicated to the HARQ entity is a configured uplink grant and the MAC entity is not configured with skipUplinkTx-r16; or

-	the grant indicated to the HARQ entity was addressed to a C-RNTI or a configured uplink grant, and the MAC entity is configured with skipUplinkTx-r16 with value true, and there is no UCI to be multiplexed on the PUSCH transmission of this grant.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	C-RNTI MAC CE or data from UL-CCCH;

-	Configured Grant Confirmation MAC CE or BFR MAC CE or Multiple Entry Configured Grant Confirmation MAC CE;

-	Sidelink Configured Grant Confirmation MAC CE;

-	LBT failure MAC CE;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for BSR, with exception of BSR included for padding;

-	Single Entry PHR MAC CE or Multiple Entry PHR MAC CE;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among Configured Grant Confirmation MAC CE, Multiple Entry Configured Grant Confirmation MAC CE, and BFR MAC CE is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over transmission of NR sidelink communication.