3GPP TSG-RAN WG2 Meeting #113e	 R2-2100410

Online, Jan 25 – Feb 5, 2021

Agenda Item:	8.12.3.2

Source: 	Xiaomi Communications

Title:  	Discussion on RRM relaxation for RedCap UE

Document for:	Discussion and Decision

- 1 Introduction

In RAN2#112e meeting, the following agreements on RRM relaxation for RedCap UE are achieved:

- The target REDCAP UE, considering mobility, is not limited to a fixed UE, but can also experience some low mobility, and this, during some “stationary” periods of time.

- The RRM relaxation of REDCAP UEs is triggered based on measurements, as a baseline. Other triggering conditions for the “level-1” (still device at fixed location) UEs are not excluded, e.g. the possibility to signal their stationary property explicitly.

- R16 NR RRM relaxation procedures are taken as a baseline to study further enhancements of neighbour cells RRM relaxation for REDCAP UEs in RRC IDLE/INACTIVE.

- Relaxation of neighbour cells RRM measurements in RRC_CONNECTED will be studied in this SI/WI.

In this contribution, we will analyse the scope, criterion and method of RRM relaxation for RedCap UE.

- 2 Discussion

- 2.1 Background

In Rel-16 power saving, UE can perform relaxed measurement on intra and inter frequency in idle/inactive mode based on RRM relaxation mechanism [1], and the relaxation mechanism mainly includes relaxation criterions and relaxation methods. Furthermore in Rel-17, the enhancement of RedCap UE power saving and battery lifetime will be studied. Regarding RRM relaxation for RedCap UE, the relaxation criterions and methods in any RRC states will be discussed in this contribution.

- 2.2 Neighbour cell RRM relaxation in RRC idle/inactive

After RAN2#112e meeting, RAN2 agreed to take Rel-16 RRM relaxation mechanism as a baseline for Redcap UEs in RRC_IDLE/INACTIVE, and enhancements can be further studied. As RedCap UE is likely to stay in these states for most of time, it is meaningful and should be given a higher priority to study.

Proposal 1: Neighbour cell relaxation in idle/inactive should be given a higher priority to study.

The relaxation criterions

Based on the SID, we discuss RRM relaxation for mobility scenario in this contribution. Multiple speed evaluation levels require multiple relaxation methods which increase UE complexity, and we are also not sure the power saving gain when introducing multiple speed evaluation levels from RAN2 perspective. Therefore at most 2 levels are sufficient which stand for stationary and low mobility respectively.

Rel-16 RRM relaxation can be as a baseline, and it is a simple and straightforward way to introduce an additional SsearchDeltaP_stationary threshold to distinguish stationary from low mobility based on the range of RSRP-drop. When the criterion is fulfilled, RRM relaxation can be performed as the RSRP tends to be stable. However there are some cases that need to be clarified. UE still can move while its RSRP does not change (e.g. the UE is spinning around the cell). If RRM relaxation is performed in this case, a moving UE can not perform cell reselection/handover when a neighbour cell is missed whose RSRP is higher than serving cell’s. Therefore additional beam switching criterion for RRM relaxation can be further discussed.

Other enhancements including subscription information for stationary property and additional TsearchDeltaP_stationary. Regarding stationary property, we think that the subscription information is bound to UE type leading to restricting its use case. Even for the same type of device there may be different classification according to the user’s preference. Therefore this is not a flexible method to perform RRM relaxation.

For additional TsearchDeltaP_stationary, it will reduce the accuracy of criterion, for example, the case is that RSRP may goes through a big change then it comes back to the same value during this period, and the criterion will be fulfilled. Therefore we do not prefer this method for relaxation criterions.

Proposal 2: it is a simple and straightforward way to introduce an additional SsearchDeltaP_stationary threshold to distinguish stationary from low mobility based on the range of RSRP-drop.

The relaxation methods

It is noted that how to perform relaxation should be finally decided in RAN4, so from RAN2 perspective, we just provide some suggestions for RAN4 consideration. And Rel-16 RRM relaxation can be as a baseline for RedCap UE.

For idle/inactive RedCap UE that fulfils the criterion, it can stop the measurement on intra/ inter frequency for a longer time (e.g. T > 1h) than normal UE. And the stop time of stationary should be greater than low mobility’s.

And for other relaxation methods (e.g. minimize the number of measured frequencies), the power saving gain is not clear for now. Therefore we do not prefer to introduce these into RRM relaxation unless other companies can provide significant persuasion.

Proposal 3: For idle/inactive RedCap UE that fulfils the criterion, it can stop the measurement on intra/inter frequency for a longer time (e.g. T > 1h) than normal UE.

- 2.3 Neighbour cell RRM relaxation in RRC connected

For neighbour cell relaxation in connected, it is known that this relaxation is not supported in Rel-16. However, considering RedCap stationary UE may not have handover requirement, thus RAN2 has agreed to study this scenario in this SI. But we think it can yield power saving gain with risk of degrade system performance, and should be given a lower priority and studied carefully.

Proposal 4: Neighbour cell relaxation in connected should be given a lower priority and studied carefully.

The relaxation criterions

As mentioned above, to keep it simple and efficient, at most 2 levels are sufficient which stand for stationary and low mobility respectively.

For Neighbour cell relaxation in connected, if this scenario is introduced, the same relaxation criterion for idle/inactive can be reused. But the difference is that the network should provide the criterion parameters to UE via dedicated signalling when the network supports UE to perform RRM relaxation. And we think the criterion-based method is more reliable than other methods (e.g. UE reported information).

Proposal 5: For neighbour cell relaxation in connected, the same relaxation criterion for idle/inactive can be reused if this scenario is introduced.

The relaxation methods

For connected RedCap UE, it can yield power saving gain with risk of degrade system performance, and should be studied carefully. So it should only be required to extend the measurement period on intra/inter frequency rather than stop if we decide to introduce it. And the extend time of stationary is greater than low mobility’s.

Proposal 6: For connected RedCap UE, it should only be required to extend the measurement period on intra/inter frequency rather than stop if we decide to introduce it.

- 2.4 Serving cell RRM relaxation

For serving cell RRM relaxation in any states, it may lead to too late cell reselection/handover which has a great impact on system performance. Besides considering RRM relaxation criterion is based on serving cell measurement, due to serving cell measurement relaxation, the criterion can not perform precisely. UE may continue relaxing even if the criterion is not fulfilled. Therefore we think serving cell relaxation should not be considered.

Observation 1: Serving cell RRM relaxation would has a great impact on system performance and the criterion can not perform precisely due to serving cell measurement relaxation.

Proposal 7: Serving cell relaxation should not be considered.

- 3 Conclusions

Based on the discussion, our observations and proposals are provided as follows:

Observation 1: Serving cell RRM relaxation would has a great impact on system performance and the criterion can not perform precisely due to serving cell measurement relaxation.

Proposal 1: Neighbour cell relaxation in idle/inactive should be given a high priority to study.

Proposal 2: it is a simple and straightforward way to introduce additional SsearchDeltaP_stationary threshold to distinguish stationary from low mobility based on the range of RSRP-drop.

Proposal 3: For idle/inactive RedCap UE that fulfils the criterion, it can stop the measurement on intra/inter frequency for a longer time (e.g. T > 1h) than normal UE.

Proposal 4: Neighbour cell relaxation in connected should be given a lower priority and studied carefully.

Proposal 5: For neighbour cell relaxation in connected, the same relaxation criterion for idle/inactive can be reused if this scenario is introduced.

Proposal 6: For connected RedCap UE, it should only be required to extend the measurement period on intra/inter frequency rather than stop if we decide to introduce it.

Proposal 7: Serving cell relaxation should not be considered.

- 4 References

[1] TS 38.304