3GPP TSG-RAN2 #113e meeting	 R2-2100489

Online, January 25th-February 5th, 2021

Agenda Item:    8.8.2

Source:         Xiaomi

Title:         	Cell (re)selection based on preferred frequency(s) per slice

Document for:	Discussion and Decision

Introduction

Here are relevant agreements to issues and solutions of cell (re)selection made at RAN2#112e meeting[1]：

- Agreements

5.1: These issues will be studied in this SI:
Issue 1: The UE is unaware of the slices supported on different cells or frequencies, which prevents UE from (re)select to the cell or frequency supporting the intended slice.
Issue 2: Dedicated priorities would not be available to the UE prior to first RRC connection establishment and only remain valid before T320 expires upon entering IDLE mode. In addition, dedicated priorities are discarded each time when UE entering CONNECTED mode and need to be configured again before UE leaving CONNECTED mode. 
Issue 3: Operator may require different frequency priority configurations for the specific slice in different areas, however the dedicated priority always overwrites the broadcast priorities if configured. 
Issue 4: If the serving cell is unable to support the requested slices for the subsequent access of the UE, the serving cell may bring on handover or rejection of access request. That may increase control plane signalling overhead as well as long control plane latency for the UE to access the network.

7: The following solution approaches are captured in the TR and will be studied in this SI:
Solution 1: Legacy dedicated priority via RRCRelease message.
Solution 2: Rel-15 mechanisms such as HO, CA, DC and redirection can be used to access the intended slice in different cell 
Solution 3: Slice related cell selection info, the slice info of serving cell and neighboring cells is provided in the system information or RRCRelease message. FFS: what information is broadcast.
Solution 4: Slice related cell reselection info (e.g. Cell reselection priority per slice), the slice info of neighboring cells is provided in the system information or RRCRelease message. FFS: what information is broadcast.

In the Rel-15/16, for cell (re)selection, legacy mechanism (e.g.dedicated priority) has considered allowed NSSAI and allowed NSSAI is supported within RA. If UE requests slices outside of allowed NSSAI or moves out of RA, registration procedure is performed to update allowed NSSAI and dedicated priority can be updated accordingly. Thus, legacy mechanism can work to enable UE to access the intended slice based on Rel-15/16 system design for slicing.

As legacy mechanism can work well based on current system design for slicing, in Rel-17, the deployment scenarios should be clarified and aligned with other work groups, after which we can decide whether and how to have enhancement in Rel-17 on slice based cell (re)selection based on the deployment scenarios.

In this contribution, we share the considerations on the deployment scenarios of slice based cell (re)selection and have a further discussion on the detail aspect on cell (re)selection based on the deployment scenarios.

Discussion

In the SA2#140e meeting, SA2 sent an LSout to declare that SA2’s assumption is that all S-NSSAIs in the Allowed NSSAI are supported within the TA and also in all TAs of the RA in Rel-15/16 and not considering to change it in Rel-17. Meanwhile, SA2 asked CT1, RAN2 and RAN3 to provide their views on whether each cell in the tracking area supports the same S-NSSAI(s) in Rel-15/16[2].

At last meeting, CT1 and RAN3 provided their views towards the LS. CT1 stated that from its view, it has assumed that all S-NSSAIs in the allowed NSSAI are supported in all tracking areas of the registration area even if the registration area includes tracking areas belonging to multiple PLMNs (that are equivalent to each other). As for the question whether each cell in a tracking area supports the same S-NSSAI(s) is not in the remit of CT1[3]. And RAN3 replied “yes” (i.e. each cell in the tracking area supports the same S-NSSAI(s) in Rel-15/16) to SA2 LS[4]. To be aligned with other work groups, RAN2 should reply “yes” to SA2 LS.

Observation 1: For the question SA2 asked in the LS whether each cell in the tracking area supports the same S-NSSAI(s) in Rel-15/16, RAN3 replied “yes” and CT1 replied that is not in the remit of CT1.

Proposal 1: RAN2 should reply “yes” (i.e. each cell in the tracking area supports the same S-NSSAI(s) in Rel-15/16) to SA2 LS to be aligned with other work groups.

Considering that SA2 is not considering to change the deployment assumption in Rel-17, RAN2 should also support the deployment assumption that allowed NSSAI is supported within RA, and all cells in the same TA should support the same set of slices in Rel-17.

Proposal 2: In Rel-17, the deployment assumption that allowed NSSAI is supported with RA and all cells in the same TA support the same set of slices should be supported by RAN2 to be aligned with Rel-15/16.

There are deployment scenarios of cell (re)selection that have been captured in the TR 38.832 as following:[5]

Fig 1 Examples for slice deployment scenarios

Based on deployment assumption that all cells in the same TA should support the same set of slices, the captured deployment scenarios should be further clarified that cells in the geographical location 1-4 which support different sets of slices should be allocated to different TAs.

Proposal 3: These deployment scenarios of RAN slicing captured in the TR 38.832 should be further clarified based on the deployment assumption that all cells in the same TA should support the same set of slices.

To provide a more flexible slice deployment and meet the diverse needs of users, although all cells in the same TA support the same set of slices, different slices in the same set can be optimally served in different frequencies, for example, lower frequency band is preferable for eMBB slice while higher frequency band is preferable for URLLC slice. In this case, operator can assign different preferred frequency(s) for different slices in the same TA to meet the requirement of different services, as geographical location4 captured in the TR shown.

As different slices may have different preferred frequency(s) where slices are optimally served, besides the the supported slices info of cells/frequencies, preferred frequency(s) per slice should also be considered and provided to UE for cell (re)selection.

For cell selection, as UE has not established an RRC connection with network, UE can only get the supported slices info and preferred frequency(s) per slice from SIB to select a suitable cell.

For cell reselection, as we commented in the post-meeting email discussion[6], providing supported slices info and preferred frequency(s) per slice in RRCRelease can solve issue 1/4 and for issue 2, it is not a big issue as UE has ever established a RRC connection with network and for the maximum value of T320 is min180 which is long enough to avoid frequent reconfiguration of dedicated priorities. For issue 3, it can be resolved by configuring valid area in RRCRelease message.

Besides, supported slices info and preferred frequency(s) per slice provided in SIB are configured per cell while that in RRCRelease message are configured per UE considering network conditions and UE requests. Hence, for cell reselection, supported slices info and preferred frequency(s) per slice can be provided to UE in RRCRelease message as well as SIB, and supported slices info and preferred frequency(s) per slice in RRCRelease message always overwrite that in SIB.

Proposal 4: For cell selection, supported slices info and preferred frequency(s) per slice can be provided to UE in SIB.

Proposal 5: For cell reselection, supported slices info and preferred frequency(s) per slice can be provided to UE in both SIB and RRCRelease message. And supported slices info and preferred frequency(s) per slice in RRCRelease message always overwrite that in SIB.

Proposal 6: To resolve issue3, supported slices info and preferred frequency(s) per slice can be configured with valid area in RRCRelease message.

If a slice is assign with multiple preferred frequencies, the priority between multiple preferred frequencies may need to be defined to decide the frequency priority of cell (re)selection.

Proposal 7: If there are multiple preferred frequencies assigned to a slice, priority between preferred frequencies need to be defined.

If UE requests multiple slices which are supported on different preferred frequencies, slice priority needs to be defined for deciding the frequency priority between preferred frequency(s) of different slices used to control the frequency on which the UE camps.

In Rel-17, SA2 is considering that the prioritization of slices is up to UE internal configuration in the TR 23.700 solution#30[7]. In this case, RAN2 needs to consider whether slice priority is decided by UE or by network.

Proposal 8: If UE requests multiple slices with different preferred frequencies, slice priority need to be defined for deciding frequency priority of cell (re)selection.

Proposal 9: RAN2 needs to consider whether slice priority is decided by UE or by network.

Conclusions

During the discussion above, we have the following observations and proposals:

Observation 1: For the question SA2 asked in the LS whether each cell in the tracking area supports the same S-NSSAI(s) in Rel-15/16, RAN3 replied “yes” and CT1 replied that is not in the remit of CT1.

Proposal 1: RAN2 should reply “yes” (i.e. each cell in the tracking area supports the same S-NSSAI(s) in Rel-15/16) to SA2 LS to be aligned with other work groups.

Proposal 2: In Rel-17, the deployment assumption that allowed NSSAI is supported with RA and all cells in the same TA support the same set of slices should be supported by RAN2 to be aligned with Rel-15/16.

Proposal 3: These deployment scenarios of RAN slicing captured in the TR 38.832 should be further clarified based on the deployment assumption that all cells in the same TA should support the same set of slices.

Proposal 4: For cell selection, supported slices info and preferred frequency(s) per slice can be provided to UE in SIB.

Proposal 5: For cell reselection, supported slices info and preferred frequency(s) per slice can be provided to UE in both SIB and RRCRelease message. And supported slices info and preferred frequency(s) per slice in RRCRelease message always overwrite that in SIB.

Proposal 6: To resolve issue3, supported slices info and preferred frequency(s) per slice can be configured with valid area in RRCRelease message.

Proposal 7: If there are multiple preferred frequencies assigned to a slice, priority between preferred frequencies need to be defined.

Proposal 8: If UE requests multiple slices with different preferred frequencies, slice priority need to be defined for deciding frequency priority of cell (re)selection.

Proposal 9: RAN2 needs to consider whether slice priority is decided by UE or by network.

References

- RAN2-112e LTE DCCA Mobility RAN slicing and Multi-SIM (Tero)_2020-11-13-eom UTC

- S2-2006526 LS on Cell Configuration within TA/RA to Support Allowed NSSAI

- S2-2008351 LS from CT WG1: LS on Cell Configuration within TA/RA to Support Allowed NSSAI

- S2-2009099 LS from RAN WG3: Reply LS on Cell Configuration within TA/RA to Support Allowed NSSAI

- 3GPP TR 38.832

- [Post112-e][253] Prioritized solutions for RAN slicing v24_Rapp Summary_3

- 3GPP TR 23.700-40-050