3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2100548

25 January – 05 February 2021

Agenda item:	8.7.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	QoS for L3 UE-to-Network Relay

WID/SID:	FS_NR_SL_relay - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In section 4.6.2 of the latest version of TR 38.836 there is the following Editor's Note:

Editor note: whether other QoS solution (e.g. whether gNB can perform PDB split) is introduced depends on SA2.

This paper proposes to remove that Editor's Note based on the conclusions of SA2 study.

# 2	Discussion

SA2 concluded the following regarding to L3 Relay QoS solution [Section 8.3 of TR 38.836]:

For QoS handling, following aspects in Solution #24 and Option #2 of Solution #25 are selected as basis for normative work:

-	L3 Relay can be configured with the 5QIs and PQIs mapping. Based on the mapping or, in case of a non-configured mapping of a requested QoS parameter, based on its implementation, the L3 relay translates the Uu QoS parameters to PC5 QoS parameters and vice versa.

-	To support the dynamic QoS handling, relay UE determines the Uu QoS parameters and PC5 QoS parameters by taking into account the end-to-end QoS requirements provided by remote UE based on its configured QoS mapping information or, in case of a non-configured mapping of a requested QoS parameter, based on its implementation, and initiates PDU session modification procedure and L2 link modification procedure to setup corresponding QoS Flows over Uu and PC5.

-	The SMF of the L3 Relay provides the corresponding QoS rules and flow level QoS parameters to the L3 Relay as part of the PDU session establishment or modification procedures as defined in TS 23.502 [8], clause 4.3.2 and 4.3.3. Alternatively, reflective QoS control over Uu as defined in TS 23.501 [6], clause 5.7.5.3 can be leveraged for dynamic QoS handling of Remote UE to save on signalling between SMF and L3 Relay.

-	Based on signalled QoS rules (via SMF) or derived QoS rules (Uplink Uu via reflective QoS), the UE-to-Network Relay may use the L2 Link Modification procedures as defined in TS 23.287 [5], clause 6.3.3.4 to either move the corresponding ProSe service(s) to the mapped existing PC5 QoS flow or to set up a new PC5 QoS flow.

These conclusions are in line with the description of QoS for L3 Relay that is in section 4.6.2 of TR 38.836. The conclusions in the SA2 TR do not include any other QoS solution (e.g. PDB split performed in gNB) and thus the Editor's Note in section 4.6.2 of TR 38.836 has become obsolete.

Proposal 1: Remove the following Editor's Note from section 4.6.2 of TR 38.836:

Editor note: whether other QoS solution (e.g. whether gNB can perform	 PDB split) is introduced depends on SA2.

# 3	Conclusion

This document has made the following proposal:

Proposal 1: Remove the following Editor's Note from section 4.6.2 of TR 38.836:

Editor note: whether other QoS solution (e.g. whether gNB can perform	 PDB split) is introduced depends on SA2.