3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2100549

25 January – 05 February 2021

Agenda item:	8.7.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Path switching enhancement for L3 UE-to-Network relay

WID/SID:	FS_NR_SL_relay - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#112e, the following agreements have been made according to draft RAN2#112e meeting report:

Agreements:

Proposal 1-1 (20/22): The requirement of service continuity is only for U2N relay, but not for U2U relay, during mobility in this release.

Proposal 1-2 (22/22): R2 should study the mobility scenario of “between direct (Uu) path and indirect (via the relay) path” for U2N relay.

Proposal 1-3 (22/22): R2 deprioritize work specific to the mobility scenario of “between indirect (via a first relay UE) and indirect (via a second relay UE)” for path switching in the SI phase, which can be studied in the WI phase, if needed.

Proposal 1-5 (13+/22): R2 deprioritize the group mobility scenario in the SI phase, which may be discussed in WI phase, if needed.

Proposal 2-1 (19+/22): L2 U2N relay uses the RAN2 aspects of the R15 NR HO procedure as the baseline AS layer solution to guarantee service continuity (i.e. gNB hands over the remote UE to a target cell or target relay UE, including HO preparation type of procedure between gNB and relay UE (if needed), RRCReconfiguration to remote UE, remote UE switching to the target, and HO complete message, similar to the legacy procedure).  Exact content of the messages (e.g. HO command) can be discussed in WI phase.  This does not imply that we will send INM over Uu.

Proposal 3-1 (16+/22): Working Assumption: For service continuity in L3 U2N relay, R2 assume no AS layer solution will be studied to guarantee the service continuity, and leave it to the upper layer (e.g. application layer) solution. This does not exclude studying some enhancements in mobility scenario for other purposes.

This contribution discusses the potential enhancement of L3 U2N relay path switching.

# 2	Discussion

During the path switching of remote UE between indirect and direct path, if the lossless service continuity is required/configured for the ongoing active traffic, it can be enabled by identifying which packet has been delivered to the final destination entity (i.e. gNB for UL traffic and remote UE for DL traffic) successfully via the old path before/during path switching so that the source entity (i.e. gNB for DL traffic and remote UE for UL traffic) knows which packets it needs to transmit or retransmit via the new path after path switching. In U2N relay scenario, the successful packet delivery from the source entity to the relay UE in the first hop does not mean the packet is delivered to the final destination entity via the second hop correctly. Therefore, transferring of PDCP SN status of first hop to source entity does not enable lossless path switching. In addition, there is an end-to-end access stratum (AS) layer protocol stack for each hop independently in L3 U2N relay solution. It does not work to transfer PDCP SN status of second hop to source entity without additional information as source entity is not involved in the end-to-end AS protocol of second hop.

Observation 1: Hop specific PDCP status transfer during indirect to direct path switching does not enable lossless service continuity in L3 U2N relay.

In L3 U2N relay, the relay UE is aware of the packet delivery status of both hops. Therefore, relay UE is able to maintain and provide PDCP SN status based on the packet delivery situation on both hops. To support lossless service continuity during indirect to direct path switching, the relay UE may be triggered to transfer PDCP SN status to the source entity using the existing SN status transfer signalling procedure. But the PDCP SN status should not only take into account the PDCP SDU delivery status on the first hop but also the PDCP SDU delivery status on the second hop. To achieve this, relay UE needs to map and associate the PDCP PDUs/SDUs delivered in the first hop and second hop as there is end-to-end PDCP entity in each hop for L3 U2N relay. To make the mapping and association of the PDCP PDUs/SDUs in two hops easier, it can be configured to have one-to-one mapping of radio bearers in SL and Uu for the traffic flow that requires lossless service continuity.

Proposal 1: RAN2 should consider allowing the Relay UE to transfer PDCP SN status considering the second hop PDCP PDU/SDU delivery status during path switching in order to support lossless service continuity.

# 4	Conclusion

This document has made the following observation and proposal:

Observation 1: Hop specific PDCP status transfer during indirect to direct path switching does not enable lossless service continuity in L3 U2N relay.

Proposal 1: RAN2 should consider allowing the Relay UE to transfer PDCP SN status considering the second hop PDCP PDU/SDU delivery status during path switching in order to support lossless service continuity.