3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2100601

Elbonia, 25 January – 05 February 2021

Agenda item:	8.13.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RACH Report logging of 2-step and 4-step RACH Information

WID/SID:	NR_ENDC_SON_MDT_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

The following agreements have been made in the previous RAN2 meeting:

Agreements:

Confirm that the information included in Rel-16 RA report which also applied to 2-step RA at least contains:

Cell ID of the cell in which the RA is performed

RA purpose

Frequency information of the BWP where RA is performed

Frequency information of RA resources

Number of preambles sent on an SSB

Beam index

Contention detection per RA attempt

Beam quality indication. FFS on the details.

Agreements:

At least following RACH frequency related information should be included in RACH report for optimization of 2-step RACH:

	msgA-FrequencyStart-r17

	msgA-FrequencyStartCFRA-r17

	msgA-SubcarrierSpacing-r17

	msgA-SubcarrierSpacingCFRA-r17

	msgA-FDM-r17

	msgA-FDMCFRA-r17

In this contribution we discuss further details on logging of RACH information in the NR UE RACH Report.

# 2	NR UE RACH Report Information Logging

Currently, as per Rel-16 TS38.331, NR UE RACH Report logs 4-step RACH information of up to 8 RACH procedures. After RAN#88, 2-step RACH is also in the scope of RACH Optimization. Similar information can be logged for 2-step RACH procedures.

An LS was received from RAN3 indicating the following agreements:

Following is the information that all companies agree to include into the RACH report for two-step RA at least: - Information included in Rel-16 RA report which also applied to 2-step RA e.g. CBRA VS CFRA, SSB where the RACH access is performed. - Information that can distinguish 2-step RA from 4-step RA in the granularity of per-RA attempt. - Whether the DL beam quality associated to each random access attempt is above or below the msgA-RSRP-ThresholdSSB-r16. - For each 2-step RA attempt, whether it is fallen back to 4-step RA following indication from the network. For whether the DL beam quality is above or below the msgA-RSRP-Threshold-r16, all companies agree that this information should be included, but view is split on the granularity, i.e. whether it should be per-RA-procedure or per-RA-attempt.

If a UE logs both 4-step and 2-step RACH information, a first question to answer is whether this information should be logged in a common RACH Report or whether separate reports should be used for the logging. Since a 2-step RACH procedure may switch or fallback to a 4-step RACH procedure, it seems natural to log both information in a common RACH Report. This will allow the network to associate more efficiently the various RACH events, e.g., the events that triggered a 2-step RACH change into a 4-step RACH.

Proposal 1: NR UE RACH report should log information of both 2-step and 4-step RACH procedures.

However, in this situation the network should be able to distinguish the type of RACH attempt logged by the UE. In the NR UE RACH Report, there can be logged 4-step RACH procedures, 2-step RACH procedures and 2-step RACH procedures with fallback to 4-step RACH.

Proposal 2: Confirm the RAN3 agreement that it should be indicated per RACH attempt whether the type of RACH is 2-step or 4-step RACH.

A RACH procedure can additionally be characterized based on the RACH type that initiated it.

Proposal 3: Indicate per RACH procedure whether the RACH procedure is initiated by 4-step RACH or by 2-step RACH.

In addition, as in 4-step RACH case, a 2-step RACH procedure can be CBRA or CFRA. In order to allow the network to separately optimize CBRA and CFRA resources, logging the contention type of RACH in the NR UE RACH Report will be useful to the network.

Proposal 4: Confirm the RAN3 agreement that NR UE RACH Report should also log the type of contention mechanism of RACH access, i.e., whether Random Access is Contention-Based (CBRA) or Contention-Free (CFRA).

A 2-step RACH attempt can fail because there is a failure a) both in the PRACH preamble and the PUSCH payload or b) because even though PRACH preamble is successfully received, the PUSCH payload is not received successfully by the network. Failure in the MSGA payload can be for example because the message size chosen by the UE was too “big” to be successfully transmitted for the given channel conditions. This implies that the parameters for preamble group selection are not properly set.

The UE selects preamble group A or group B by comparing its buffer size to a threshold ra-MsgASizeGroupA. If the buffer size is greater than ra-MsgASizeGroupA and if preamble group B has been configured, then the UE will select the preambles from group B. Thus, if the network knows the size of MSGA payload chosen by the UE as well as the outcome of the transmission it can use this information to optimize ra-MsgASizeGroupA parameter, as well as the used physical layer parameters for PUSCH size of “MSGA”, preambles and preamble groups A and B. However, if the MSGA payload is lost, the network cannot know the size of MSGA payload selected by the UE.

Observation 1: In 2-step RACH, a RACH failure can occur when only MSGA payload or when both MSGA payload and preamble are not successfully received by the gNB.

Observation 2: In case both MSGA payload and preamble are not successfully received by the gNB, the latter cannot know the payload size that was used in the transmission.

Proposal 5: Include in the NR UE RACH Report the payload size transmitted in MSGA for a 2-step RACH attempt. Additionally, the group type of a preamble (group A or group B) for the 2-step RACH attempt can be logged in the NR UE RACH Report.

The actual preamble ID of the RACH transmission could also be logged. With this information the network can further evaluate potential collisions on the PUSCH Occasions, since the preamble ID will also indicate which PUSCH Occasion (for the given preamble group) were used by the UE for the MSGA PUSCH transmission. Furthermore, including the Preamble ID in the RACH Report could be useful to map CFRA RACH resources to a given preamble and subsequently to a given UE.

Proposal 6: Log the preamble ID used in the RACH transmission in the NR UE RACH Report.

MSGA PUSCH transmissions can be scrambled using PCI or the broadcasted values, dataScramblingIdentityPUSCH or msgA-dataScramblingIndex ([1]). A given gNB knows the scrambling identity being used or configured in its cells. However, since a NR UE RACH Report can store up to 8 RACH procedures it is possible that the scrambling identity is being configured to a different value for the different RACH procedures stored in the report. Including information on the type of scrambling identity per MSGA PUSCH attempt can help the network detect and overcome interference problems from neighbouring gNBs using the same PUSCH frequency and time resources.

Proposal 7: Log the type of scrambling identity (PCI or msgA-dataScramblingIndex) per MSGA PUSCH transmission in the NR UE RACH Report. In case msgA-dataScramblingIndex is configured, the UE can further include the index in the RACH Report.

Even though when a 2-step RACH procedure changes to 4-step RACH the procedure may still be successful, this would introduce an additional round-trip time which is undesirable for the network. Such additional delay could be especially problematic in unlicensed access when channel occupancy time is more critical. It should be possible for the network to know whether a 2-step RACH procedure was successful without or with fallback/switching to 4-step RACH. The UE should therefore indicate for each 2-step RACH procedure whether it was successful (and thus no fallback/switching was needed), whether there was a fallback to 4-step RACH or whether there was a switching to 4-step RACH.  The network in this way can collect statistics on the average number of messages/time needed until a RACH process (4-step, 2-step, or 2-step with fallback) is successful.

Proposal 8: Confirm the RAN3 agreement that for each 2-step RACH attempt in the NR UE RACH report the UE should indicate whether there was a fallback/switching to 4-step RACH. If 2-step RACH succeeded without changing to 4-step RACH this can also be indicated.

A UE that first initiated a 2-step RACH procedure may change its type to 4-step RACH due to several reasons. One case is when the UE transmits a maximum of msgA-TransMax preamble transmissions, but the gNB does not receive the preamble. In this case, the UE switches autonomously to 4-step RACH.  As another alternative, in unlicensed access, a UE may need to do a separate LBT to send the PRACH preamble and then another one for sending PUSCH for payload transmission. In this case, the UE can change the RACH type to 4-step RACH because the channel is unavailable and not because of failure in PUSCH detection. As another example, the UE may receive a FallbackRAR from the network in which case it performs a fallback from 2-step to 4-step RACH. Besides fallback from 2-step to 4-step RACH, another possibility is fallback from 4-step CFRA to 4-step CBRA in the event that no suitable beam is found by the UE.

Proposal 9: Include in the NR UE RACH Report a reason for changing (switching after N attempts or falling back) of a RACH procedure.

A UE may also log in the RACH report power level information about the switch or fallback of a 2-step to 4-step RACH attempt. For example, it may be logged whether the UE was operating at maximum power at the time when the RACH procedure changed from 2-step to 4-step RACH. MSGA preamble and MSGA PUSCH transmission power of the last attempt before the change from 2-step to 4-step RACH can also be logged in the NR UE RACH Report. By logging MSGA preamble and PUSCH transmission power the network can determine whether the reason why it did not receive the transmission by the UE is due to transmission power problems. Network can collect this information from one or more UEs and use it to readjust the power parameters.

Proposal 10: Power level information can be included in the NR UE RACH Report. This information may comprise MSGA preamble and MSGA PUSCH transmission power level of the last RACH attempt before the change from 2-step to 4-step RACH or whether the UE transmits at its maximum power level.

Regarding the open issue from the RAN3 LS and the FFSs from the previous RAN2 meeting on beam quality indication, in 2-step RACH, PUSCH resource reservation is done in a static manner and selection between 2-step and 4-step RACH is based on whether RSRP measured at initiation of the RA procedure is above msgA-RSRP-ThresholdSSB-r16. However, this is not checked for further attempts. Per RA procedure logging would be useless since if 2-step is initiated it implies RSRP is above the threshold. Per attempt logging would make more sense since whether RSRP is above threshold for further attempts is the part that is unknown to the network. To optimize the PUSCH occasions, network needs to know the measured RSRP at which a UE selects 2-step or 4-step RACH to balance the threshold parameter configuration and PUSCH resource efficiency. This information can be logged in the NR UE RACH Report.

Proposal 11: It should be logged in the NR UE RACH Report whether DL beam quality associated to each random access attempt is above the msgA-RSRP-ThresholdSSB-r16 (or the actual RSRP value for the associated beam) per RACH attempt granularity.

# 3	RACH Prioritization

RACH prioritization has been supported for handover and beam failure recovery since Rel-15 for 4-step RACH where different power ramping steps and scaling factors for back-off can be configured. RACH prioritization is also applicable to 2-step RACH. In RAN2 #107 it was also agreed to provide RACH prioritization for access identity 1 (where Multimedia Priority Services (MPS) are used exclusively) and for Mission Critical Services (MCS). In [2], the impact of prioritization on initial access attempts for MPS and MCS is shown and compared to the non-prioritized access case. It was observed that the benefit of prioritization decreases as the number of priority users grows. This is a natural effect of prioritization since when it is enabled for a set of UEs it reduces the back-off parameter as compared to non-prioritized access which can therefore increase the probability that those UEs collide with each other in their RACH attempts.

Proposal 12: RAN2 should consider logging of RACH prioritization information in the NR UE RACH Report.

# 4	Conclusion

We make the following observations and proposals:

Proposal 1: NR UE RACH report should log information of both 2-step and 4-step RACH procedures.

Proposal 2: Confirm the RAN3 agreement that it should be indicated per RACH attempt whether the type of RACH is 2-step or 4-step RACH.

Proposal 3: Indicate per RACH procedure whether the RACH procedure is initiated by 4-step RACH or by 2-step RACH.

Proposal 4: Confirm the RAN3 agreement that NR UE RACH Report should also log the type of contention mechanism of RACH access, i.e., whether Random Access is Contention-Based (CBRA) or Contention-Free (CFRA).

Observation 1: In 2-step RACH, a RACH failure can occur when only MSGA payload or when both MSGA payload and preamble are not successfully received by the gNB.

Observation 2: In case both MSGA payload and preamble are not successfully received by the gNB, the latter cannot know the payload size that was used in the transmission.

Proposal 5: Include in the NR UE RACH Report the payload size transmitted in MSGA for a 2-step RACH attempt. Additionally, the group type of a preamble (group A or group B) for the 2-step RACH attempt can be logged in the NR UE RACH Report.

Proposal 6: Log the preamble ID used in the RACH transmission in the NR UE RACH Report.

Proposal 7: Log the type of scrambling identity (PCI or msgA-dataScramblingIndex) per MSGA PUSCH transmission in the NR UE RACH Report. In case msgA-dataScramblingIndex is configured, the UE can further include the index in the RACH Report.

Proposal 8: Confirm the RAN3 agreement that for each 2-step RACH attempt in the NR UE RACH report the UE should indicate whether there was a fallback/switching to 4-step RACH. If 2-step RACH succeeded without changing to 4-step RACH this can also be indicated.

Proposal 9: Include in the NR UE RACH Report a reason for changing (switching after N attempts or falling back) of a RACH procedure.

Proposal 10: Power level information can be included in the NR UE RACH Report. This information may comprise MSGA preamble and MSGA PUSCH transmission power level of the last RACH attempt before the change from 2-step to 4-step RACH or whether the UE transmits at its maximum power level.

Proposal 11: It should be logged in the NR UE RACH Report whether DL beam quality associated to each random access attempt is above the msgA-RSRP-ThresholdSSB-r16 (or the actual RSRP value for the associated beam) per RACH attempt granularity.

Proposal 12: RAN2 should consider logging of RACH prioritization information in the NR UE RACH Report.

# References

[1] 3GPP TS 38.211, NR; Physical Channels and Modulation, V16.4.0 (2020-12) (Release 16)

[2] R2-1911735 PRACH Prioritization for MPS and MCS