3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2100605

Elbonia, 25 January – 05 February 2021

Agenda item:	8.13.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Delay measurement configuration for DC cases

WID/SID:	NR_ENDC_SON_MDT_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#112 post e-meeting/e-mail discussions, reported in R2-2100703, the delay measurement (M6) coordination between MN and SN for MDT was discussed. In this contribution we provide some elements for the configuration.

# 2	Background

In the [852] e-mail discussion on L2 measurements, the total delay measurement M6 is discussed. The delay configuration and estimation coordination is needed between MN and SN for MN/SN terminated split bearers and MN terminated SCG bearer and SN Terminated MCG bearer.

Figure 1: Different type of bearers used in DC

# 3	Discussion

To begin we would like to make the following observation:

Observation 1: In case of split bearer architecture, for MN/SN terminated split bearer and MN terminated SCG bearer and SN terminated MCG bearer scenarios, the delay measurements configuration should be co-ordinated.

As discussed in R2-2009427, we think that there may be some differences in the configuration for MN terminated vs SN terminated bearers.

##### Observation 2: The configuration are different for MN Terminated vs SN Terminated bearers.

We observed the following steps for the setup for the different cases:

##### Configuration of MN/SN terminated Split bearer / MN Terminated SCG bearer / SN terminated MCG bearer for DL

##### Configuration of MN terminated Split bearer / MN Terminated SCG bearer for UL

##### Configuration/Reporting of SN terminated Split bearer / SN Terminated MCG bearer for UL

In split bearer architecture, for DL, it’s proposed that MN CU CP triggers the measurement configuration with other network components.

For example, MN/SN terminated split bearer/ MN terminated SCG bearer / SN terminated MCG bearer for DL, the delay measurement  can be configured by MN CU CP to all other components. i.e MN CU CP to MN CU UP, MN DU and SN CU CP (which in turn configures SN CU CP and SN DU)

Concerning UE configuration, we propose that the node hosting the PDCP entity of the bearer that needs to be measured configures the UE as discussed in  R2-2009427.

For example, in case of MN terminated split bearer/ MN Terminated SCG bearer for UL delay measurement, UE is configured by MN CU CP apart from other NW components configuration, which is similar to DL.

Similarly, for SN terminated split bearer/ SN Terminated MCG bearer for UL delay measurement, UE is configured by SN CU CP, apart from other NW components configuration, which is similar to DL.

Proposal 1: In split bearer architecture, for DL delay measurement, it’s proposed that MN CU CP always triggers the measurement configuration with other network components.

Proposal 2: In split bearer architecture, for UL delay measurement, it’s proposed the node that hosts PDCP can configure the UE.

# 4	Conclusion

In this contribution we have made the following observations and proposals:

Observation 1: In case of split bearer architecture, for MN/SN terminated split bearer and MN terminated SCG bearer and SN terminated MCG bearer scenarios, the delay measurements configuration should be co-ordinated.

##### Observation 2: The configuration are different for MN Terminated vs SN Terminated bearers.

We propose the following steps for the setup for the different cases:

Proposal 1: In split bearer architecture, for DL delay measurement, it’s proposed that MN CU CP always triggers the measurement configuration with other network components.

Proposal 2: In split bearer architecture, for UL delay measurement, it’s proposed the node that hosts PDCP can configure the UE.