3GPP TSG RAN WG2 Meeting #113e	      					R2-2100615

e-Meeting, 25 Jan – 05 Feb 2020

Agenda item:	8.5.2

Source:		Intel Corporation

Title: 		RAN Enhancements for Support of Timing Synchronization

Document for: 	Discussion

# Introduction

Time synchronization under Rel-17 is included in the WID objectives [1] as follows

Enhancements for support of time synchronization:

a)  RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2]

b)  Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4]

The topic of propagation delay compensation has been discussed in the last two RAN2 meetings #111e, 112e. Mobility issues were briefly discussed in RAN2 #111e, however, no agreement was reached. In this contribution we provide an overview of the progress made for the propagation delay compensation topic, and the RAN2 impact for support of the multiple PDC options currently on the table. We also discuss the issue of accurate reference time information in mobility scenarios (e.g. during handover).

# Discussion

Propagation Delay Compensation

In RAN2 #112e, the topic of propagation delay compensation was discussed, specifically in reference to the time synchronization budget over the Uu interface for two scenarios, Control-to-control communication and Smart-grid communications and it was communicated to RAN1 in [10]. Following agreements were made in RAN2 #112e

Agreements

1: RAN2 should consider the following three scenarios, with a focus on Scenario 2 and 3:

•	Scenario 1: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the CN. The 5GS introduced error is caused by the relative time-stamping inaccuracy at the NW-TT and the DS-TTs.

•	Scenario 2: In the control-to-control communication use case, where TSC devices behind a target UE are synchronized to any TD, from a GM behind the UE. The 5GS introduced error is caused by the relative time-stamping inaccuracies at the involved DS-TTs.

•	Scenario 3: In the smart grid use case, where the TSC devices behind a target UE are synchronized to the 5G GM TD. The 5GS introduced error is caused by the synchronization of the 5G clock to the DS-TT.

2	RAN2 should evaluate the synchronicity budget by dividing the 5GS E2E path into three parts: Network, Device, and Uu interface. Where the Uu interface is understood as the maximum 5GS time synchronization error between the UE and the gNB-DU (i.e. DU-CU interface error is not included)

3 RAN2 assumes the two Uu interfaces in Scenario 2 have the same time synchronization error budget.

4 The Uu interface budget for Scenario 1, 2 and 3 are respectively calculated as following:

•	Scenario 1: Uu budget = 900ns – Device – Network scenario1

•	Scenario 2: Uu budget = (900ns – 2xDevice – 2xNetwork scenario2)/2 (assumption is based on GPTP)

•	Scenario 3: Uu budget = 1000ns – Device – Networkscenario3 (baseline assumption that this is based on GNSS)

5  The Device part time synchronization accuracy budget is assumed to be in the range ±50 to ±100ns, this applies to all three scenarios

6  The error caused by the limited granularity of referenceTimeInfo-r16 IE (±5ns) is to be included in the network part budget, and RAN1 should be informed not to include this error in Uu interface.

7  The Network part time synchronization accuracy budget for Scenario 1, 2, and 3 are assumed to be the following:

•	Scenario 1: ±120 to ±200ns (NetworkScenario1) (assuming 3-5 hops worst case scenario

•	Scenario 2: ±240 to ±400ns (2xNetworkScenario2) (assuming 6-10hops worst case scenario)

•	Scenario 3: ±100ns (NetworkScenario3)

8	Based on Proposal 4, 5, 6 and 7, the per Uu interface time synchronization accuracy for Scenario 1, 2 and 3 are as following:

•	Scenario 1: ±595ns to ±725ns

•	Scenario 2: ±145ns to ±275ns

•	Scenario 3: ±795ns to ±845ns

9	LS to RAN1 providing the scenarios and values.  Indicate to RAN1 that they should aim to meet the most stringest requirements, but a number within the range is also acceptable

10	It is up to RAN1 to decide which PDC options should be supported for Scenario 1, 2 and 3 in Release-17.

Agreements:

The following options for propagation delay compensation are further studied in RAN1

- Option 1: TA-based propagation delay

  - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity).

  - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te)

  - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected)

- Option 2: RTT based delay compensation: Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning).

As highlighted in the agreements above, it was left up to RAN1 to decide which propagation delay compensation options should be supported for Rel-17. In this section, we would like to analyze each of the options from RAN2 perspective in view of which specification changes and impacts are expected with each of the PDC options. The following options have been agreed in RAN1 meeting # 102e.

The follows are the main approaches need to be re-evaluated and down-selected in R17:

-	Option 1a: Leave this up to UE implementation and do not specify any enhancements.

-	Option 1b: Leave this up to UE implementation but specify finer granularity of TA command to assist the UE calculation.

-	Option 2a: Specify in the specifications propagation delay compensation based on TA command (no TA granularity enhancements).

-	Option 2b: Specify in the specifications propagation delay compensation based on TA command and enhance TA granularity.

-	Option 3: Perform pre-compensation on the network side (up to network implementation) and add the indication in the network to UE signalling that the time information was pre-compensated.

- Option 4: reuse some aspects of the positioning framework timing difference measurements for propagation delay compensation

Similarly, in RAN2 #111e post email discussion [4], following options were put forth

RAN2 options are comparable to the PDC options agreed in RAN1, with the exception of the additional Option 3, to perform pre-compensation on the network side. Note that this option is precluded from RAN1 options since it is in RAN2 scope. Based on the comment by the email rapporteur as follows, we assume that network pre-compensation is not precluded at this point.

network pre-compensation can be considered as an additional option only relevant to the signalling to the UE, and not as a stand-alone PD estimation technique.  The rapporteur propose that RAN2 acknowledges the propagation delay estimation options from RAN1, and adds the additional option (3) where the network conducts pre-compensation (no RAN1 impact).

In our companion RAN1 paper, detailed propagation delay analysis with comments on the feasibility of each of the PDC option with reference to meeting the synchronization budget has been put forth. From among the considered PDC options, following three meet the upper-bound requirement for the Uu budget, and none of the options can meet the most stringent requirement put forth by RAN2 for control-to-control communication scenario with two Uu interfaces (Scenario 2). For smart grid communications, any option including the legacy TA based compensation is deemed to meet the synchronization budget requirements based on the analysis by RAN1. Here we provide some preliminary analysis of the expected RAN2 impact for each of the three identified viable options

- Enhanced granularity TA-based compensation with tightened requirements (reduced Te): Any change to the UE uplink transmission error, or Te is in RAN4 scope and does not have any specification impact for RAN2. On the other hand, enhanced TA granularity may bring about specification impact in RAN2. Since the legacy TA method is stable, it is expected that the gNB may simultaneously support both legacy TA as well as TA with increased granularity. In which case, it may be required to introduce a new MAC CE to support the new increased granularity TA option.

- Pre-compensation at gNB: Network compensation can avoid error components in the propagation delay compensation which arise due to TA estimation at the UE side. Similar to Release 16, as in the case of UE compensation, the details of the propagation delay compensation method can be left to the gNB implementation. However, it will be imperative to introduce some measure to indicate to the UE when pre-compensation has been applied by the gNB to avoid double compensation of the propagation delay at both the UE and the network side. For the unicast case, either the gNB or the UE may perform propagation delay compensation, in which case the network can provide RRC signalling for configuration to disable propagation delay compensation at the UE. As earlier proposed in R2-2001047 [5], an optional field delayCompensationAllowed may be introduced in the IE ReferenceTimeInfo.

- RTT-based compensation: The RTT based compensation method is based on the existing positioning framework [3] which makes use of the propagation delay measurements to determine the distance between the UE and the gNB. Compared to the TA based compensation, a main difference is absence of UE uplink transmission error (Te) if the Rx-Tx measurement is delivered from the gNB to the UE,  and a different signaling granularity for Rx-Tx time difference. The RTT-based compensation for IIoT setting may require the use of the UE Rx-Tx time difference measured by the UE and the measured gNB Rx-Tx time difference measurements. These measurements can be exchanged between the UE and the gNB through unicast signalling similar to the case of network pre-compensation where the signaling requirement will be based on the reporting method chosen. Three methods can be identified for reporting

  - Option 1: In this option the UE reports the UE Rx-Tx measurement to the gNB, which calculates the propagation delay as (UE Rx-Tx + gNB Rx-Tx) and signals it to the UE, where compensation is left up to UE implementation

  - Option 2: In this option the UE reports the UE Rx-Tx measurement to the gNB, which calculates the propagation delay as (UE Rx-Tx + gNB Rx-Tx) and performs pre-compensation at the network side in which case it would require RRC signaling for configuration to disable the propagation delay compensation at the UE to avoid double compensation.

  - Option 3: The gNB reports the gNB Rx-Tx time difference to the UE and the UE can then calculate the propagation delay, where the compensation is left up to UE implementation. In signaling the gNB Rx-Tx measurement to the UE, whether higher layer unicast/groupcast signaling may be adopted or PHY layer unicast/groupcast will be used needs further discussion in RAN1 and RAN2.

Proposal 1: RAN2 to wait for the analysis performed by RAN1 to gauge the viability of PDC methods, and further discuss the RAN2 impacts for the supported PDC options as decided by RAN1.

Timing Synchronization maintenance during mobility

In this section we discuss timing synchronization continuity in mobility scenarios (e.g. during handover). The procedure for exposure of time synchronization is provided in SA2 solution # 7 and 8 in TR 23.700-20 [6], which describes the process for the AF to request time synchronization with specified requirements and supply information to be used to configure the time synchronization procedure for connected devices. When the 5GS time source is used by the 5GS, either the UPF/NW-TT is configured to create the gPTP message for conveying the timing information as seen in method 2 in 23.700-20 [6] or the 5G-AN provides a 5G reference time to the UE via the 3gpp radio layer e.g. via SIB/RRC messages. It is assumed that the 5G internal system clock is made available to all user plane nodes in the 5G system. Note that in the SA2 solutions in TR 23.700-20 [6] regarding timing synchronization exposure method, the UE impact is only in the NAS layer with no potential RAN impact identified. Moreover, timing synchronization maintenance in handover is not captured in these SA2 solutions.

Proposal 2: Given the current SA2 discussion, no issues are identified regarding timing synchronization in mobility.

Nonetheless, if it is considered relevant to discuss timing synchronization maintenance during handover in high mobility and time critical environments, we can refer to the control-to-control communications use-case in TS 22.104 [2] with the stringent 5GS synchronization budget of 900 ns. For handover scenarios, the target gNB can know if the UE requires reference time from the UEAssistanceInformation sent by UE [7].  In addition, the source gNB can provide reference time information to UE proactively e.g. when deciding to perform handover, enabling the target gNB to be synchronized to the 5GS within several seconds after handover such that the synchronization requirement can be satisfied, hence ensuring synchronization continuity. The UE only needs to be provided with reference time when the clock drift between UE clock and 5GS time  the synchronization budget. Based on the RAN4 requirement on frequency error in TS 38.101-1 [8], “the UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of 1 ms compared to the carrier frequency received from the NR Node B.” This implies that the UE can afford a maximum clock drift of ±0.1 PPM which is equivalent to 0.1us per second. For time synchronization with 5G network, under the reference clock model, where 5sthe overall drift considering both gNB and UE with respect to the TSN GM becomes 0.2us per second. This implies that to adhere to the precision requirement of 1us as given in D.3 TS 22.104 [2], the network needs to deliver the reference time at most every 5s (a conservative approach would be a refresh time of 1s as discussed in R2-2004585 [9]). This implies that once the UE indicates that it requires reference time information using UEAssistanceInformation, it can rely on the periodic clock refresh from the network which is sufficiently frequent to contain the clock drift within the synchronicity budget.

Proposal 3: If there is a need to handle timing synchronization maintenance during handover, from RAN2’s perspective, handling can be up to gNB implementation without specification impact (e.g. the source gNB can proactively provide reference time information to the UE when deciding to perform handover to ensure synchronization continuity).

Observation 1: If the UE indicates to the gNB via UEAssistanceInformation, the need for reference time information, it can rely on the periodic clock refresh from the network without violating the clock synchronization budget.

# Conclusions

In this contribution we discuss the issues of propagation delay compensation and mobility in Rel 17 IIoT and make the following proposals and observations.

Proposal 1: RAN2 to wait for the analysis performed by RAN1 to gauge the viability of PDC methods, and further discuss the RAN2 impacts for the supported PDC options as decided by RAN1.

Proposal 2: Given the current SA2 discussion, no issues are identified regarding timing synchronization in mobility.

Proposal 3: If there is a need to handle timing synchronization maintenance during handover, from RAN2’s perspective, handling can be up to gNB implementation without specification impact (e.g. the source gNB can proactively provide reference time information to the UE when deciding to perform handover to ensure synchronization continuity).

Observation 1: If the UE indicates to the gNB via UEAssistanceInformation, the need for reference time information, it can rely on the periodic clock refresh from the network without violating the clock synchronization budget.

# References

[1] RP-201310 Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, RAN 88e, June 2020.

[2] TS 22.104 V17.2.0 (2019-12) Service requirements for cyber-physical control applications in vertical domains

[3] TS 38.305 Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN

[4] R2-2009755: Summary of E-mail discussion: [Post111-e][924][R17 URLLC/IIoT] Propagation delay for TSN (Nokia)

[5] R2-2001047 Propagation delay compensation, 3GPP RAN2 109e, Feb 2020.

[6] TR 23.700-20 V1.0.0 (2020-09) Study on enhanced support of Industrial Internet of Things (IIoT) in the 5G System (5GS)

[7] 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[8] 3GPP TS 38.101-1 V16.5.0 (2020-09) NR; User Equipment (UE) radio transmission and reception

[9] R2-2004585 RAN2#110 Open issues on Accurate Reference Timing

[10] R2-2010837 Reply LS on propagation delay compensation enhancements