3GPP TSG-RAN WG2#113 e-meeting                                   R2-2100681

Online, Jan 25th – Feb 5th, 2021                       (Revision of R2-2010251/2010252)

Source: 		Sharp, Ericsson

Title:	UE information transmission in LTE CHO case

Agenda Item:		6.7.2

Document for:	Discussion and Decision

Introduction

In this contribution, similar issue discussed in [1] is reconsidered for LTE system.

In RAN2#110 e-meeting, J032 is concluded as rejected in [2] for NR RRC. However, since the details was not touched and discussed by companies during the meeting and this issue also exists for LTE RRC specification, so we would like to explain more about the issue in LTE case in this contribution and re-discuss it. Possible changes to 36.331 RRC specification are provided in the annex part.

Agreements (no document was submitted)

J032: ConcReject.

S307: ConcAgree.

S306: ConcReject.

X004: ConcAgree: Intent is agreeable, but need to discuss how to clarify the wording (current condition may cover this)

X005: ConcAgree (add MAC reset)

Discussion

In current RRC specification [2], a UE may send RRC messages including UE information (i.e. InDeviceCoexIndication/UEAssistanceInformation/MBMSInterestIndication/SidelinkUEInformation) to report the newest UE assistance information to the source cell in the handover preparation phase. As this is done after the source cell has sent the Handover Request message to the target cell, such UE information cannot reach the target cell. In order to make the target cell of the handover acquire the latest UE information, the UE may re-initiate transmission of the UE information messages at the end of the handover procedure.

#### 5.3.5.4	Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover)

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

<OMIT>

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure; or

1>	if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured:

2>	stop timer T304;

<OMIT>

2>	if the UE is configured to provide IDC indications:

3>	if the UE has initiated the transmission of an InDeviceCoexIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

2>	if the UE is configured to provide power preference indications, overheating assistance information, SPS assistance information, delay budget report or maximum bandwidth preference indications:

3>	if the UE has initiated the transmission of a UEAssistanceInformation message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	if the UE has initiated the transmission of a MBMSInterestIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

4>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4>	determine the set of MBMS services of interest in accordance with 5.8.5.3a;

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	if SystemInformationBlockType18 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of sidelink communication related parameters relevant in target PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if SystemInformationBlockType19 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of sidelink discovery related parameters relevant in target PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo; or

2>	if SystemInformationBlockType21 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of V2X sidelink communication related parameters relevant in target PCell (i.e. change of v2x-CommRxInterestedFreqList or v2x-CommTxResourceReq) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo:

3>	initiate transmission of the SidelinkUEInformation message in accordance with 5.10.2.3;

<OMIT>

2>	the procedure ends;

Observation 1: the UE may initiate transmission of a UE information RRC message(s) at the end of a handover procedure, if it has transmitted such a RRC message(s) within 1 second before reception of the handover command.

In conditional handover case, UE stores the received condition reconfiguration upon reception of the conditional handover command and only executes the handover when the associated triggering condition is fulfilled. That means the time that UE receives a conditional handover command is not always the same to the time of executing the conditional handover.

Take SidelinkUEInformation message for example, in Figure 1.

- At time T1, the UE receives the CHO command, and then starts the condition evaluation.

- At T2, the UE triggers transmission of a SidelinkUEInformation message.

- Later at T3, the CHO is executed due to the condition for evaluation is considered as met.

Figure 1

If following the current specification as in observation 1, UE may only initiate transmission of a SidelinkUEInformation message if it has initiated the transmission of such messages before T1. Then the SidelinkUEInformation message transmitted at T2 will not be re-transmitted by UE to the target cell. This may lead to a result that the target cell cannot get the latest UE information.

Observation 2: in CHO case, the target cell cannot get the latest UE Information that has been transmitted before the CHO execution and after reception of the CHO command.

To deal with this issue, the specification can be clarified to allow the UE to initiate the transmission of UE information RRC message(s) if it has been initiated for transmission before the CHO execution and after reception of the CHO command

Proposal: allow UE to initiate the transmission of InDeviceCoexIndication/UEAssistanceInformation/MBMSInterestIndication/SidelinkUEInformation message if it has been initiated for transmission before the CHO execution and after reception of the CHO command.

Conclusion

The observations and proposals in this contribution include:

Observation 1: the UE may initiate transmission of a UE Information RRC message(s) at the end of a handover procedure, if it has transmitted such a RRC message(s) within 1 second before reception of the handover command.

Observation 2: in CHO case, the target cell cannot get the latest UE information that has been transmitted before the CHO execution and after reception of the CHO command.

Proposal: allow UE to initiate the transmission of InDeviceCoexIndication/UEAssistanceInformation/MBMSInterestIndication/SidelinkUEInformation message if it has been initiated for transmission before the CHO execution and after reception of the CHO command.

Reference

- R2-2100680 UE information transmission in NR CHO case, Sharp, Ericsson

- Chairman notes for RAN2#110 e-meeting

- TS 36.331-g30

Annex

Start of change

#### 5.3.5.4	Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover)

If the RRCConnectionReconfiguration message includes the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

<OMIT unchanged part>

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission;

1>	if MAC successfully completes the random access procedure; or

1>	if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured:

2>	stop timer T304;

2>	if daps-HO is configured for any DRB:

3>	stop timer T310, if running;

3>	stop timer T312, if running;

3>	for each DAPS bearer trigger UL data switching, as specified in TS 36.323 [8];

2>	release rach-Skip;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 3:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	if the UE is configured to provide IDC indications:

3>	if the UE has initiated the transmission of an InDeviceCoexIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo, or, in case the RRCConnectionReconfiguration message was included in a conditional reconfiguration, if the UE has transmitted an InDeviceCoexIndication message since reception of conditional reconfiguration configuration:

4>	initiate transmission of the InDeviceCoexIndication message in accordance with 5.6.9.3;

2>	if the UE is configured to provide power preference indications, overheating assistance information, SPS assistance information, delay budget report or maximum bandwidth preference indications:

3>	if the UE has initiated the transmission of a UEAssistanceInformation message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo, or, in case the RRCConnectionReconfiguration message was included in a conditional reconfiguration, if the UE has transmitted a UEAssistanceInformation message since reception of conditional reconfiguration configuration:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3;

2>	if SystemInformationBlockType15 is broadcast by the PCell:

3>	if the UE has initiated the transmission of a MBMSInterestIndication message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo, or, in case the RRCConnectionReconfiguration message was included in a conditional reconfiguration, if the UE has transmitted a MBMSInterestIndication message since reception of conditional reconfiguration configuration:

4>	ensure having a valid version of SystemInformationBlockType15 for the PCell;

4>	determine the set of MBMS frequencies of interest in accordance with 5.8.5.3;

4>	determine the set of MBMS services of interest in accordance with 5.8.5.3a;

4>	initiate transmission of the MBMSInterestIndication message in accordance with 5.8.5.4;

2>	if SystemInformationBlockType18 is broadcast by the target PCell; and the UE initiated the transmission of  a SidelinkUEInformation message indicating a change of sidelink communication related parameters relevant in target PCell (i.e. change of commRxInterestedFreq or commTxResourceReq, commTxResourceReqUC if SystemInformationBlockType18 includes commTxResourceUC-ReqAllowed or commTxResourceInfoReqRelay if PCell broadcasts SystemInformationBlockType19 including discConfigRelay) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo, or, in case the RRCConnectionReconfiguration message was included in a conditional reconfiguration, if the UE has transmitted a SidelinkUEInformation message since reception of conditional reconfiguration configuration; or

2>	if SystemInformationBlockType19 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of sidelink discovery related parameters relevant in target PCell (i.e. change of discRxInterest or discTxResourceReq, discTxResourceReqPS if SystemInformationBlockType19 includes discConfigPS or discRxGapReq or discTxGapReq if the UE is configured with gapRequestsAllowedDedicated set to true or if the UE is not configured with gapRequestsAllowedDedicated and SystemInformationBlockType19 includes gapRequestsAllowedCommon) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo, or, in case the RRCConnectionReconfiguration message was included in a conditional reconfiguration, if the UE has transmitted a SidelinkUEInformation message since reception of conditional reconfiguration configuration; or

2>	if SystemInformationBlockType21 is broadcast by the target PCell; and the UE initiated the transmission of a SidelinkUEInformation message indicating a change of V2X sidelink communication related parameters relevant in target PCell (i.e. change of v2x-CommRxInterestedFreqList or v2x-CommTxResourceReq) during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo, or, in case the RRCConnectionReconfiguration message was included in a conditional reconfiguration, if the UE has transmitted a SidelinkUEInformation message since reception of conditional reconfiguration configuration:

3>	initiate transmission of the SidelinkUEInformation message in accordance with 5.10.2.3;

2>	remove all the entries within VarConditionalReconfiguration, if any;

<OMIT unchanged part>

2>	the procedure ends;

End of change