3GPP TSG-RAN WG2 Meeting #113-e		R2-2100683

Online, Jan 25th – Feb 5th 2021

Source:	vivo

Title:          Discussion on A-PRS and semi-persistent PRS

Agenda Item:	8.11.2.1

Document for:	Discussion and Decision

- Introduction

During RAN1#102-e[1], the following agreement has been reached for A-PRS and semi-persistent PRS:

Agreement: - Semi-persistent and a-periodic transmission and reception of DL PRS will be investigated in Rel-17. - FFS: the details on when and how to enable semi-persistent and a-periodic DL PRS - FFS: to be supported for which positioning methods, e.g., - UE-assisted and/or UE-based positioning - DL positioning and/or Multi-RTT

And, in RAN1#103-e meeting [2], the following agreement has been reached for A-PRS and semi-persistent PRS which can be further studied and if needed, specified in WID.

Agreement: Capture the following in the TR: Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work. - Note: Aperiodic reception in the above corresponds to DCI-triggered reception Agreement: Capture the following in the TR[3]: Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work. - Note: Semi-persistent reception in the above corresponds to MAC-CE activated reception

In this contribution, we will discuss what can be studied for A-PRS and semi-persistent PRS from RAN2 and how to align with the agreement from RAN1 from RAN2’s perspective for on-demand PRS, A-PRS and semi-persistent PRS.

- Discussion

At RAN#86 meeting, the study item on NR Positioning Enhancements was approved [4]. the SI includes the enhancement for the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.)

And the R17 requirement has been defined in RAN1 as below.

Agreement: - In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - End-to-end latency for position estimation of UE (< 100 ms) - Physical layer latency for position estimation of UE (< 10 ms) - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< 0.2 m) for 90% of UEs - Vertical position accuracy (< 1 m) for 90% of UEs - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired) - Physical layer latency for position estimation of UE (<10ms)

According to the agreement, 100ms, even 10ms is the R17 target for End-to-end latency. While based on the consensus for the latency evaluation in RAN1 and the latency components are provided by RAN2, the requirement can not be met by NR positioning without enhancement (the detailed are provided in [5]). And it is intuitive that how to achieve the R17 latency target is a critical issue in R17.

Conclusion: Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings - Note: The following assumptions are made - One DL PRS frequency layer in FR1 - CSSF = 1 - NRxBeam, i = 1, - Nsample = 4 (DL PRS RSTD measurements are done across 4 DL PRS periods) - Both DL PRS periodicity and MGRP are equal to 20ms - Configured DL PRS resources are within UE DL PRS processing capacity (N,T) = (0.5ms, 8ms)

Table 1: 	Latency Components

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TUEProc-RRCReconf | 10 | RRC Reconfiguration processing |
| TUEProc-RRCDLInfo | 5 | RRC DL information transfer |
| TUEProc-RRCULInfo | 2-5 | RRC UL information transfer |
| TUEProc-RRCLocationMeas | 2-5 | RRC Location Measurement Indication |
| TUEProc-LPPCapab | 10-20 | LPP Provide Capabilities |
| TUEProc-LPPAssi | 10 | LPP Provide Assistance Data |
| TUEProc-LPPLocationRe | 5 | LPP Request/Provide Location Information |
| TUEProc-MAC-SRSAct | 1-3 | MAC-CE SRS Activation/Deactivation |
| TgNBProc-RRC | 3 | RRC Processing |
| TgNBProc-NRPPa | 3 | NRPPa Processing |
| TgNBProc-NAS/LPP | 3 | NAS/LPP Processing |
| TAMFProc | 3 | AMF Processing |
| TLMFProc | 3 | LMF Processing |
| Signalling Propagation Delays between Nodes |  |  |
| TUE-gNB | 0-0.5 |  |
| TgNB-AMF | 3-10 |  |
| TAMF-LMF | 1-10 |  |
| TAMF-GMLC | 3-10 |  |
| Positioning Measurement Latencies |  |  |
| TLMF-Calc | 2-30 | Position Calculation latency |
| TDL-Meas | (88.5ms) | PHY DL-PRS measurement time; best possible case |
| TUL-Meas | (12ms) | PHY UL-PRS measurement time; assume the same value as for DL-PRS. |

RAN1 also identify the major components for affecting physical layer latency in the following agreement, it is observed that DL PRS alignment and MG alignment time (which is the time for waiting for the PRS or MG periodicity) are the main factors for the physical layer latency. So, A-PRS is considered for reducing the alignment time. Furthermore, network efficiency and power consumption also are the targets of R17, it is logical that on-demand PRS, A-PRS is beneficial for network efficiency and power consumption based on the observation of RAN1. From the RAN2 perspective, we also think it is reasonable to support on-demand PRS, A-PRS and semi-persistent PRS for latency, network efficiency and power consumption.

Agreement: - The following list provides the major physical layer latency components for Rel.16 DL TDOA/DL-AOD UE-assisted NR Positioning - DL PRS alignment, transmission, measurement (including processing time) and report delay - Measurement gap request, configuration and alignment time - UE/gNB higher layer (LPP/RRC) processing times

Agreement: Capture the following observations in the TR (Editorial modifications and updates to references to be made when capturing in the TR): - The results for the PRS resource utilization were analyzed by 3 sources (Huawei/HiSi, vivo, CATT) out of 17 sources - In one source (Huawei), the PRS resource utilization was evaluated for the case of 160 ms DL PRS periodicity, 30 kHz subcarrier spacing, and 12, 4, and 1 symbol per PRS resource: - PRS with 12, 4, and 1 symbol has positioning resource utilization of 2.14 %, 0.714 %, and 0.179 %, respectively - In one source (vivo), the PRS resource utilization was evaluated: - In FR1, for 20 ms DL PRS periodicity and MG periodicity, 3ms MGL, 30 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 1 beams per site, PRS resource utilization is 3.21% while the MGL/MGRP (UE overhead) is 15%. - In FR2, for 20 ms DL PRS periodicity, 20ms for MGL and MGRP, 120 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 64 beams per site, PRS resource utilization is 51.42% while the MGL/MGRP (UE overhead) is 100% - It was observed by the source that the network and device efficiency can be improved by on-demand PRS (assuming the same latency) compared to periodic PRS - In one source (CATT), the PRS resource utilization was evaluated for the case of 20 ms DL PRS periodicity, 30 kHz subcarrier spacing, and 12 symbols per PRS resource: - PRS with 12 symbols has positioning resource utilization of 2.1 %.

Observation 1: The latency target can not be achieved by R16 DL positioning.

Observation 2: A-PRS is beneficial for reducing the alignment time, network efficiency and power consumption.

Proposal 1: Follow RAN1 agreement, on-demand PRS, A-PRS and semi-persistent PRS will be studied further or specified in the WID.

  - A-periodic/semi-persistent PRS

In this subsection, we discuss the spec work from the RAN2 perspective if A-PRS/semi-persistent PRS is supported.

For A-PRS, from the RAN2 perspective, only some new configuration messages and procedures are needed to be introduced as the DCI field is the RAN1 work. Besides, additional MAC CE also is introduced for semi-persistent PRS.

Although some spec works need to be done, it is not a brand new work as Aperiodic CSI-RS and Aperiodic SRS have been supported. The procedure and message of CSI-RS or SRS can be a reference for semi-persistent/ aperiodic DL PRS.

For instance. If all the TRPs belong to the serving cells (ie, in the IIoT scenario), the procedure and message of CSI-RS can be used as a reference for semi-persistent/ aperiodic DL PRS. Otherwise, the procedure and message can be the same as SRS( A exemplary procedure is shown in Figure 1 ) and carry the activation time for aligning the active timing of the serving cell and neighbor cell.

Figure 1 the exemplary procedure for DL A-PRS

Observation 3: It is feasible for supporting semi-persistent/aperiodic DL PRS as the aperiodic/semi-persistent CSI-RS or SRS have been supported.

  - On-demand PRS with A-periodic/semi-persistent reception

Agreement: Capture the following in the TR: From a physical layer perspective, on-demand transmission and reception of DL PRS, which includes at least the following is recommended - UE-initiated request of on-demand DL PRS transmission - LMF (network)-initiated request of on-demand DL PRS transmission - Above enhancements are recommended for both DL and DL+UL positioning methods and both UE-based and UE-assisted positioning solutions.

Agreements on on-demand PRS RAN2 study on-demand PRS mechanism for DL-based, UL&DL-based methods (e.g. multi-RTT), and UE-Based and UE-assisted positioning methods in this SI

Based on the above agreement, RAN1 has reached an agreement for specifying on-demand PRS in R17 and RAN2 also has reached an agreement to study on-demand PRS. It is obvious that we will support on-demand PRS in R17. So, at least, some new procedures and messages are needed for specificing on-demand PRS. And it is noted that the on-demand PRS is about UE or LMF request/suggesting/recommending specific PRS pattern, ON/OFF, periodicity, BW, etc in an agreement in RAN1 102e meeting. If combining the on-demand PRS and aperiodic/semi-persistent reception, the additional benefits can be gotten with a minor change of specification. Especially, aperiodic/semi-persistent reception also can reuse some procedure of on-demand PRS. Take the example as shown in Figure 2, Request and Ack procedure also can be reused for aperiodic/semi-persistent reception. Besides, The combination also is beneficial for reducing latency, network efficiency and power consumption. For example, the alignment time can be reduced by aperiodic reception, the network efficiency can be improved by on-demand PRS and power consumption can be reduced if the UE can be deactivated on time.

Otherwise, if on-demand PRS only supports periodic configuration, the periodicity and slot offset will determine when to enable on-demand DL PRS. And some specific configurations and activations can not be used flexibly and timely. So, it is reasonable to combine semi-persistent/aperiodic reception and on-demand PRS.

Figure 2: An example of LMF-initiated request for on demand PRS

Observation 4: It is beneficial to combine semi-persistent/aperiodic reception and on-demand PRS for latency, network efficiency and power consumption.

- Conclusion

Based on the discussion, we discuss potential positioning enhancements with the following observations and proposals:

Observation 1: The latency target can not be achieved by R16 DL positioning.

Observation 2: A-PRS is beneficial for reducing the alignment time, network efficiency and power consumption.

Observation 3: It is feasible for supporting semi-persistent/ aperiodic DL PRS as the aperiodic/semi-persistent CSI-RS or SRS have been supported.

Observation 4: It is beneficial to combine semi-persistent/aperiodic reception and on-demand PRS for latency, network efficiency and power consumption.

Proposal 1: Follow RAN1 agreement, on-demand PRS, A-PRS and semi-persistent PRS will be studied further or specified in the WID.

- Reference

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 102, e-Meeting, August 17th – 28th, 2020.

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 103, e-Meeting, October 26th – November 13th, 2020.

- TR 38.875, “Study on NR Positioning Enhancements”

- 3GPP RP-193237, New SID on NR Positioning Enhancements, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019

- [Post112-e][617][POS] Evaluation of latency enhancement solutions (CATT)_Summary