3GPP TSG-RAN WG2 Meeting #113-e		R2-2100685

Online, Jan 25th – Feb 5th 2021

Source:	vivo

Title:          Discussion on latency enhancement for R17 positioning

Agenda Item:	8.11.2.1

Document for:	Discussion and Decision

- Introduction

The agreement on latency in potential positioning enhancements after RAN1 #103-e meeting [1] is below:

Agreement 1: Capture the following in the TR: - The enhancements of signaling & procedures for reducing NR positioning latency are recommended for normative work, including DL and DL+UL positioning methods - The details of the solutions are left for further discussion in normative work, which may include the following aspects: - Latency reduction related to the measurement gap - Latency reduction related to the reporting and request of the measurements (e.g., via RRC signaling, MAC-CE and/or physical layer procedure, and/or priority rules) - Latency reduction related to measurement time - The following enhancements of signaling & procedures for reducing NR positioning latency can be studied and specified, if needed - Latency reduction related to the request and response of positioning assistance data (e.g., via RRC signaling, MAC-CE and/or physical layer procedure) - Latency reduction related to the reception of DL PRS (e.g., priority rules for the reception of DL PRS) - No assumptions are made on whether the LCS architecture specified in TS 23.273 is enhanced or not.

In this contribution, we will discuss what can be studied and specified for latency enhancement from RAN2 and how to align with the agreement from RAN1 from RAN2’s perspective for latency.

- Discussion

At RAN#86 meeting, the study item on NR Positioning Enhancements was approved [2]. the SI includes the enhancement for the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.)

And the R17 requirement has been defined in RAN1 as below.

Agreement 2: - In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for 90% of UEs - Vertical position accuracy (< 3 m) for 90% of UEs - End-to-end latency for position estimation of UE (< 100 ms) - Physical layer latency for position estimation of UE (< 10 ms) - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< 0.2 m) for 90% of UEs - Vertical position accuracy (< 1 m) for 90% of UEs - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired) - Physical layer latency for position estimation of UE (<10ms)

According to the agreement, 100ms, even 10ms is the R17 target for End-to-end latency.

For physical layer latency, the conclusion has been achieved in RAN1 as following, L1 latency for DL is 88.5ms and L1 latency for UL is 12ms. it is obvious that L1 latency can not achieve without enhancement. So, how to achieve the R17 latency target especially for DL positioning is a critical issue in R17 from RAN1 perspective.

Conclusion: Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings - Note: The following assumptions are made - One DL PRS frequency layer in FR1 - CSSF = 1 - NRxBeam, i = 1, - Nsample = 4 (DL PRS RSTD measurements are done across 4 DL PRS periods) - Both DL PRS periodicity and MGRP are equal to 20ms - Configured DL PRS resources are within UE DL PRS processing capacity (N,T) = (0.5ms, 8ms) Conclusion: - SRS for positioning measurement time of 12 ms can be achieved under certain SRS for positioning configuration settings depending on the frame configuration - Note: The following assumptions are made - SRS for positioning alignement time 0.5 ms - SRS for positioning transmission time 0.5ms - SRS for positioning processing time 5 ms - 30 kHz SCS in FR1 - Single SRS resource set with single SRS resource - Four SRS instances - Note: Considering UL link budget and interference on SRS for positioning signals, the longer transmission time may be needed that will further increase SRS for positioning measurement time

For RAN2, The latency components of End to end latency are summarized in R2-2010872[3] and shown in Table 1.

Table 1: 	Latency Components

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TUEProc-RRCReconf | 10 | RRC Reconfiguration processing |
| TUEProc-RRCDLInfo | 5 | RRC DL information transfer |
| TUEProc-RRCULInfo | 2-5 | RRC UL information transfer |
| TUEProc-RRCLocationMeas | 2-5 | RRC Location Measurement Indication |
| TUEProc-LPPCapab | 10-20 | LPP Provide Capabilities |
| TUEProc-LPPAssi | 10 | LPP Provide Assistance Data |
| TUEProc-LPPLocationRe | 5 | LPP Request/Provide Location Information |
| TUEProc-MAC-SRSAct | 1-3 | MAC-CE SRS Activation/Deactivation |
| TgNBProc-RRC | 3 | RRC Processing |
| TgNBProc-NRPPa | 3 | NRPPa Processing |
| TgNBProc-NAS/LPP | 3 | NAS/LPP Processing |
| TAMFProc | 3 | AMF Processing |
| TLMFProc | 3 | LMF Processing |
| Signalling Propagation Delays between Nodes |  |  |
| TUE-gNB | 0-0.5 |  |
| TgNB-AMF | 3-10 |  |
| TAMF-LMF | 1-10 |  |
| TAMF-GMLC | 3-10 |  |
| Positioning Measurement Latencies |  |  |
| TLMF-Calc | 2-30 | Position Calculation latency |
| TDL-Meas | (88.5ms) | PHY DL-PRS measurement time; best possible case |
| TUL-Meas | (12ms) | PHY UL-PRS measurement time; assume the same value as for DL-PRS. |

Combined the Table1 and the procedure of DL-TDOA/UL-AoA/Multi-RTT/E-CID, the End-to end latency of NR positioning is provided inTable 2.

Table 2: 	End-to end latency for NR positioning

| Positioning technique |  |  |  |
| --- | --- | --- | --- |
| Methods | Value Range (ms) | [100]ms are met No. performance gaps (ms) | [10]ms are met No. performance gaps (ms) |
| DL-TDOA/DL-AoD | 222.5-353 | 253 | 343 |
| UL-TDOA/UL-AoA | 149-322 | 222 | 322 |
| Multi-RTT | 300.5-498 | 398 | 488 |
| Downlink NR E-CID | 176.5-286.5 | 186.5 | 276.5 |
| Uplink NR E-CID | 59-125.5 | 25.5 | 115.5 |

It is noted that some messages （such as transfer UE capability, assistance data）are not needed for every measurement. For example, for every period measurement, maybe only DL/UL measurement time and reporting time needs to be considered for every measurement. So, the latency for DL/UL measurement time and reporting time with different techniques are provided in Table 3.

Table 3: The latency for DL/UL measurement and reporting for NR positioning

| Positioning technique |  |  |
| --- | --- | --- |
| Methods |  | Value Range (ms) |
| DL-TDOA/DL-AoD | TDL-Meas+ TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPLocationRe+ TLMF-Calc | 110.5-158 |
| UL-TDOA/UL-AoA | TUL-measc+ TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa+ TLMF-Calc | 27-71 |
| Multi-RTT | TDL-measc +TUL-measc+ TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPLocationRe+TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa+ TLMF-Calc | 135.5-199 |
| Downlink NR E-CID | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPLocationRe +TLMF-Calc | 22-69.5 |
| Uplink NR E-CID | TUEProc-RRCULInfo+ TUE-gNB+ TgNBProc-RRC + TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa +TLMF-Calc | 20-67.5 |

Compared with the requirement and above analysis, the requirement cannot be met by any R16 methods from RAN2 perspective, Even only consider the measurement and reporting latency, there are also some methods (eg. DL-TDOA/DL-AoD/ Multi-RTT) that cannot meet the requirement. So, for RAN2, the enhancement for reducing the latency of positioning is also a critical issue in R17.

Observation 1: The latency target for End to end cannot be met by R16 positioning methods.

Observation 2: Even if only consider the latency of measurement and reporting, DL-TDOA/DL-AoD/ Multi-RTT cannot meet the target.

Proposal 1: For satisfying the R17 latency requirement, the enhancement for latency reduction is needed to be studied and specified in the WID.

  - The solution for latency reduction

During RAN1#103-e [1], the major components which affect the latency, and the solutions ( eg. Latency reduction related to the measurement gap, the reporting and request of the measurements) for latency reduction have been identified and shown in Agreement 1. And in email discussion [617] of RAN2, the measurement gap, the reporting and request of the measurements also are supported by the majority of companies in email discussion [617], we believe it is natural to follow RAN1 agreement and recommend continuing the work in WI

Proposal 2:	RAN2 agree latency reduction related to the measurement gap, the reporting and request of the measurements following RAN1 agreement and recommend continuing the work in WI.

In this section, we want to emphasize the enhancement with CG grant and priority indication.

2.2.1 CG for latency reduction

If the UE reports positioning measurement based on UL grant from gNB, the SR, periodicity and offset for PUCCH, frame configuration and buffer status may affect the latency. And the delay is difficult to guarantee, especially for 10ms target. So we consider using CG to report the location information.

For the two options in the email discussion [4,617], we are open to the two options if all the PRS requirements can be satisfied. For example, suitable for PRS measurement report, priority buffer the location information.

- Option 1(summarized from companies’ comments): Using the existed CG-based transmission for a certain logical channel.

- Option2: New type or separate CG for positioning which is used to adapt the PRS period and positioning specific configured grant may be introduced in Rel-17.

For the applicable status, we also agree with Qualcomm and Apple, For RRC_INACTIVE UE, the CG can be part of a solution for reporting.

For the spec impact, it is depended on the application scenarios. For us, we prefer the periodicity and slot offset CG for positioning have a relation to the PRS are configured. And the priority of CG for positioning has a relation to the requesting of positioning. And if a new type CG is introduced, may some new messages are defined and introduced.

Proposal 3: Latency reduction related to the reporting and request of the measurements can include the CG method.

2.2.2 priority indication for latency reduction

Based on the following agreement in RAN4#97-e meeting, the measurement periodicity will be extended if the PRS or SRS is drop. Although without the drop, the DL measurement time is 88.5ms and far beyond the 10ms L1target. Therefore, the enhancement with priority indication is beneficial latency reduction.

- Measurement period extension due to SSB collision - Existing RSTD measurement period is defined for cases when PRS samples are not dropped. - UE is allowed to extend the RSTD measurement period if one or more PRS samples are dropped due to SSB collision, but the exact value is not specified.

Furthermore, priority indication has been introduced for many issues, eg, URLLC, SL. It is feasible for introducing a priority indication to reduce the latency.

Proposal 4: Follow the RAN1 agreement, priority will be studied further if needed, specified in the WID.

- Conclusion

Based on the discussion, we observe and propose the following:

Observation 1: The latency target for End to end cannot be met by R16 positioning methods.

Observation 2: Even if only consider the latency of measurement and reporting, DL-TDOA/DL-AoD/ Multi-RTT cannot meet the target.

Proposal 1: For satisfying the R17 latency requirement, the enhancement for latency reduction is needed to be studied and specified in the WID.

Proposal 2:	RAN2 agree latency reduction related to the measurement gap, the reporting and request of the measurements following RAN1 agreement and recommend continuing the work in WI.

Proposal 3: Latency reduction related to the reporting and request of the measurements can include the CG method.

Proposal 4: Follow the RAN1 agreement, priority will be studied further if needed, specified in the WID.

- Reference

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 103, e-Meeting, October 26th – November 13th, 2020.

- 3GPP RP-193237, New SID on NR Positioning Enhancements, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019

- R2-2010872, Summary of latency results

- [Post112-e][617][POS] Evaluation of latency enhancement solutions (CATT)