3GPP TSG-RAN WG2 Meeting #113 e                  	R2-2100705

E-Meeting, Jan 25th – Feb 5th, 2021

Source:	vivo

Title:	Remaining issues on RACH configuration

Agenda Item:	8.8.3

Document for:	Discussion and Decision

- Introduction

At RAN2#112-e meeting, RAN2 has made the following agreements on RAN slicing discussion:

- Agreements

10: The intentions and use cases for slice-based RACH configuration are as follows:
Intention 1: RA resource isolation. From marketing point of view, some of the industrial customers have the requirement for access resource isolation, in order to provide guaranteed RA resources for their sensitive slices.
Intention 2: Slice access prioritization. In R15/16, all slices are sharing the same RA resources and cannot be differentiated by network side. But some slices may need to be prioritized during the RA procedure.

11: The following solutions will be studied and captured in the TR 38.832:
Solution 1: Slice-specific separate RACH resources pool can be configured per slice or per slice group, in addition to the existing common RACH resources.
Solution 2: Slice-specific RACH parameters prioritization can be configured per slice or per slice group.
Neither solution may not be applicable to all possible slices.

This contribution discusses remaining issues on slice based RACH configuration.

- Discussion

To allow UE direct and fast access to its intended slice, there could be some correlation between supported slice and RACH resources, for example to access slice 1 network reserves RACH resource 1 and for slice 2 network reserves RACH resource 2. So, when accessing slice 1, UE uses RACH resource 1. Thus, the correlation can be RACH resources per slice.

RACH configuration per slice would also allow to enforce UE access control, for UE cannot uses RACH resource 2 to access slice 1, alternatively, if network does not allocate RACH resource 1, UE cannot access to slice 1.

- RACH configuration per slice is a way to enforce UE access control.

For slice-based RACH configuration the following two solutions were identified:

- Solution 1: Slice-specific separate RACH resources pool can be configured per slice or per slice group, in addition to the existing common RACH resources.

- Solution 2: Slice-specific RACH parameters prioritization can be configured per slice or per slice group.

These two solutions were identified to address the following two intentions:

- Intention 1: RA resource isolation. From marketing point of view, some of the industrial customers have the requirement for access resource isolation, in order to provide guaranteed RA resources for their sensitive slices.

- Intention 2: Slice access prioritization. In R15/16, all slices are sharing the same RA resources and cannot be differentiated by network side. But some slices may need to be prioritized during the RA procedure.

Solution 1 meets both above intention 1 and intention 2. But, solution 2 can just cover intention 2. Solution 1 has the advantage of reducing RACH collision and delay for some slice deployment requirement. But solution 1 may induce RACH fragmentation.

Solution 2 can be useful in case slices are of different priority. But, configuration of solution 2 may be complex.

It is worth noting that none of these solutions can cover all potential slice deployment scenarios.

- Neither solution 1 nor Solution 2 can cover all potential slice scenarios

Thus, to allow slice-based RACH configuration can cover most slice scenarios, it is proposed:

- Both solution 1 and solution 2 are considered for normative work

- Conclusion

This paper discussed some remaining issues on RAN slices. The paper concludes with:

Observation 1:	RACH configuration per slice is a way to enforce UE access control.

Observation 2:	Neither solution 1 nor Solution 2 can cover all potential slice scenarios

Proposal 1：	Both solution 1 and solution 2 are considered for normative work