3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2100725

Online, Jan 25 – Feb 5, 2021

Agenda Item:	8.3.3

Source:	Charter Communications

Title:	Network Switching for Multi-SIM UEs

Document for:	Discussion, Decision

# Introduction

Multi-SIM WID [1] identifies the following objective:

- Mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose): - RAT Concurrency: Network A is NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx

During RAN2#112e [2], the following agreements regarding above objective were reached:

- RAN2 will continue to discuss RRC-based switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED. There may be different mechanisms (short/long, leaving/returning, etc.).

- 1a: The sub-Case 3-1 is supported in WI, i.e., the switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED includes the case where Dual-RX/Single-TX UE is in RRC_CONNECTED state in NW A while performing only reception in NW B (i.e., in RRC_idle State and RRC inactive state).

- 1b: For Sub-Case 3-1, whether the Rx capability coordination between UE and NW is needed can be decided after the RRC-based switching/leaving and returning procedure is defined.

- 2: The Sub-Case 3-2, i.e., Dual-RX/Single-TX UE stays in RRC_CONNECTED mode in NW A while performing reception and transmission in NW B (in RRC_ CONNECTED or during RRC setup/resume period), is not considered in the WI from RAN2 viewpoint. Scheduling gap is not excluded.

Given above agreements, this contribution evaluates potential solutions for short- and long-time switching.

# Network switching

This contribution considers the scenario where a multi-SIM UE is in RRC connected state in one network and needs to performs tasks in another network. The tasks to be performed could be idle-mode monitoring tasks such as monitoring paging occasions, or performing short signalling for TAU and RNAU, or a longer leave e.g for initiating a PDU session. An intentional leave by the UE without notifying the first network could cause waste of the resources of the first network, and depending on what procedures the first network performs after the UE absence, such leave could affect network statistics negatively.

The second objective in the WID [1] seeks mechanisms for UE to notify a network of its switch from the network. Such mechanism enables a UE to leave a network in coordination of the network, where the leave may be a short or long leave, periodic or aperiodic.

To address a fitting solution for short- vs long-leave, it is best to distinguish each leave/switch. We suggest to formalize the definition short- vs long-switch as follows:

- Short-time switching: where the UE switches either RX-only for a short duration (e.g., for paging and cell measurement on network B), or switches TX and/or RX for a short duration (e.g., for TAU/RNAU or to send busy indication on network B). The upper and lower bounds of the switch are FFS. Core network may not have visibility into switching at RAN-level.

- Long-time switching: where the UE switches TX and/or RX for a long duration (e.g., to initiate a PDU session on network B). Core network is aware that UE performs long-time switching.

The authors are of the opinion that the UE is in the best position to determine what classifies as short vs long duration, and which task falls into which category. The network supporting MUSIM feature ought to accommodate specific behaviors for both cases.

## Short-time switching

Short duration leaves may be helpful for various tasks that a UE is expected to perform. For instance, monitoring paging or performing measurements on a second network are among the tasks that a UE is expected to perform periodically, for which a short leave from the first network may suffice. On the other hand, tasks such as, Mobility Management (MM) or Session Management (SM) signaling, TAU, RNAU etc. also likely would be concluded within a short interval, although it takes longer than paging monitoring. For such tasks, a leave with longer duration (than short duration) from the first network may be required.

To address these cases, a UE may notify the first network of the leave by seeking a scheduling gap from the network. Such scheduling gap has similarity with measurement gap that currently is specified. During such gap, the gNB is aware of the unavailability of the multi-SIM UE and avoids any scheduling for the UE. The UE should be in the position to request the attributes of the scheduling gap.

Proposal 1: RAN2 should consider using existing procedures, such as measurement gaps, to address short-time switching. FFS possibility to enhance the gap length, periodicity and offset.

To address various tasks for short-time switching, a UE may be configured with multiple measurement gaps with various attributes. Each measurement gap may be activated/activated via MAC CE.

Proposal 2: To address various tasks for short-time switching, a UE may be configured with multiple measurement gaps with various attributes. Each measurement gap may be activated/activated via MAC CE.

Other applications of short-time switch are mobility management such as TAU and RNA or SM signalling. Such signalling is less attributed with a precise periodicity and would take longer than a paging monitoring. Also, as defined above, during such signalling both RX and TX of a multi-SIM UE tunes away from the first network. In a recent email discussion, such short-time switch is referred to as one-shot short-time switch. We suggest that similar to periodic short-time switching procedure, measurement gap may be used for one-shot/aperiodic short-time switch (where the periodicity attribute is set accordingly, e.g. infinite).

Proposal 3: Similar to periodic short-time switching procedure, measurement gap may be used for one-shot/aperiodic short-time switch (where the periodicity attribute is set accordingly, e.g. infinite).

Other existing procedures that may be used to accommodate short-time switching are UE Assistance Information RRC signaling and Overheating Assistance procedure. For instance, UE Assistance Information RRC signaling can be reused for one-shot short-time switching where a UE can inform gNB of the expected duration that tunes to the second network, along with any reduced transmitter and/or receiver capability that UEAssistanceInformation signaling supports.

For UEs with multiple RF chains to be able to perform additional transmission or reception in a second network, the UE has to make sure that the first network is not expecting all RF chains to be in service for the first network. For instance, for a UE to use one of its RF chains on a second network during a short-time switch, it’d suffice for the UE to let the first network know that some of the RF chains are not available for an upcoming interval. UE assistance information or overheating assistance framework may be used to allow gNB to temporarily free up RF resources from PHY control. Another example where UE assistant information would be helpful is where a 2-RX multi-SIM UE is in RRC connected state on both networks and the combined instantaneous downlink data from both networks may exceed the L1/L2 buffer capacity of the UE. To avoid buffer overflow, and minimizing packet discarding in the L2 buffer, the UE may manage its buffer size by scaling down its available L2 buffer and indicating so to both networks.

## Busy indication

SA2 has considered a scenario where a multi-SIM UE is in RRC connected state in a first network and may use scheduling gap in the first network to monitor paging occasions in a second network. Additionally, SA2 considers that the UE may decide to send a busy indication to the second network and then return to the first network [3].

In order to evaluate if a scheduling gap on a first network is sufficient for transmission of a busy indication on a second network, RAN2 should consider the total duration required and the expected behaviour from the UE given the paging cause on the second network.

Proposal 4: In order to evaluate if a scheduling gap on a first network is sufficient for transmission of a busy indication on a second network, RAN2 should consider the total duration required and the expected behaviour from the UE given the paging cause on the second network.

## Long-time switching

Beside short duration leaves, a UE may be required to maintain a session for a long-term activity on the second network. Without notifying the first network of the leave, the network would go through various procedures, at RAN and core sides, that eventually waste the resources. Therefore, it’s desired if the UE notifies the first network of a long-term leave before switching to the second network.

The distinction or decision between a short vs long leave may not be possible for a multi-SIM UE, hence it is possible to consider a case where a UE decides on a short leave but due to the activity on the second network the leave might take longer. Regardless of whether there is an initial short-time switch or not, a UE automatously transitioning to RRC idle state impacts the first network negatively, e.g. missing downlink and/or uplink transmissions, hence RAN2 should aim for solutions that properly transition a UE to RRC idle for a long-time switch.

Proposal 5: UE automatously transitioning to RRC idle state impacts the first network negatively, hence RAN2 should aim for solutions that properly transition a UE to RRC idle for a long-time switch.

The debate for long-time switch has been around whether RRC- or NAS-based solutions should be used. SA2 has also debated extensively on the applicability of both types of solutions [3] and it appears that SA2 has not concluded its debate on this regard. We believe that RAN2 should debate the effectiveness and applicability of RRC- vs NAS-based solutions for long-time switching.

Proposal 6: RAN2 should debate the effectiveness and applicability of RRC- vs NAS-based solutions for long-time switching.

For instance, during a long-time switch, if specific PDUs need be suspended, then RRC-only solution is not feasible, without breaking layer separation. In such cases, core-based Suspend and Resume on a per PDU session basis should be used. Therefore, we believe that for a selective suspension of PDU sessions in the first network in a long-time switch, NAS-based solution is preferred. Hence, we suggest that RAN2 to send an LS to SA2 and indicate such preference.

Proposal 7: For a selective suspension of PDU sessions in the first network in a long-time switch, NAS-based solution is preferred. Hence, we suggest that RAN2 to send an LS to SA2 and indicate such preference.

# Conclusion

Proposal 1: RAN2 should consider using existing procedures, such as measurement gaps, to address short-time switching. FFS possibility to enhance the gap length, periodicity and offset.

Proposal 2: To address various tasks for short-time switching, a UE may be configured with multiple measurement gaps with various attributes. Each measurement gap may be activated/activated via MAC CE.

Proposal 3: Similar to periodic short-time switching procedure, measurement gap may be used for one-shot/aperiodic short-time switch (where the periodicity attribute is set accordingly, e.g. infinite).

Proposal 4: In order to evaluate if a scheduling gap on a first network is sufficient for transmission of a busy indication on a second network, RAN2 should consider the total duration required and the expected behaviour from the UE given the paging cause on the second network.

Proposal 5: UE automatously transitioning to RRC idle state impacts the first network negatively, hence RAN2 should aim for solutions that properly transition a UE to RRC idle for a long-time switch.

Proposal 6: RAN2 should debate the effectiveness and applicability of RRC- vs NAS-based solutions for long-time switching.

Proposal 7: For a selective suspension of PDU sessions in the first network in a long-time switch, NAS-based solution is preferred. Hence, we suggest that RAN2 to send an LS to SA2 and indicate such preference.

# References

- RP-201309	Support for Multi-SIM devices for LTE/NR

- R2-2010701	Report from session on LTE legacy, Mobility, DCCA, Multi-SIM and RAN slicing

- TR 23.761 	Study on system enablers for devices having MUSIM