3GPP TSG-RAN WG2 #112 electronic	R2-2100750

E-Meeting, 25 Jan – 05 Feb, 2021

Source:		NEC

Agenda item:		8.3.3 UE notification on network switching for multi-SIM

Title:		UE notification procedure for short time switching

Document for:		Discussion

# 1. Introduction

During last RAN2 meeting, the coordinated switch for MUSIM UE is further divided into two scenarios [1]:

- Scenario 1: Long-time switching, such as VoLTE/VoNR voice call

- Scenario 2: Short-time switching, such as paging reception, measurements, TAU, RNAU, MO SMS

For long-time switching, the UE is required to be transmitted to IDLE/INACTIVE state to switch to the other network. While for short-time switching, the UE can remain at CONNECTED state during the switching time.

In this paper, we provide our consideration on how to support short-time switching.

# 2. Discussion

While UE remains at RRC CONNECTED state within network A, a short-time switching procedure can be used to switch to network B to perform short time activities, includes paging reception, measurements, TAU, RNAU, etc. One possible way to realize short-time switch is to base on UE implementation, i.e. by leveraging existing measurement gap or DRX off duration. However, this may not always workable. For example, when the data transmission on network A continues for a long time, there will be no DRX off time at all. Besides, the periodicity and length of measurement gap are fixed to several pattern, which may be not suitable for the service of network B. Therefore, we propose to support dedicated gap with flexible gap pattern for short-time switching for MUSIM UE.

Proposal 1. RAN2 support dedicated scheduling gap for short-time switching.

If the UE detects that using existing mechanisms (i.e. measurement gap or DRX-off time) is inadequate to cope with the shot-time service on network B, the UE can send notification to network A for scheduling gap request. As the network A is not aware of the service information on network B, UE shall provide assistance information to network A, which including the suggest gap pattern, or in other words the service pattern. The pattern information can include gap length, periodicity and offset.

Proposal 2. The UE provides suggested gap pattern to the network for scheduling gap request.

Within the scheduling gap, the UE shall apply a similar behaviour as measurement gap, i.e. do not perform any UL/DL transmission and PDCCH monitoring except for random access procedure. The service on network A stops for a short time, but the UE can remain at CONNECTED state.

During last meeting, it is agreed to support the following case for short-time switching:

Agreements

1a: The sub-Case 3-1 is supported in WI, i.e., the switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED includes the case where Dual-RX/Single-TX UE is in RRC_CONNECTED state in NW A while performing only reception in NW B (i.e., in RRC_idle State and RRC inactive state).

1b: For Sub-Case 3-1, whether the Rx capability coordination between UE and NW is needed can be decided after the RRC-based switching/leaving and returning procedure is defined.

In the case above, the Dual Rx/Single-Tx UE can be configured with both RF1 and FR2 serving cells (for example the UE is configured with DC, with MCG at FR1 and SCG at FR2) at network A, and would like to perform DL reception in network B for e.g. IDLE/INACTIVE mode measurement or paging monitoring. One possible solution in this case is to support RX capability coordination to switch to network B. However, as short-time switching is a very small period of time, there is no need to reduce UE Rx capability in network A. Instead, similar to per-FR measurement gap, in this case, to minimize traffic interruption at network A, per-FR scheduling gap can be supported. For example if the UE requests a scheduling gap to perform paging monitoring for network B, and the camped cell of network B is a FR1 cell, then one FR1 scheduling gap can be configured to the UE, thus the traffic on FR2 serving cells is not impacted.

As network A is not aware of the frequency range information for the short-time service at network B, the UE shall provide the FR related information to network A, such that network A can make decision on if per-FR scheduling gap or per-UE scheduling gap is configured.

Proposal 3. Per-FR scheduling gap should be supported for the case where Dual-RX/Single-TX UE is in RRC_CONNECTED state in NW A while performing only reception in NW B.

Proposal 4. The UE provides frequency range related information to the network for scheduling gap request.

Once one periodic scheduling gap is configured to the UE, the UE can switch to the other network based on the configuration without any further notification to the network. The UE may not need the configured periodic scheduling gap some time later, for example due to change of camped cell, the UE shall indicate to the network A to release a previous configured periodic scheduling gap, and request for a new scheduling gap with different pattern if necessary.

Proposal 5. The UE can indicate the network to release the configured periodic scheduling gap.

# 3. Conclusion

In this contribution, we provide our consideration on the support short-time switching for MUSIM case, and have the following proposals:

Proposal 1. RAN2 support dedicated scheduling gap for short-time switching.

Proposal 2. The UE provides suggested gap pattern to the network for scheduling gap request.

Proposal 3. Per-FR scheduling gap should be supported for the case where Dual-RX/Single-TX UE is in RRC_CONNECTED state in NW A while performing only reception in NW B..

Proposal 4. The UE provides frequency range related information to the network for scheduling gap request.

Proposal 5. The UE can indicate the network to release the configured periodic scheduling gap.

# Reference

[1] Meeting report of RAN2 #112e