3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2100751

Online, Jan 25 – Feb 5, 2021

Agenda Item:	5.4.1.3

Source: 	Fujitsu

Title:	The validity of a stored SIB if SI Area ID is absent

Document for:	Discussion & Decision

- Introduction

In NR, Stored SIB is introduced. By this mechanism, the UE can store an acquired SIB; the UE may use a valid stored version of the SIB if there is a valid stored version of the SIB after checking the validity of each stored version of a SIB.

In this contribution, the UE behavior in case that SI Area ID is not provided is discussed.

- Discussion

The UE behavior of checking the validity of a stored version of a SIB is specified in [1]:

The UE shall:

1>	delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid;

1>	for each stored version of a SIB:

2>	if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the UE is NPN capable and the cell is an NPN-only cell and the first NPN identity included in the NPN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

3>	else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell:

3>	if the UE is NPN capable and the cell is an NPN-only cell and the first NPN identity in the NPN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

3>	else if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB:

4>	consider the stored SIB as valid for the cell;

1>	for each stored version of a posSIB:

2>	if the areaScope is associated and its value for the stored version of the posSIB is the same as the value received in the posSIB-MappingInfo for that posSIB from the serving cell:

3>	if the systemInformationAreaID included in the si-SchedulingInfo and the valueTag (if available) [49] for the posSIB received from the serving cell are identical to the systemInformationAreaID and the valueTag (if available) associated with the stored version of that posSIB; and if the expirationTime [49] associated with the stored posSIB has not been expired:

4>	consider the stored posSIB as valid for the cell;

2>	if the areaScope is not present for the stored version of the posSIB and the areaScope value is not included in the posSIB-MappingInfo for that posSIB from the serving cell:

3>	if the cellIdentity and the valueTag (if available) [49] for the posSIB received from the serving cell are identical to the cellIdentity and the valueTag (if available) associated with the stored version of that posSIB; and if the expirationTime [49] associated with the stored posSIB has not been expired:

4>	consider the stored posSIB as valid for the cell;

According to the above description (highlighted in green), the UE will check the value of systemInformationAreaID or cellIdentity and the value of valueTag in order to determine whether a stored SIB is valid or not.

Observation 1: The UE will check the value of systemInformationAreaID or cellIdentity and the value of valueTag in order to determine whether a stored SIB is valid or not.

The presence of these parameters:

- systemInformationAreaID: The parameter is optional present in SI-SchedulingInfo

- cellIdentity: The parameter is mandatory present, and it is included in PLMN-IdentityInfo of cellAccessRelatedInfo in SIB1

- valueTag: The parameter is mandatory present in SI-SchedulingInfo for the SIB except SIB6, SIB7 or SIB8.

Observation 2: For a SIB when a UE may need to check its validity, both cellIdentity and valueTag are mandatory while systemInformationAreaID is optional.

It means that the network may not provide the systemInformationAreaID in some cases even some SIBs are associated with ‘areaScope’. For a SIB associated with ‘areaScope’ in serving cell, the UE needs to check the systemInformationAreaID to determine whether a stored version of the SIB is valid or not. In case that the systemInformationAreaID is not provided, UE behaviour is not specified in current specification.

The first problem is how to determine whether the systemInformationAreaID is identical or not if it is absent. Generally, it is not identical if the systemInformationAreaID is present either in a stored version or in SIB1. However, whether it is identical or not is not clear in case that the systemInformationAreaID is absent in a stored version and absent in SIB1. These cases are summarized in Table 1.

Table 1: whether the systemInformationAreaID is identical or not

|  | The presence of systemInformationAreaID in a stored version | The presence of systemInformationAreaID in SIB1 | Identical or not? |
| --- | --- | --- | --- |
| Case1 | Present | Absent | Not identical |
| Case2 | Absent | Present | Not identical |
| Case3 | Absent | Absent | Not clear |

Observation 3: Whether the systemInformationAreaID is identical or not is not clear in case that the systemInformationAreaID is absent in a stored version and absent in SIB1.

Regarding Case3, the following possibilities are observed:

- Possibility 1: UEs consider it as not identical

- Possibility 2: UEs consider it as identical

- Possibility 3: it is up to UE implementation so that some UEs consider it as not identical while others consider it as identical

Proposal 1: RAN2 is asked to confirm the following possibilities for the case where the systemInformationAreaID is absent in a stored version and absent in SIB1:

- Possibility 1: UEs consider it as not identical

- Possibility 2: UEs consider it as identical

- Possibility 3: it is up to UE implementation so that some UEs consider it as not identical while others consider it as identical

If above Possibility 1 is confirmed, the above description (highlighted in green) of TS38.331 needs to be updated so that the UE will check systemInformationAreaID only if it is configured and consider a stored version as valid when the configured SI area ID is identical with stored one.

A Text Proposal for Rel-16 TS 38.331[1] based on Possibility 1 is provided as below:

The UE shall: 1> delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid; 1> for each stored version of a SIB: 2> if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell: 3> if the UE is NPN capable and the cell is an NPN-only cell and the first NPN identity included in the NPN-IdentityInfoList, the systemInformationAreaID, if present, and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell; 3> else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID, if present, and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell; 2> if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell: 3> if the UE is NPN capable and the cell is an NPN-only cell and the first NPN identity in the NPN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the cellIdentity and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell; 3> else if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell; 1> for each stored version of a posSIB: 2> if the areaScope is associated and its value for the stored version of the posSIB is the same as the value received in the posSIB-MappingInfo for that posSIB from the serving cell: 3> if the systemInformationAreaID, if present, included in the si-SchedulingInfo and the valueTag (if available) [49] for the posSIB received from the serving cell are identical to the systemInformationAreaID and the valueTag (if available) associated with the stored version of that posSIB; and if the expirationTime [49] associated with the stored posSIB has not been expired: 4> consider the stored posSIB as valid for the cell; 2> if the areaScope is not present for the stored version of the posSIB and the areaScope value is not included in the posSIB-MappingInfo for that posSIB from the serving cell: 3> if the cellIdentity and the valueTag (if available) [49] for the posSIB received from the serving cell are identical to the cellIdentity and the valueTag (if available) associated with the stored version of that posSIB; and if the expirationTime [49] associated with the stored posSIB has not been expired: 4> consider the stored posSIB as valid for the cell;

Proposal 2: Apply the proved TP to Rel-16 TS 38.331[1] if Possibility 1 in Proposal 1 is confirmed.

A Text Proposal for Rel-15 TS 38.331[2] based on Possibility 1 is provided as below:

The UE shall: 1> delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid; 1> for each stored version of a SIB: 2> if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell: 3> if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID, if present, and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell; 2> if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell: 3> if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell;

Proposal 2bis: Apply the proved TP to Rel-15 TS 38.331[2] if Possibility 1 in Proposal 1 is confirmed.

If above Possibility 2 or 3 is confirmed, i.e. UEs may consider a stored version of a SIB without SI area ID as valid when no SI area ID is included in SIB1 from current serving cell. If the SIB is not changed, e.g. the serving cell is same, using the stored version of the SIB has no problem. However, if the SIB is changed, UEs may not acquire the SIB from the current serving cell and may use the stored version of the SIB which is wrong.

Observation 4: Interoperability problem is observed in Possibility 2 or 3 because UEs may not acquire a SIB and may use the stored version of the SIB which is different from the one from serving cell.

There are 2 options to solve the interoperability problem when Possibility 2 or 3 is confirmed:

- Option 1): areaScope is absent if systemInformationAreaID is not configured

In this option, areaScope may be present only if systemInformationAreaID is configured so that the UE can check SI area ID to determine the validity of a stored SIB in case that areaScope is associated. If systemInformationAreaID is not configured

However, this option restricts flexibility of the configuration. Backward compatibility issue at network side is observed.

- Option 2): check systemInformationAreaID if it is configured and check cellIdentity if systemInformationAreaID is not configured

In this option, the UE behavior is clear since the UE will check systemInformationAreaID only if it is configured. In case that systemInformationAreaID is not configured, the UE will check cellIdentity instead.

This option has minor benefits in the case that systemInformationAreaID is not included in SIB1 from a cell at this moment while it is included in SIB1 from the same cell when a SIB is acquired and stored by the UE. In this case, the UE considers the stored version as valid and does not need to re-acquire the SIB.

However, backward compatibility issue at UE side is observed.

Proposal 3: RAN2 is asked to select a solution if Possibility 2 or 3 in Proposal 1 is confirmed:

- Option 1): areaScope is absent if systemInformationAreaID is not configured

- Option 2): check systemInformationAreaID if it is configured and check cellIdentity if systemInformationAreaID is not configured

- Summary

In this contribution, we discuss the UE behavior of checking the validity of a stored SIB in case that SI Area ID is not provided. The following observations and proposals are made:

Observation 1: The UE will check the value of systemInformationAreaID or cellIdentity and the value of valueTag in order to determine whether a stored SIB is valid or not.

Observation 2: For a SIB may need to check its validity, both cellIdentity and valueTag are mandatory while systemInformationAreaID is optional.

Observation 3: Whether the systemInformationAreaID is identical or not is not clear in case that the systemInformationAreaID is absent in a stored version and absent in SIB1.

Proposal 1: RAN2 is asked to confirm the following possibilities for the case where the systemInformationAreaID is absent in a stored version and absent in SIB1:

- Possibility 1: UEs consider it as not identical

- Possibility 2: UEs consider it as identical

- Possibility 3: it is up to UE implementation so that some UEs consider it as not identical while others consider it as identical

Proposal 2: Apply the proved TP to Rel-16 TS 38.331[1] if Possibility 1 in Proposal 1 is confirmed.

Proposal 2bis: Apply the proved TP to Rel-15 TS 38.331[2] if Possibility 1 in Proposal 1 is confirmed.

Observation 4: Interoperability problem is observed in Possibility 2 or 3 because UEs may not acquire a SIB and may use the stored version of the SIB which is different from the one from serving cell.

Proposal 3: RAN2 is asked to select a solution if Possibility 2 or 3 in Proposal 1 is confirmed:

- Option 1): areaScope is absent if systemInformationAreaID is not configured

- Option 2): check systemInformationAreaID if it is configured and check cellIdentity if systemInformationAreaID is not configured

# References

- 3GPP TS 38.331 V16.3.1 (2020-12)

- 3GPP TS 38.331 V15.12.0 (2020-12)