3GPP TSG-RAN WG2 Meeting #113-e	R2-210xxxx

# Online, January 25th – February 5th 2021

Agenda Item: 	8.17

Source: 			Huawei, HiSilicon

Title: 	Summary of [AT113-e][034][NR17 Other] NR17 other (Huawei)

Document for: 	Discussion and decision

# Introduction

This document summarizes the following offline discussion.

[AT113-e][034][NR17 Other] NR17 other (Huawei)

Scope: Treat R2-2100054, R2-2100896, R2-2100897, R2-2100950, R2-2100951, T2-2100952, R2-2100953, R2-21002259, R2-21001457, R2-21001458, R2-2100046, R2-2101415, R2-2100055, R2-21001612, R2-21001613

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs and LS out if applicable.

Intended outcome: Report, Agreed CRs, approved LS  if any is agreeable.

Deadline: Prepare such that results can be available Feb 3 (for potential CB Feb 4).

# Contact from companies

| Company | Email |
| --- | --- |
| Huawei, HiSilicon | Yiru Kuang (kuangyiru@huawei.com) |
| T-Mobile USA | John.Humbert2@T-Mobile.com |

# Discussion

## FR2 FWA power class

FR2 FWA - Power Class Release Indep R15

R2-2100054	LS for FR2 FWA power class (R4-2016876; contact: Softbank)	RAN4	LS in	Rel-17	NR_FR2_FWA_Bn257_Bn258	To:RAN2

R2-2100896	Introducing UE capability for power class 5 for FR2 FWA	SoftBank, Huawei	draftCR	Rel-17	38.331	16.3.1	C	NR_FR2_FWA_Bn257_Bn258-Core

R2-2100897	Introducing UE capability for power class 5 for FR2 FWA	SoftBank, Huawei	draftCR	Rel-17	38.306	16.3.0	C	NR_FR2_FWA_Bn257_Bn258-Core

R2-2100950	Introduction of PC5 for FR2	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.331	15.12.0	2368	-	B	NR_FR2_FWA_Bn257_Bn258-Core

R2-2100951	Introduction of PC5 for FR2	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.3.0	2369	-	A	NR_FR2_FWA_Bn257_Bn258-Core

R2-2100952	Introduction of PC5 for FR2	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.306	15.12.0	0495	-	B	NR_FR2_FWA_Bn257_Bn258-Core

R2-2100953	Introduction of PC5 for FR2	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.3.0	0496	-	A	NR_FR2_FWA_Bn257_Bn258-Core

There are two options for introducing power class 5:

- The power class 5 is introduced from Rel-17 with “Early implementation of this CR by Rel-15/16 UEs does not cause any inter-operability issues” in the cover sheet. (CRs R2-2100896/R2-2100897)

- The power class 5 is introduced from Rel-15. (CRs R2-2100950~R2-2100953)

Q1-1 Which option listed in above do companies support?

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | (1) | A new power class 1.5 was introduced in Rel-16, it was added as an Rel-16 signalling with stating “implementation of the change from Rel-15” in the cover sheet of the CR. For this new power class 5, we understand the same operation can be applied, and it will be clearer. |

Q1-2 If the option (1) is selected, do companies agree the CRs R2-2100896/R2-2100897? Please companies provide your comments on the CRs if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes, but | We agree with the signalling change in 38.331, however we have some concerns on the inter-operability issue, if the new UE reports the new field PC5 but the legacy cannot understand it, which power class should be applied? |

Q1-3 If the option (2) is selected, do companies agree the CRs R2-2100950~R2-2100953? Please companies provide your comments on the CRs if any.

| Company | Yes/No | Comments |
| --- | --- | --- |

## 35 and 45 MHz channel Bandwidths

FR1_35MHz_45MHz_BW - Release Indep R15

All Moved from 5.4.3:

R2-2102259	LS to RAN2 on 35 and 45 MHz channel Bandwidths (R4-2017846; contact: T-Mobile)	RAN4	LS in	Rel-15	NR_FR1_35MHz_45MHz_BW-Core	To:RAN2

R2-2101457	Support of 35 MHz and 45 MHz channel bandwidth for FR1	Apple Inc, T-Mobile	CR	Rel-15	38.306	15.12.0	0511	-	F	NR_FR1_35MHz_45MHz_BW-Core

R2-2101458	Support of 35 MHz and 45 MHz channel bandwidth for FR1	Apple Inc, T-Mobile	CR	Rel-16	38.306	16.3.0	0512	-	A	NR_FR1_35MHz_45MHz_BW-Core

The changes in the CRs are given as below:

channelBWs-DL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For FR1, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingDL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability for and validate instead the channelBW-90mhz and the supportedBandwidthCombinationSet. For serving cells with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthDL. For each of the channel bandwidths indicated in channelBWs-DL-v1590, for the network to use the relevant FeatureSetDownlinkPerCC, the UE shall include at least one FeatureSetDownlinkPerCC with supportedBandwidthDL where the supported bandwidth value is greater than the channel bandwidth indicated in channelBWs-DL-v1590.

channelBWs-UL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-UL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For FR1, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For FR1, the leading/leftmost bit in channelBWs-UL-v1590 indicates 70 MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz and all the remaining bits in channelBWs-UL-v1590 shall be set to 0. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingUL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz the network may ignore this capability for and validate instead the channelBW-90mhz and the supportedBandwidthCombinationSet. For serving cells with other channel bandwidths the network validates the channelBWs-UL, the supportedBandwidthCombinationSet, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthUL. . For each of the channel bandwidths indicated in channelBWs-UL-v1590, for the network to use the relevant FeatureSetUplinkPerCC, the UE shall include at least one FeatureSetUplinkPerCC with supportedBandwidthUL where the supported bandwidth value is greater than the channel bandwidth indicated in channelBWs-UL-v1590.

Q2-1 Do companies agree the CRs R2-2101457/R2-2101458? Please companies provide your comments on the CRs if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes, but | We are fine with the first change, but not sure about the second change for NOTE, why we need such restrict for per CC BW and per band BW? |
| T-Mobile USA | Yes | The clarification on max supported BW is needed, this clarifies the case when the maximum channel BW supported by a band is increased. |

## UL MIMO restrictions for SUL

FR1 enh - UL MIMO restrictions for SUL

R2-2100055	LS on removing restriction on configuring UL MIMO for SUL band (R4-2016909; contact: CMCC)	RAN4	LS in	Rel-17	NR_RF_FR1_enh-Core	To:RAN2	Cc:RAN1

R2-2101612	Draft CR: Remove the maximum number of MIMO layers configuration restrictions for SUL	CMCC, Huawei, HiSilicon, CATT	draftCR	Rel-17	38.331	16.3.1	B	NR_RF_FR1_enh

R2-2101613	Draft CR: Remove the maximum number of MIMO layers restrictions for SUL	CMCC, Huawei, HiSilicon, CATT	draftCR	Rel-17	38.306	16.3.0	B	NR_RF_FR1_enh

The changes in the 38.331 CRs are given as below:

maxMIMO-Layers Indicates the maximum MIMO layer to be used for PUSCH in all BWPs of the normal UL of this serving cell (see TS 38.212 [17], clause 5.4.2.1). If present, the network sets maxRank to the same value. The field maxMIMO-Layers refers to DCI format 0_1.

maxMIMO-LayersForDCI-Format0-2 Indicates the maximum MIMO layer to be used for PUSCH for DCI format 0_2 in all BWPs of the normal UL of this serving cell (see TS 38.212 [17], clause 5.4.2.1). If present, the network sets maxRankForDCI-Format0-2 to the same value.

The changes in the 38.306 CRs are given as below:

| maxNumberMIMO-LayersCB-PUSCH Defines supported maximum number of MIMO layers at the UE for PUSCH transmission with codebook precoding. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset. | FSPC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| maxNumberMIMO-LayersNonCB-PUSCH Defines supported maximum number of MIMO layers at the UE for PUSCH transmission using non-codebook precoding. UE supporting non-codebook based PUSCH transmission shall indicate support of maxNumberMIMO-LayersNonCB-PUSCH, maxNumberSRS-ResourcePerSet and maxNumberSimultaneousSRS-ResourceTx together. | FSPC | No | N/A | N/A |
| maxNumberSimultaneousSRS-ResourceTx Defines the maximum number of simultaneous transmitted SRS resources at one symbol for non-codebook based transmission to the UE. | FSPC | No | N/A | N/A |
| maxNumberSRS-ResourcePerSet Defines the maximum number of SRS resources per SRS resource set configured for codebook or non-codebook based transmission to the UE. | FSPC | No | N/A | N/A |

Q3-1 Do companies agree the CRs R2-2101612/R2-2101613? Please companies provide your comments on the CRs if any.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

## Broadcast of gNB ID length

R3 TEI17 - Broadcast of gNB ID length

R2-2100046	LS on broadcasting gNB ID length in system information block (R3-207226; contact: Ericsson)	RAN3	LS in	Rel-17	TEI17	To:RAN2	Cc:SA3

R2-2101415	On broadcasting gNB ID length in SIB1 (reply LS to R3-207226)	Ericsson	discussion

According to the LS in R2-2100046, RAN2 is supposed to answer RAN3, which asked:

RAN3 WG respectfully asks RAN2 WG to check the feasibility of broadcasting the gNB ID’s length in system information blocks and related UE behaviour including reporting for ANR purposes.

Based on the above and the proposals in R2-2101415 (copied below), the moderator ask companies to reply the questions below.

Observation 1 The current TNL address discovery is not well prepared for the RAN node ID’s flexible length. Proposal 1 Include gNB ID length in PLMN-IdentityInfo IE in SIB1 for each cell that is served by that gNB. Proposal 2 Include gNB ID length in reportCGI measurement report.

Q4-1 Do companies agree that Proposal 1 is feasible from RAN2 point of view? Which pros and cons do you see in the proposal?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | This would be too costly to broadcast. It will not work for legacy UEs. |

Q4-2 Do companies agree that Proposal 2 is feasible from RAN2 point of view? Which pros and cons do you see in the proposal?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | It will not work for legacy UEs. |

# 4	Conclusions

- To be added…

# 5	References

R2-2100054	LS for FR2 FWA power class (R4-2016876; contact: Softbank)	RAN4

R2-2100896	Introducing UE capability for power class 5 for FR2 FWA	SoftBank, Huawei

R2-2100897	Introducing UE capability for power class 5 for FR2 FWA	SoftBank, Huawei

R2-2100950	Introduction of PC5 for FR2	Nokia, Nokia Shanghai Bell

R2-2100951	Introduction of PC5 for FR2	Nokia, Nokia Shanghai Bell

R2-2100952	Introduction of PC5 for FR2	Nokia, Nokia Shanghai Bell

R2-2100953	Introduction of PC5 for FR2	Nokia, Nokia Shanghai Bell

FR1_35MHz_45MHz_BW - Release Indep R15

R2-2102259	LS to RAN2 on 35 and 45 MHz channel Bandwidths (R4-2017846; contact: T-Mobile)	RAN4

R2-2101457	Support of 35 MHz and 45 MHz channel bandwidth for FR1	Apple Inc, T-Mobile

R2-2101458	Support of 35 MHz and 45 MHz channel bandwidth for FR1	Apple Inc, T-Mobile

R2-2100055	LS on removing restriction on configuring UL MIMO for SUL band (R4-2016909; contact: CMCC)	RAN4

R2-2101612	Draft CR: Remove the maximum number of MIMO layers configuration restrictions for SUL	CMCC, Huawei, HiSilicon, CATT

R2-2101613	Draft CR: Remove the maximum number of MIMO layers restrictions for SUL	CMCC, Huawei, HiSilicon, CATT

R2-2100046	LS on broadcasting gNB ID length in system information block (R3-207226; contact: Ericsson)	RAN3

R2-2101415	On broadcasting gNB ID length in SIB1 (reply LS to R3-207226)	Ericsson