3GPP TSG-RAN WG2 Meeting #113 electronic	                                                                             R2-21xxxxx

25th January – 5th February 2021

Agenda Item:	7.3.2

Source: 	ZTE (email discussion rapporteur)

Title:  	                  Report of [AT113-e][302][eMTC R16] Paging narrowband selection (ZTE)

Document for:	Discussion and Decision

# Scope of the offline email discussion

This document contains the summary of the offline email discussion “[AT113-e][302][eMTC R16] Paging narrowband selection in RRC_INACTIVE for GWUS capable UEs”, as indicated below:

- [AT113-e][302][eMTC R16] Paging narrowband selection in RRC_INACTIVE for GWUS capable UEs (ZTE)

Scope:

Week 1: Try to converge on solution and agreeable proposals.

Week 2: Agree the CRs / potential LS.

Intended outcome:

Week 1: Report in R2-2102152

Week 2: Agreed CRs / potential LS.

Deadline:

Week 1: Jan 27 1100 UTC

Week 2 (if needed): Feb 04 1100 UTC

# Offline email discussion

## 2.1 Solutions

In last RAN2#112-e meeting, based on the contributions, the issue of paging narrowband selection for UE in RRC_INACTIVE when WUS is enabled was discussed and the following agreement has been achieved:

- UE in RRC_INACTIVE needs to monitor CN and RAN paging in the same paging narrowband.

In the following email discussion [1], three alternatives were proposed for implementing the agreement:

- Alt 1: UE in RRC_INACTIVE needs to monitor CN and RAN paging in the same paging narrowband and this paging narrowband is selected from the narrowbands that are configured with GWUS.

- Alt 2: UE in RRC_INACTIVE needs to monitor CN and RAN paging in the same paging narrowband and this paging narrowband is selected from the narrowbands that are provided in system information.

- Alt 3: UE in RRC_INACTIVE needs to monitor CN and RAN paging in the same paging narrowband and this paging narrowband is selected from the narrowbands that are provided in system information. Moreover, UE in RRC_IDLE monitors CN paging also in the paging narrowband selected from the ones that are provided in system information (e.g., eMTC UE always selects paging narrowbands from the ones provided in system information).

However, no final agreement on how to resolve this issue were achieved.

In current RAN2#113 e-meeting, several contributions are submitted with the following proposals:

| Contribution (Company) | Proposals | Initial Comments |
| --- | --- | --- |
| R2-2100943 (ZTE) | Proposal 1: RAN 2 firstly decide the Option 1 or Option 2 is used to guarantee that UE in RRC_INACTIVE monitors CN and RAN paging in the same paging narrowband. - Option 1: UE always selects paging narrowbands provided in system information - Option 2: The UE monitoring GWUS selects paging narrowbands that are configured with GWUS, and UE radio capability for paging is provided for the new anchor ng-eNB if the anchor ng-eNB changes during HO or RRC re-establishment/resume procedure. Proposal 1a: if Option 1 is agreed, the CRs[4][5] are adopted. Proposal 1b: if Option 2 is agreed, the CR[6] and LS[7] are adopted | Option1 is similar as Alt3 in [1] with additional requirement groupNarrowBandList would not be configured in this release. Option 2 is similar as Alt1 in [1]. |
| R2-2101037 (Huawei) | - Option 1: UE in RRC_INACTIVE camping on the ‘last used cell’ monitors CN and RAN paging in the paging narrowband selected among the paging narrowbands configured for GWUS. - Option 2: UE in RRC_INACTIVE ignores the ‘last used cell’ information and monitors CN and RAN paging in all paging narrowbands. Proposal 1: RAN2 to address the issue in RRC_INACTIVE rather than disabling the possibility to restrict GWUS to a subset of narrowbands in RRC_IDLE. Proposal 2: UE in RRC_INACTIVE discards the ‘last used cell’ information and monitors CN and RAN paging in all paging narrowbands. In case of AN release without the UE being released, the eNB does not report the ‘recommended cell for paging’ to the AMF. | Proposal 2 (option 2) looks similar as Alt2 in [1]. |
| R2-2101152 (Qualcomm) | Proposal 1: Ask RAN3 to make UE Radio Capability for Paging IE and eDRX configuration available to the paging RAN node to reliably page the UE in RRC_INACTIVE state. Proposal 2: Update RAN2 specification to allow UE in RRC_INACTIVE to use a paging narrowband that supports GWUS. | The option in Proposal 2 is similar as Alt1 in [1]. |
| R2-2101548 (Ericsson) | Proposal 1: If UE supports GWUS and the feature is enabled in the camped cell, the UE monitors a narrowband from a set of paging narrowbands used for paging provided in paging-narrowBands regardless of whether it is in RRC_IDLE or RRC_INACTIVE. Proposal 2: If UE supports GWUS and the feature is enabled in the cell, the UE monitors a paging narrowband from a set of paging narrowbands used for paging provided in paging-narrowBands when the cell, UE has camped in, supports connection to 5GC. | Three options are mentioned in the doc: Option 1 is similar as Alt3 in [1] and preferred. Option 2 is similar as Alt1 in [1]. |

In [R2-2101037], company suggest to resolve the issue for only UE in RRC_INACTIVE, e.g., UE in RRC_INACTIVE ignores the ‘last used cell’ information and monitors CN and RAN paging in all paging narrowbands. At the same time it’s still allowed to restrict GWUS to a subset of narrowbands and UE in RRC_IDLE can still select paging narrowband among the paging narrowbands that are configured with GWUS. This option (Option 2) looks similar as Alt2 in [1]. However, in the analysis for Alt2 in [1], the main issue is that when eNB receives CN paging and needs to send it to the UE, eNB would assume UE in RRC_IDLE (but the UE know itself in RRC_INACTIVE) and select the paging narrowband from the ones that are configured with GWUS. This would be different from the paging narrowband selected by the UE in RRC_INACTIVE and then the CN paging would be missed. In order to resolve this mismatch and other possible issue, [R2-2101037] mentions the following requirements for the preferred Option 2:

Observation 6: For Option 2, the GWUS capable eNB shall not report the ‘last cell’ information to the AMF when releasing the NG connection without releasing the UE.

Observation 7: For Option 2, the GWUS capable UE shall discard the ‘last cell’ information when entering RRC_INACTIVE.

This seems to mean, after the eNB release the UE to RRC_INACTIVE, it will not provide “last cell” information to the core network when later releasing NG connection. By this way, the GWUS can be implicitly disabled when next time the core network/eNB sends CN paging to the UE. Rapporteur think the feasibility of this suggestion may need more discussion. Moreover, in current RAN2/RAN3/SA2 specs, for eNB capable of GWUS, the last used cell should always be provided. Otherwise, the UE behavior is not specified. Then the impacts of this suggestion may be not small.

Another option (Option 1) is also mentioned in [R2-2101037] “UE in RRC_INACTIVE camping on the ‘last used cell’ monitors CN and RAN paging in the paging narrowband selected among the paging narrowbands configured for GWUS.(same as UE in RRC_IDLE)” (similar as Alt1 in [1]). For this option, [R2-2101037] raises an additional issue as: “Observation 4: For option 1, changes on the Ng and Xn interface messages are required to provide the eNB with the “last used cell” for WUS for UEs in RRC_INACTIVE.” Therefore, company doesn’t prefer this option. Rapporteur think this new issue exists. But it can also be inform to RAN3 for further CR discussion if the related scheme can be agreed in RAN2.

In [R2-2101548], Option 1 is similar as Alt3 in [1] and preferred. Moreover, option 1 is with additional assumption that “Since it is not be possible for such UEs to monitor a paging narrowband from a set of narrowbands configured for GWUS when in RRC_IDLE mode, GWUS can only be enabled if it is supported on all paging narrowbands.” Rapporteur understand this is similar as the additional requirement of Option 1 in [R2-2100943], e.g., groupNarrowBandList would not be configured in this release.

In [R2-2101548], another option, Option 3 has been mentioned. It assumes an implementation that UE can monitor two different narrowbands and switch between separate narrowbands quickly for monitoring, when needed. Rapporteur think this option has been discussed in last meeting and more companies think it infeasible. Therefore we have the above agreement.

Based on the above roughly analysis, rapporteur think there are two kinds of solutions on the table. Therefore, the following solutions are compiled for further discussion.

Q1: Companies are invited to indicate which solution is preferred?

Type#1 solution: UE in RRC_INACTIVE and UE in RRC_IDLE use same scheme for paging narrowband selection.

- Solution 1: For both UE in RRC_IDLE and UE in RRC_INACTIVE, if UE supports GWUS and the feature is enabled in the camped cell, the UE monitors a narrowband selected from the complete set of paging narrowbands provided in system information regardless of which paging narrowbands support GWUS.

- Solution 2: If UE & network configurations require UE to use GWUS in RRC_IDLE then a UE in RRC_INACTIVE monitors a narrowband selected from the set of paging narrowbands configured with GWUS.

Type#2 solution: UE in RRC_INACTIVE and UE in RRC_IDLE use different schemes for paging narrowband selection but it needs to ensure the selection result is the same for UE and eNB.

- Solution 3: The UE in RRC_IDLE monitors a narrowband selected from the ones configured with GWUS while the UE in RRC_INACTIVE monitors a narrowband selected from the ones provided in system information. Moreover, the UE in RRC_INACTIVE needs to discard the ‘last used cell’ information.

- Solution 4: The UE in RRC_IDLE monitors a narrowband selected from the ones configured with GWUS while the UE in RRC_INACTIVE monitors a narrowband selected from the ones provided in system information.  Moreover, GWUS can only be enabled if it is supported on all paging narrowbands or groupNarrowBandList would not be configured.

Other solution.

| Company | Preference | Detailed comments |
| --- | --- | --- |
| Qualcomm | Solution 2 | We consider RAN2 changes are relatively small to support this solution. Regardless of what solution RAN2 selects, RAN3 changes are required to properly support RRC_INACTIVE for BL UE and UE in CE regardless of which solution RAN2 selects. We think other solutions are regressive for the following reason: Solution 1: Removes the advantage of using GWUS on subset of the paging narrowbands and allow all UEs support GWUS to take advantage. If GWUS support on subset of the paging narrowbands is removed then when network configures GWUS on subset of the paging narrowbands it is a lottery which UE can benefit from this feature. Solution 3: This solution can lead to paging miss when CN and RAN Pos overlap but UE. Solution 4: This can lead paging mismatch when there is state mis-match between UE and RAN. Therefore, solution 2 has the following benefits: - Retains benefit of GWUS usage on sub-set of the paging narrowbands. - Avoids missed paging due to RAN & CN PO collision - Avoids missed paging due to UE state mismatched between UE and RAN. |
| Huawei, HiSilicon | Type #2 | We think Type #2 was the initial assumption. We think the description of the solutions is not really correct. Solution 1: We assume this is ZTE’s option 1 ? It actually disables the use of GWUS narrowband in the system. As per the ZTE’ CR it impacts both UEs and eNB. We do not see any benefit to this option compared to Ericsson’s option. Solution 2: We assume this is Qualcomm’s option. This option is not enough on its own (many other changes are required in RAN2 and RAN3 including non-backward compatible ASN.1 changes). Qualcomm’s CR introduces a new discrepancy when the UE is not on the last WUS cell. Solution 3: We assume this is Huawei’s option. The option does not change the functionality, i.e. usage of GWUS specific NB is still available in 5GC when RRC_INACTIVE is supported by the eNB. The option impacts both the UE and the eNB but has no impact on the interfaces. There is no paging miss in the mismatch case as both the UE and the eNB does not use GWUS until the UE is explicitly released by the eNB to RRC_IDLE. Solution 4: we assume this is Ericsson option ? But the description is incomplete as it does not indicate that the NW does not configure a restricted set of NB for GWUS when 5GC is supported by the eNBs. This option disables the use of GWUS narrowbands in 5GC, but it could be limited to the case where RRC_INACTIVE is supported by the eNB. However, this option is simple, and only impacts on the eNB. We also think that the configuration of GWUS specific narrowband is just an optimisation and is not an essential aspect of the feature. |

Conclusion:

Proposal:

## 2.2 Possible changes for solutions

Based on the submitted CRs, rapporteur give a summary in the following table and quote the main changes for different solutions below the table:

|  | solution | Possible changes | CR example |
| --- | --- | --- | --- |
| Type#1 solution | Solution 1 | - TS 36.304 spec needs to be modified that both UE in RRC_IDLE and UE in RRC_INACTIVE select a paging narrowband among the ones provided in system information. - TS 36.331 spec needs to be modified, e.g., to avoid redundant/useless groupNarrowBandList configuration. | ZTE-option 1, Ericsson Option with adding TS 36.304 changes |
|  | Solution 2 | - TS 36.304 spec needs to be modified that UE in RRC_INACTIVE also selects a paging narrowband among the ones configured with GWUS even though UE in RRC_INACTIVE is not required to monitor GWUS signal. - LS to RAN3 is required (to make UE Radio Capability for Paging IE and last cell info available to the paging RAN node) | QC option, ZTE-option 2 |
| Type#2 solution | Solution 3 | It’s assumed that in current TS 36.304 spec, UE in RRC_INACTIVE by default would select a paging narrowband among the ones provided in system information as it doesn’t monitor GWUS signal. Therefore, for this Type#2 solution, it seems no change to TS 36.304 is needed. - At least TS 36.300 and maybe TS 38.413 need to clarify that the GWUS capable eNB shall not report the ‘last cell’ info to the AMF when releasing the NG connection without releasing the UE and the GWUS capable UE shall discard the ‘last cell’ info when entering RRC_INACTIVE. | N/A |
|  | Solution 4 | - TS 36.331 needs to have restriction that GWUS can only be enabled if it is supported on all paging narrowbands or groupNarrowBandList would not be configured. | Ericsson Option, ZTE-option 1 with removing TS 36.304 change |
| Other solution |  |  |  |

ZTE-option 1:

TS 36.304 ## 7.1 Discontinuous Reception for paging ……. - Nn: number of paging narrowbands (for P-RNTI monitored on MPDCCH) or paging carriers (for P-RNTI monitored on NPDCCH) determined as follows: If P-RNTI is monitored on MPDCCH: this is the number of paging narrowbands provided in system information. else if P-RNTI is monitored on NPDCCH: If UE monitors GWUS according to clause 7.5.1: this is the number of paging carriers that are configured with GWUS. else: this is the number of paging carriers provided in system information. TS 36.331: groupNarrowBandList List indicating which paging narrowbands support group WUS see TS 36.304 [4]. First entry in the list indicates WUS support for first paging narrowband, second entry in the list indicates WUS support for second paging narrowband, and so on. If E-UTRAN includes groupNarrowBandList, the number of entries is equal to the value of paging-narrowBands. If this list is absent, group WUS is supported on all paging narrowbands. In this version, this field should not be configured..

Ericsson option:

TS 36.331 groupNarrowBandList List indicating which paging narrowbands support group WUS see TS 36.304 [4]. First entry in the list indicates WUS support for first paging narrowband, second entry in the list indicates WUS support for second paging narrowband, and so on. If E-UTRAN includes groupNarrowBandList, the number of entries is equal to the value of paging-narrowBands. If this list is absent, group WUS is supported on all paging narrowbands. When E-UTRA is connected to 5GC, this list is absent..

QC option:

TS 36.304 ## 7.1 Discontinuous Reception for paging ……. If UE monitors GWUS according to clause 7.5.1 or for UE in RRC_INACTIVE the conditions defined in clause 7.5.x are met: this is the number of paging narrowbands (paging carriers) that are configured with GWUS. else: this is the number of paging narrowbands (paging carriers) provided in system information ……. ### 7.5.x Conditions for determining paging narrowband set When all of the following conditions are met, the UE in RRC_INACTIVE shall select paging narrowand from a set of paging narrowbands configured with GWUS: - the UE supports GWUS; - GWUS configuration (gwus-Config) is provided in system information; - groupAlternation is present in gwus-Config and UE supports GWUS with group resource alternation; or - groupAlternation is not present in gwus-Config.

The draft LS out to RAN3 can be found in [R2-2101153] and the corresponding RAN3 CRs can be found in [R3-210338	] and [R3-210339].

ZTE-option 2:

TS 36.304 ## 7.1 Discontinuous Reception for paging - Nn: number of paging narrowbands (for P-RNTI monitored on MPDCCH) or paging carriers (for P-RNTI monitored on NPDCCH) determined as follows: If UE monitors GWUS according to clause 7.5.1: this is the number of paging narrowbands (paging carriers) that are configured with GWUS. else: this is the number of paging narrowbands (paging carriers) provided in system information. Note: A UE in RRC_INACTIVE uses the same paging narrowband list as that the UE uses in RRC_IDLE so that the same paging narrowband can be selected for the CN paging and RAN paging.

The draft LS out to RAN3 can be found in [R2-2100968] and the corresponding RAN3 CRs can be found in [R3-210338	] and [R3-210339].

Q2: Companies are invited to provide their detailed suggestions or comments for the corresponding CRs.

(Note: please comment with main intention to evaluate the feasibility and spec impacts for the solutions. The wording issue can be left to Phase 2 discussion after determining the solution).

| Company | Preferred CR changes | Comments |
| --- | --- | --- |
| Qualcomm | QC option with changes. | There are many ways of updating TS 36.304 to specifiy that UE in RRC_INACTIVE selects a paging narrowband the same way it would for RRC_IDLE mode in this cell. The ZTE solution for Option 2 does not work because for a UE in RRC_INACTIVE the conditions defined in section 7.5.1 would not be met because one of the condition in section 7.5.1 is UE is in RRC_IDLE. Therefore, further changes would be needed in section 7.5.1 to take into account RRC_INACTIVE state. We find such changes lead to more complex text in section 7.5.1. We consider the QC solution is cleaner and does not affect RRC_IDLE, but it does need additional modification (shown highlighted) to ensure subset of paging narrowbands only apply in the cell where UE entered RRC_INACTIVE in response to RRC connection release message. ### 7.5.x Conditions for determining paging narrowband set When all of the following conditions are met, the UE in RRC_INACTIVE shall select paging narrowand from a set of paging narrowbands configured with GWUS: - the UE supports GWUS; - GWUS configuration (gwus-Config) is provided in system information; - groupAlternation is present in gwus-Config and UE supports GWUS with group resource alternation; or - groupAlternation is not present in gwus-Config. - the UE most recently entered RRC_INACTIVE in this cell triggered by reception of RRCConnectionRelease not including noLastCellUpdate |
| Huawei, HiSilicon |  | we should agree the solution first |

Conclusion:

Proposal:

# Summary

Conclusion:

Updated CRs:

# References

[1] 3GPP, R2-2010910_Report of R16 Paging narrowband selection, RAN2#112-e, November 2020

[2] 3GPP, R2-2100943	Discussion for correction on paging narrowband selection	, ZTE Corporation, Sanechips, discussion

[3] 3GPP, R2-2100957	Correction on paging narrowband selection-Option 1, ZTE Corporation, Sanechips,	CR 36.304 16.3.0	 0819, RAN2#113-e, January 2021

[4] 3GPP, R2-2100965	Correction on paging narrowband selection-Option 1, ZTE Corporation, Sanechips,	CR 36.331 16.3.0	 4556, RAN2#113-e, January 2021

[5] 3GPP, R2-2100966	Correction on paging narrowband selection-Option 2, ZTE Corporation, Sanechips,	CR 36.304 16.3.0	 0821, RAN2#113-e, January 2021

[6] 3GPP, R2-2100968	Draft LS to RAN3 on UE radio capability provision	, ZTE Corporation, Sanechips, LS out, To:RAN3, RAN2#113-e, January 2021

[7] 3GPP, R2-2101037	Paging monitoring in RRC_INACTIVE for GWUS capable Ues, Huawei, HiSilicon, discussion, RAN2#113-e, January 2021

[8] 3GPP, R2-2101152	Paging narrowband/carrier selection after RRC connection release, 		Qualcomm Incorporated,  discussion, RAN2#113-e, January 2021

[9] 3GPP, R2-2101154	Paging narrowband selection in RRC-INACTIVE state	Qualcomm Incorporated, CR 36.304 16.3.0  0823, RAN2#113-e, January 2021

[10] 3GPP, R2-2101153	[draft] LS on parameters needed at paging RAN node to reliably page an eMTC UE in RRC-INACTIVE state, Qualcomm Incorporated, LS out, To:RAN3, RAN2#113-e, January 2021

[11] 3GPP, R2-2101548	Paging narrowband selection in RRC_INACTIVE for LTE-M, Ericsson	, discussion, RAN2#113-e, January 2021

[12] 3GPP, R2-2101549	Correction to paging narrowband selection in RRC_INACTIVE for LTE-M	, Ericsson 	CR 36.331 16.3.0 	4581, RAN2#113-e, January 2021

[13] 3GPP, R3-210338	Use of the UE’s Radio Capability for Paging in RRC_INACTIVE, Qualcomm Incorporated, ZTE, RAN3#111-e, January 2021

[14] 3GPP, R3-210339	Supporting use of UE Radio Capability for Paging in RRC_INACTIVE, Qualcomm Incorporated, ZTE, RAN3#111-e, January 2021

# Participants

| Company | Name | e-mail address |
| --- | --- | --- |
| Qualcomm | Mungal Dhanda | mdhanda@qti.qualcomm.com |