3GPP TSG-RAN WG2 Meeting #113-e	R2-21xxxxx

Electronic, January 25 – February 5, 2021

Agenda item:	5.5

Source: 	Qualcomm Incorporated

Title: 	Summary of Email discussion [AT113-e][603][POS] NR Rel-15 positioning CRs

Document for:		Discussion and Decision

# 1. 	Introduction

This document summarizes the following email discussion.

- [AT113-e][603][POS] NR Rel-15 positioning CRs (Qualcomm)

Scope: Discuss and conclude on R2-2101380/R2-2101381, R2-2101465, R2-2101468, R2-2100397, R2-2100398/R2-2100399, R2-2100400/R2-2100401, R2-2101816/R2-2101817, and R2-2101926/R2-2101927

Intended outcome: Agreed CRs

Deadline:  Monday 2021-02-01 1200 UTC

References:

[1]		R2-2100397, "Remove the NOTE in architecture figure in TS38.305", CATT
						CR	 Rel-15, 38.305, 5.7.0, 0054	, F, NR_newRAT-Core.

[2a]	R2-2100398, "Corrections on the indication for the not provided assistance data and location information in 							TS38.305", CATT
						CR	 Rel-15, 38.305, 15.7.0, 0055, F, NR_newRAT-Core.

[2b]	R2-2100399, "Corrections on the indication for the not provided assistance data and location information in 							TS38.305", CATT
						CR	 Rel-16, 38.305, 16.3.0, 0056, A, NR_newRAT-Core.

[3a]	R2-2100400, "Corrections on the descriptions of RequestLocationInformation message in TS38.305", CATT
						CR Rel-15, 38.305, 15.7.0, 0057, F , NR_newRAT-Core.

[3b]	R2-2100401, "Corrections on the descriptions of RequestLocationInformation message in TS38.305", CATT
						CR Rel-16, 38.305, 16.3.0, 0058, A, NR_newRAT-Core.

[4a]	R2-2101465, "Support OTDOA assistance data for case of NR serving cell", Qualcomm Incorporated, Ericsson						CR Rel-15, 38.305, 15.7.0, 0061, F, NR_newRAT-Core.

[4b]	R2-2101468, "Support OTDOA assistance data for case of NR serving cell", Qualcomm Incorporated, Ericsson						CR Rel-16, 38.305, 16.3.0, 0062, F, NR_newRAT-Core.

[5a	]	R2-2101815, "Clarification on E-CID and NR E-CID", Huawei, HiSilicon
						discussion, Rel-15, NR_newRAT-Core

[5b]	R2-2101816, "Correction to E-CID-R15", 	Huawei, HiSilicon
						CR Rel-15, 38.305, 15.7.0, 0063, F, NR_newRAT-Core

[5c]	R2-2101817,	"Correction to E-CID-R16", Huawei, HiSilicon
						CR Rel-16, 38.305, 16.3.0, 0064, A, NR_newRAT-Core

[6a]	R2-2101926, "Correction on the description for UE capability transfer-R15", Huawei, HiSilicon
						CR	 Rel-15, 38.305, 15.7.0, 0066, F, NR_newRAT-Core

[6b]	R2-2101927, "Correction on the description for UE capability transfer-R16", Huawei, HiSilicon
						CR Rel-16, 38.305, 16.3.0, 0067, A, NR_newRAT-Core

[7a]	R2-2101928, "Correction to 5G support for NB-IOT positioning-R15", Huawei, HiSilicon
						CR Rel-15, 38.305, 15.7.0, 0068, F, NR_newRAT-Core

[7b]	R2-2101929, "Correction to 5G support for NB-IOT positioning-R16", Huawei, HiSilicon
						CR Rel-16, 38.305, 16.3.0, 0069, A, NR_newRAT-Core

[8a]	R2-2101379, "GNSS RTK observations resolution indication", Ericsson
						discussion, Rel-15

[8b]	R2-2101380, "Correction of A-GNSS Assistance Data RTK Observation", Ericsson
						CR	 Rel-15, 37.355, 15.1.0, 0285, F, NR_newRAT-Core.

[8c]	R2-2101381, "Correction of A-GNSS Assistance Data RTK Observation", Ericsson
						CR Rel-16, 37.355, 16.3.0, 0286, A, NR_newRAT-Core

# 2. 	Discussion

## 2.1	38.305: Remove the NOTE in architecture figure (R2-2100397 [1])

Reason for change:	In RAN3 LS R2-2008514/ R3-205719 LS on the NOTE in architecture figure in TS 38.305, 					RAN3 mentioned

RAN3 have discussed the note in the architecture figure of stage-2 TS 38.305 section 5.1, 						which reads “Proprietary interface possible”. RAN3 is of the opinion that such a note is not 					needed in the stage-2 NG-RAN architecture figure and does not add clarity. Furthermore, 						RAN3 believes that such archietcture details are in the RAN3 domain.

RAN3 therefore respectfully asks RAN2 to take the above into account and consider removing 					the note.

Summary of change:	Remove the note for the figure Figure 5.1-1.

Rapporteur's Comments:

-	The NOTE in the architecture Figure 5.1-1 was removed in the Rel-16 version of the specification (CR#0037) at RAN2#112e, but not in the Rel-15 version.

-	The mentioned RAN3 LS (R2-2008514/R3-205719) was related to the Rel-16 NR Positioning WI (NR_pos-Core) and not NR_newRAT-Core.

Question 1-1:	Do you agree that a Rel-15 correction is needed according to the Reason for Change provided in 				R2-2100397 [1]?

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 1-2: 	If your answer to Question 1-1 was "Yes", do you agree with the CR in [1]?

| Company | Yes / Yes with Modification | Comments |
| --- | --- | --- |

## 2.2	38.305: Corrections on the indication for the not provided assistance data and location information (R2-2100398, R2-2100399) [2]

Reason for change:	According to the LPP error detection in the error handling procedures in TS37.355:

1>	if the message type is an LPP RequestAssistanceData or RequestLocationInformation and 						some or all of the requested information is not supported:

2>	return any information that can be provided in a normal response, which includes 								indications on other information that is not supported.

It specifies that if some or all of the requested information is not supported for the assistance 					data or location information transfer procedure, LMF or UE will reture any information that 					can be provided in an LPP message, which includes indications on the information that is not 					provided.

However, as for the assistance data transfer procedure for A-GNSS, OTDOA, Barometric 						Pressure Sensor, WLAN and TBS positioning methods in TS38.305, it only specifies the case 					that when all of the requested assistance data is not supported, LMF will return any 								information that can be provided in an LPP message, which includes indications on the 							assistance data that is not provided. As for the case that some of the requested assistance data is 					not provided, what LMF should to do is unclear.

Besides, as for the location information transfer procedure for A-GNSS, OTDOA, E-CID, 						Barometric Pressure Sensor, WLAN, Bluethooth, TBS and Motion Sensor positioning methods 					in TS38.305, it only specifies the case that when all of the requested assistance data is not 						supported, UE will return any information that can be provided in an LPP message, which 						includes indications on the assistance data that is not provided. As for the case that some of the 					requested assistance data is not provided, what UE should to do is unclear.

Summary of change:	1.	For the assistance data transfer procedure for A-GNSS, OTDOA, Barometric Pressure 							Sensor, WLAN and TBS positioning methods in TS38.305, add a clarification that If some 						of the UE requested assistance data cannot be provided by the LMF, return any information 						that can be provided in an LPP message of type Provide Assistance Data which includes a 						cause indication for the not provided assistance data.

2.	For the assistance data transfer procedure for A-GNSS, OTDOA, E-CID, Barometric 							Pressure Sensor, WLAN, Bluethooth, TBS and Motion Sensor positioning methods in 							TS38.305, add a clarification that If some of the requested measurement information cannot 						be provided by the UE, the UE returns any information that can be provided in an LPP 							message of type Provide Location Information which includes a cause indication for the not 						provided location information.

NOTE:	Similar CRs for TS 36.305 (Rel-14 – Rel-16) are also proposed in AI 4.4 (R2-2100394, R2-2100395, R2-2100396)

Rapporteur's Comments:

-	This Stage 2 text is available since Rel-9 and seems has not created problems/misunderstandings so far. It is also unclear how this issue can result in "positioning accuracy decrease or even positioning failure", as mentioned in the "Consequences if Not Approved" section.

-	The Rel-15 and Rel-16 CRs are not exact mirrors (there are more affected clauses in the Rel-16 version compared to the Rel-15 version); i.e., both CRs (if needed) should probably be Cat F.

Question 2-1:	Do you agree that a correction is needed according to the Reason for Change provided in 						R2-2100398/R2-2100399 [2]?

| Company | Yes/No | Release 15/16/both | Comments |
| --- | --- | --- | --- |

Question 2-2:	If your answer to Question 2-1 was "Yes", do you agree with the CRs in [2a] and/or [2b]?

| Company | Yes / Yes with Modification | Comments |
| --- | --- | --- |

## 2.3	38.305: Corrections on the descriptions of RequestLocationInformation message (R2-2100400, R2-2100401) [3]

Reason for change:	1. According to the IE A-GNSS-RequestLocationInformation in  LPP 												RequestLocationInformation message in TS37.355, the positioning mode of UE-based and UE-					assisted is indicated within the RequestLocationInformation message but not within the 							positioning instructions of A-GNSS.

However, the current specification specifies that there include positioning mode of UE-based, 					UE-assisted and Standalone within the positioning instructions of A-GNSS, which is conflict 					with TS37.355.

2. According to the LPP RequestLocationInformation message in TS37.355, there is not 						include positioning instructions for WLAN, Bluethooth, TBS, Motion Sensor and Barometric 					Pressure Sensor positioning methods. Besides, the positioning mode indicated within the 						RequestLocationInformation message do not include Standalone mode.

However, the current specification specifies that there include Standalone positioning mode 					within the positioning instructions for WLAN, Bluethooth, TBS, Motion Sensor and 								Barometric Pressure Sensor positioning methods, which is conflict with TS37.355.

3. A-GNSS positioning method supporte standlone mode as described in clause 8.1. However, 					there lacks of A-GNSS positioning methods in the general descriptions of positioning methods 					supported in standlone mode in clause 4.3.1.

4. According to the LPP RequestLocationInformation message in 37.355, the positioning mode 					can also be indicated within the RequestLocationInformation message for the DL-AOD, DL-					TDOA as well as multi-RTT positioning methods, which does not described in the 								corresponding location information transfer procedure in current TS38.305.

Summary of change:	1.For the location information transfer procedure of A-GNSS, remove the positioning mode 					from the description of the positioning instructions and clarify the corresponding positioning 					mode can be indicated within the RequestLocationInformation message.

2. For the location information transfer procedure of WLAN, Bluethooth, TBS, Motion 							Sensor and Barometric Pressure Sensor positioning methods, delete the description of the 						positioning instructions and clarify the corresponding positioning mode can be indicated 						within 	the RequestLocationInformation message.

3. Add A-GNSS positioning method in the general descriptions of positioning methods 							supported in standlone mode in clause 4.3.1.

4. For the location information transfer procedure of DL-AOD, DL-TDOA and multi-RTT 						positioning methods, clarify that the corresponding positioning mode can be indicated within 					the RequestLocationInformation message.

NOTE:	The Rel-15 [3a] and Rel-16 [3b] CRs are not identical:
Change #3 above is only proposed for the Rel-15 version;
Change #4 above  is only proposed for the Rel-16 version.
(For simplicity, the Reason/Summary of Change has been merged in the description above; please see the actual CRs in [3]).

Rapporteur's Comments:

-	"Positioning Instructions" in Stage 2 are used in a generic sense which does not necessarily refer to specific LPP IEs. "Positioning Instructions" generally include common and positioning method specific instructions.

-	Assisted-GNSS was not included in the list of methods supported in standalone mode below Table 4.3.1-1 on purpose. Assisted-GNSS per se is not standalone. Clause 8.1 describes "GNSS positioning methods" incl. A-GNSS and autonomous (standalone) mode.

-	Since Rel-15 and Rel-16 CRs are not exact mirrors, both CRs (if needed) should probably be Cat F.

Question 3-1:	Do you agree that a correction is needed according to the Reason for Change provided in 						R2-2100400/R2-2100401 [3]?

| Company | Yes/No | Release 15/16/both | Comments |
| --- | --- | --- | --- |

Question 3-2:	If your answer to Question 3-1 was "Yes", do you agree with the CRs in [3a] and/or [3b]?

| Company | Yes / Yes with Modification | Comments |
| --- | --- | --- |

## 2.4	38.305: Support OTDOA assistance data for case of NR serving cell (R2-2101465, R2-2101468) [4]

Reason for change:	1. When the UE is served by a NR cell, and the LMF requests the UE to perform 									measurements for OTDOA positioning, the LMF may provide assistance data including 							optionally timing information to enable the UE to quickly acquire the LTE reference cell. 						However the LMF is not aware of timing relationships between the LTE and NR cells because 					it does not receive the SFN Initialization Time for NR cells, although it is of course able to do 					so for LTE cells.

RAN3 has agreed a Rel-15 NRPPa CR (R3-207173, with Rel-16 mirror in R3-207174) 							whereby the OTDOA Information Exchange procedure is used towards gNBs in order to 						enable the LMF to retrieve timing and access point location information for cells in a gNB. 						With this, it becomes possible for the LMF to construct the assistance data in LPP related to 					LTE-NR timing offset.

However stage 2 is not aligned to this.

2.	There is a remaining Editor's Note in clause 8.2.3.2.2 which has been removed in the Rel-					16 version (CR#0053) but not in the Rel-15 version of the specification.

3.	In clause 8.10.3.2.3, 8.13.3.3a, 8.14.3.3a: in the description of each bullet point (2) there is 					a wrong reference to the MAC specification.

Summary of change:	1. OTDOA Positioning support includes potential interaction with gNBs, and this is captured in 					relevant sections.						
					A new section is introduced under clause 8.2.2 to list the information that may be transferred 					from the gNB to LMF.
					The detail on Assistance Data Delivery between LMF and ng-eNB for OTDOA is generalized 					(i.e. ng-eNB is replaced by NG-RAN node wherever applicable).

2. Obsolete Editor's Note is deleted.

3. In clause 8.10.3.2.3, 8.13.3.3a, 8.14.3.3a: in the description of each bullet point (2) the 						reference to the MAC specification 38.321 is corrected.

NOTE:	The Rel-15 [4a] and Rel-16 [4b] CRs are not identical:
Change #2 is only proposed for the Rel-15 version;
Change #3 is only proposed for the Rel-16 version.
(For simplicity, the Reason/Summary of Change has been merged in the description above; please see the actual CRs in [4]).

Rapporteur's Comments:

-	The OTDOA Information Exchange procedure towards gNBs has been added to NRPPa Rel-15 by RAN3.

Question 4-1:	Do you agree that a correction is needed according to the Reason for Change provided in 						R2-2101465/R2-2101468 [4]?

| Company | Yes/No | Release 15/16/both | Comments |
| --- | --- | --- | --- |

Question 4-2:	If your answer to Question 4-1 was "Yes", do you agree with the CRs in [4a] and/or [4b]?

| Company | Yes / Yes with Modification | Comments |
| --- | --- | --- |

## 2.5	38.305: Correction to E-CID (R2-2101815, R2-2101816, R2-2101817) [5]

Reason for change:	The specification support in Rel-15 for a gNB to report E-UTRA measuremrent as E-CID is 					not included in the WID NR_newRAT, and is also contradicting in the current stage-2 							specification. A Rel-16 CR has already removed the support, but the Rel-15 spec has not been 					changed accordingly.

Summary of change:	1. The sentence “In the case of a serving gNB, E-CID positioning can be supported using E-						UTRA measurements provided by a UE to the serving gNB” is removed from clause 4.3.4.

2. Remove the E-UTRA measurement from the list transferred from gNB to LMF in clause 						8.3.2.3.

Rapporteur's Comments:

-	Cover Sheet: The Rel-16 version has no CR number; both CRs have no revision number (should be "-").

-	Both CRs (if needed) should probably be Cat F.

Question 5-1:	Do you agree that a correction is needed according to the Reason for Change provided in 						R2-2101816/R2-2101817 [5]?

| Company | Yes/No | Release 15/16/both | Comments |
| --- | --- | --- | --- |

Question 5-2:	If your answer to Question 5-1 was "Yes", do you agree with the CRs in [5b] and/or [5c]?

| Company | Yes / Yes with Modification | Comments |
| --- | --- | --- |

## 2.6	38.305: Correction on the description for UE capability transfer (R2-2101926, R2-2101927) [6]

Reason for change:	In Sectoin 7.1.2.1, the description for capability transfer refers the whole section to the LTE 					positioning stage2 spec for more detials. While in 36305, the description is for E-SMLC server 					instead of LMF.

7.1.2.1	Capability transfer

A UE request for capability from E-SMLC or delivery of the E-SMLC capability to the UE 						is not supported in this version of the specification.

Capabilities in an LPP context refer to the ability of a target or server to support different 						position methods defined for LPP, different aspects of a particular position method (e.g. 							different types of assistance data for A-GNSS) and common features not specific to only 						one position method (e.g. ability to handle multiple LPP transactions). These capabilities 						are defined within the LPP protocol and transferred between the target and the server using 						LPP transport.

The exchange of capabilities between a target and a server may be initiated by a request or 						sent as "unsolicited" information. If a request is used, the server sends an LPP Request 							Capabilities message to the target device with a request for capability information. The 							target sends an LPP Provide Capabilities message.

Revised paragraph similar to section 7.1.2.5 needs to be added to revise for the description of 					LMF.

Summary of change:	Add a complete description for capability transfer for NR E-CID instead of refering to the LTE 					spec.

Rapporteur's Comments:

-	The term "E-SMLC" appears only in the first sentence of the referenced description. This sentence seems generally not wrong also for 38.305.

-	Cover Sheet: Both CRs have no revision number (should be "-").

Question 6-1:	Do you agree that a correction is needed according to the Reason for Change provided in 						R2-2101926/R2-2101927 [6]?

| Company | Yes/No | Release 15/16/both | Comments |
| --- | --- | --- | --- |

Question 6-2:	If your answer to Question 6-1 was "Yes", do you agree with the CRs in [6a] and/or [6b]?

| Company | Yes / Yes with Modification | Comments |
| --- | --- | --- |

## 2.7	38.305: Correction to 5G support for NB-IOT positioning (R2-2101928, R2-2101929) [7]

Reason for change:	In the legacy LTE stage2 spec for positioning, the following paragraph has been specified for 					NB-IOT positioning.

7.1.3	UE positioning measurements in idle state for NB-IoT

[…]

While for NR spec, the above description is not given. In our understanding, NB-IOT 							supported in NR is under the scope of NB-IoT connected to 5GC  and is supported for the 						current network architecture for positioning.

Summary of change:	 Add a paragraph for the description for NB-IOT positioning in 5G.

Rapporteur's Comments:

-	Cover Sheet: Both CRs have no revision number (should be "-"); clauses affected are missing (the proposed new clause should be mentioned).

- 	36.305 has the NB-IoT text in clause 7.1.3. For 38.305, clause 7.x seems more appropriate than the proposed 5.3.x (if the CR is needed).

Question 7-1:	Do you agree that a correction is needed according to the Reason for Change provided in 						R2-2101928/R2-2101929 [7]?

| Company | Yes/No | Release 15/16/both | Comments |
| --- | --- | --- | --- |

Question 7-2: 	If your answer to Question 7-1 was "Yes", do you agree with the CRs in [7a] and/or [7b]?

| Company | Yes / Yes with Modification | Comments |
| --- | --- | --- |

## 2.8	37.355: Correction of A-GNSS Assistance Data RTK Observation (R2-2101379, R2-2101380, R2-2101381) [8]

Reason for change:	The GNSS RTK observations IE was introduced based on the RTCM MSM messages. These 					messages come in two different resolutions, standard and high, while RTK-Observations only 					represents in the high resolution. This means that the device cannot determine the resolution of 					the original MSM message. Therefore, a resolution indicator standard/high would makes the 					representation complete.

Such a resolution indication can be used by the device to assess uncertainty together with the 					RTK residuals, and also in case the messages are translated back internally in the device to 						RTCM MSM messages to use a legacy positioning engine that supports RTCM MSM 							messages.

Summary of change:	 An optional resolution indicator is added to the GNSS-RTK-Observations IE.

Rapporteur's Comments:

-	Unclear how a resolution indication "can be used by the device to assess uncertainty".

-	LPP value range allows for the high-resolution, which includes the low-resolution as special cases.

-	The low-resolution fields would also need a different scale factor.

-	Work Item code seems wrong; should be LCS_LTE_acc_enh-Core (if a CR is needed).

Question 8-1:	Do you agree that a correction is needed according to the Reason for Change provided in 						R2-2101380/R2-2101381 [8]?

| Company | Yes/No | Release 15/16/both | Comments |
| --- | --- | --- | --- |

Question 8-2: 	If your answer to Question 8-1 was "Yes", do you agree with the CRs in [8b] and/or [8c]?

| Company | Yes / Yes with Modification | Comments |
| --- | --- | --- |

# 3. 	Summary