3GPP TSG-RAN WG2 Meeting #113-e	R2-210xxxx

Electronic, 25nd Jan– 05th Feb, 2021

Agenda Item:	8.11.3.2

Source:	Huawei, HiSilicon

Title:	Text proposal for IDLE and INACTIVE positioning-Ph2

Document for: Discussion

Introduction

During RAN2#113e, the following email discussion is triggered after the meeting

[AT113-e][609][POS] Continued discussion of positioning in idle/inactive (Huawei)

Scope: Continue discussion of the issues from R2-2101230, and converge to an agreeable TP, taking as a baseline the principle that positioning in inactive is supported as recommended by RAN1.  R2-2101229 to be taken into account.

Intended outcome: Endorsable TP

Deadline:  Tuesday 2021-02-02 1200UTC

During the online discussion of the summary for the above email discussion, the following agreements have been made:

Agreements:

Proposal 1a: RAN2 confirms on the following recommendation of TSG RAN (17/17)

	Positioning in RRC_INACTIVE

	DL, UL and DL+UL positioning methods

	UE-based and UE-assisted positioning solutions

	Support of UE positioning measurements for UEs in RRC_INACTIVE state

	Options that can be considered include DL-PRS or DL-PRS and SSB

	Support of gNB positioning measurements for UEs in RRC_INACTIVE state

Proposal 1b: RAN2 confirms on the following (17/17)

	Positioning in RRC_IDLE

	It is feasible for a UE to perform DL positioning measurement in RRC_IDLE state

	It is up to RAN2 to decide whether to support the enhancements of NR positioning reporting of DL positioning measurements and/or positioning estimates for RRC_IDLE UEs.

Proposal2: RAN2 recommends the following for normative work for DL positioning

	The report of PRS measurement performed in RRC_IDLE/INACTIVE when the UE is in RRC_INACTIVE is supported (10/12)

	PRS measurement report and/or location estimate are sent from the UE to the gNB in RRC_INACTIVE.  RAN2 generally agree to do this by enhancing small data transmission in RRC_INACTIVE (details of the use of SDT to be studied in the WI phase) (15/16)

Proposal4: For DL positioning in IDLE/INACTIVE, the followings are already supported for the current spec and can be reused:

	Current stage3 spec has already supported assistance data delivery for DL positioning during RRC_CONNECTED and on-demand SI request in RRC_IDLE/ INACITVE for IDLE/INACTIVE positioning. (14/14)

	Current stage3 spec already supports the transfer of RequestLocationInformation in RRC_CONNECTED for PRS measurement in IDLE/INACTIVE. (14/14)

Proposal5: Support RAT-Independent positioning in RRC_IDLE/INACTIVE. FFS the procedures that can be supported. (13/14)

Then, a further continuation of the email discussion is triggered during online to further polish on the text proposal based on the agreements.

[AT113-e][609][POS] Continued discussion of positioning in idle/inactive (Huawei)

Scope: Continue discussion of the issues from R2-2101230, and converge to an agreeable TP, taking as a baseline the principle that positioning in inactive is supported as recommended by RAN1.  R2-2101229 to be taken into account.

Intended outcome: Endorsable TP, in R2-2102100

Deadline:  Tuesday 2021-02-02 1200UTC – extended to 2021-02-04 0200 UTC to finalise the TP

In this discussion, we continue the discussion for the text proposal for the TR for positioning enhancement in R17

Contacts

| Company | Delegate name | Delegate email |
| --- | --- | --- |

Discussion

During RAN2#113e, the following agreement has been made on the downlink positioning in RRC_INACTIVE

Agreements:

Proposal2: RAN2 recommends the following for normative work for DL positioning

	The report of PRS measurement performed in RRC_IDLE/INACTIVE when the UE is in RRC_INACTIVE is supported (10/12)

	PRS measurement report and/or location estimate are sent from the UE to the gNB in RRC_INACTIVE.  RAN2 generally agree to do this by enhancing small data transmission in RRC_INACTIVE (details of the use of SDT to be studied in the WI phase) (15/16)

Proposal4: For DL positioning in IDLE/INACTIVE, the followings are already supported for the current spec and can be reused:

	Current stage3 spec has already supported assistance data delivery for DL positioning during RRC_CONNECTED and on-demand SI request in RRC_IDLE/ INACITVE for IDLE/INACTIVE positioning. (14/14)

	Current stage3 spec already supports the transfer of RequestLocationInformation in RRC_CONNECTED for PRS measurement in IDLE/INACTIVE. (14/14)

For positioning in RRC_INACTVE, we propose the following text proposal:

- 10.1 NR positioning for UEs in RRC_INACTIVE state NR positioning for UEs in RRC_INACTIVE state is recommended for normative work, including - DL, UL and DL+UL positioning methods - UE-based and UE-assisted positioning solutions - Support of UE positioning measurements for UEs in RRC_inactive state - Options that can be considered include DL-PRS or DL-PRS and SSB - Support of gNB positioning measurements for UEs in RRC_inactive state The details of how to enable the UE positioning in RRC_ INACTIVE state can be further discussed during normative work. These details may include, but are not limited to the following aspects: - UL reference signals (e.g., SRS for positioning, PRACH preambles) for UL measurements - Signalling and procedures for support the assistance data delivery, DL-PRS configuration, UL reference signals for positioning resource configuration, measurement reporting, which may be developed based on the enhancements of existing signalling and procedures (e.g., existing 2-step and/or 4-step PRACH procedures, paging procedure, small data transmission). The following procedures are recommended for normative work for DL positioning methods in RRC_INACTIVE: - Reporting of PRS measurement and/or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE. - The reporting of PRS measurement and/or location estimate performed in RRC_IDLE/RRC_INACTIVE when the UE is in RRC_INACTIVE is enabled by enhancing small data transmission in RRC_INACTIVE. NOTE: The following procedures are considered to have already been supported and can be reused for DL positioning in RRC_INACTIVE - On-demand SI request in RRC_INACTIVE for assistance data delivery by broadcast in RRC_INACTIVE - ProvideAssistanceData in RRC_CONNECTED for PRS configuration in RRC_INACTIVE downlink positioning - RequestLocationInformation can be sent in RRC_CONNECTED for PRS measurement or location estimate in RRC_INACTIVE

Question1: Do companies think the above text proposal has faithfully captured the agreement made during online for RRC_INACTIVE positioning?

| Company | Yes/No | Comments |
| --- | --- | --- |

For positioning in RRC_IDLE, we propose the following text proposal:

- 10.9 DL positioning measurement in RRC_IDLE state From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state. - Note: This does not imply that measurements have to be reported in RRC_IDLE state. The following procedures are considered as feasible for DL positioning methods in RRC_IDLE: - Reporting of PRS measurement and/or location estimate performed in RRC_IDLE when the UE is in RRC_CONNETED. NOTE: The following procedures are considered to have already been supported and can be reused for positioning in RRC_IDLE - On-demand SI request in RRC_IDLE for assistance data delivery by broadcast in RRC_IDLE - ProvideAssistanceData can be sent in RRC_CONNECTED for PRS configuration in RRC_IDLE downlink positioning - RequestLocationInformation can be sent in RRC_CONNECTED for PRS measurement and/or location estimate in RRC_IDLE

Question2: Do companies think the above text proposal has faithfully captured the agreement during online for RRC_IDLE positioning?

| Company | Yes/No | Comments |
| --- | --- | --- |

Then, for RAT-independent positioning, during the online discussion, the following agreements have been made:

Agreements:

Proposal5: Support RAT-Independent positioning in RRC_IDLE/INACTIVE. FFS the procedures that can be supported. (13/14)

Then, we propose the following text proposal for the above agreement:

10.y RAT-Independent positioning RAT-Independent positioning in RRC_IDLE/INACTIVE is recommended for normative work. The exact procedures that can be supported for RAT-Independent positioning in RRC_IDLE/INACTVE can be further studied.

Question3: Do companies think the above text proposal has faithfully captured the agreement during online for RAT-independent positioning?

| Company | Yes/No | Comments |
| --- | --- | --- |

During the first phase of [609], we have discussed on the definition of IDLE/INACTIVE positioning, with the following text proposal

- 2 References The following documents contain provisions which, through reference in this text, constitute provisions of the present document. - References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific. - For a specific reference, subsequent revisions do not apply. - For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document. [1] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications". [2] RP-193237: "new SID on NR Positioning Enhancements". [3] 3GPP TR 38.855: "Study on NR Positioning (Release 16)". [4] R1-2009433 Evaluation results for Rel-16 positioning and Rel-17 enhancement Huawei, HiSilicon [5] R1-2007665 Evaluation of NR positioning performance vivo [6] R1-2007720 Evaluation of achievable positioning accuracy BUPT [7] R1-2007754 Evaluation of achievable accuracy and latency ZTE [8] R1-2007859 Discussion of evaluation of NR positioning performance CATT [9] R1-2007908 NLOS Identification and Mitigation FUTUREWEI [10] R1-2009390 Update of Evaluation Results for NR Positioning Performance in I-IoT Scenarios Intel Corporation [11] R1-2007997 NR Positioning Latency Evaluations Lenovo, Motorola Mobility [12] R1-2008225 Evaluation of NR positioning in IIOT scenario OPPO [13] R1-2009555 Results on evaluation of achievable positioning accuracy and latency Nokia, Nokia Shanghai Bell [14] R1-2009502 Discussion on Performance evaluation of Rel-17 positioning Sony [15] R1-2008416 Discussions on evaluation of achievable positioning accuracy and latency for NR positioning LG Electronics [16] R1-2008489 Evaluation of achievable positioning latency InterDigital, Inc. [17] R1-2009708 Evaluation of achievable Positioning Accuracy & Latency Qualcomm Incorporated [18] R1-2009428 Evaluation of positioning enhancements Fraunhofer IIS, Fraunhofer HHI [19] R1-2008720 Positioning evaluation results on potential enhancements for additional use cases CeWiT [20] R1-2008764 Evaluation of achievable positioning accuracy and latency Ericsson [21] R1-2008765 Potential positioning enhancements Ericsson [22] R1-2007666 Discussion on potential positioning enhancements vivo [23] R1-2005380 Evaluation of achievable positioning accuracy and latency vivo [24] 3GPP TS 22.261 Service requirements for the 5G system; Stage 1 (Release 17) [25] RP-202094 Revised SID: Study on NR Positioning Enhancements CATT, Intel Corporation [26] 3GPP TS 38.901 Study on channel model for frequencies from 0.5 to 100 GHz (Release 16) [xx] 3GPP TS 24.571 Control plane Location Services (LCS) procedures (Release 16) ===================================NEXT CHANGE=================================== - 10.x Scope of RRC_IDLE/INACTIVE positioning The following UE positioning procedures are under the scope of RRC_IDLE/RRC_INACTIVE positioning if any of them are performed when the UE is in RRC_IDLE/RRC_INACTIVE. - NAS-transported positioning signalling - LCS messages defined in Clause 4.1.2 for location services in TS 24.571 [xx] - LPP signaling for positioning (e.g., Capability transfer, Assistance data transfer, Location information transfer) - NRPPa - E-CID information transfer (UE-associated) - Positioning information transfer (UE-associated) - Measurement information transfer (non-UE-associated) - Uu Signaling and procedure - RRC signaling for positioning (e.g., posSRS configuration) - MAC procedure/L1 signaling (e.g., activation/deactivation for semi-persistent/aperiodic posSRS) - Transmission of UL-PRS and reception of DL-PRS - Reception for assistance information broadcast ====================================END OF CHANGE================================

The main difference with the first version of the text proposal [1] is that a separate section 7.x was created to capture the scope of discussion for IDLE/INACTIVE positioning.

Question4: Do companies think the current structure of the text proposal is OK?

| Company | Yes/No | Comments |
| --- | --- | --- |

Text proposal history

[1] Version 1: R2-2101229	TP for IDLE and INACTIVE postiioning	Huawei, HiSilicon	discussion	Rel-17	FS_NR_pos_enh

[2] Version 2: R2-2102100	(TP from [609])	Huawei, HiSilicon	discussion	Rel-17	FS_NR_pos_enh