3GPP TSG-RAN WG2 Meeting #113-e	R2-21xxxxx

Online, 25th Jan – 5th Feb 2021

Agenda item:	6.4.2

Source:	vivo (Rapporteur)

Title:	[Offline-702] [V2X] T400 expiry in timer table and protection of RRC message

Document for:	Discussion and Agreement

# 1	Introduction

This is to report the result of the following email discussion at RAN2#113-e meeting.

[AT113-e][702][V2X/SL] T400 expiry in timer table and protection of RRC messages (vivo)

Scope: discuss the corrections in R2-2101761, R2-2100788, R2-2100978, R2-2100790, R2-2100976, R2-2101760 and R2-2100977. Normative text may also need to be updated if adds some additional/different UE behaviour at T400 expiry. Merge the changes and prepare the agreeable CR.

Intended outcome: agreeable 38.331 CR in R2-2102175 and discussion summary in R2-2102176 (if needed).

Deadline: Feb 04 0430 (UTC)

The document consists of phase-1 and phase-2, the deadline of each phase is outlined as follow:

- Phase 1: determine agreeable parts, deadline: Monday Feb. 1, 2021, 11:00 UTC.

- Phase 2: for agreeable parts Work on CRs, deadline: Thursday Feb. 4, 2021, 02:30 UTC

# 2	Contact Information

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provide input to provide their contact information in this table:

| Company | Contact: Name (E-mail) |
| --- | --- |
| vivo | Kimba Dit Adamou, Boubacar (kimba@vivo.com) |
| OPPO | Qianxi Lu (qianxi.lu@oppo.com) |
| Apple | Zhibin Wu (zhibin_wu@Apple.com) |
| Nokia | Buthler, Jakob (Jakob.buthler@nokia.com) |
| Samsung | Hyunjeong Kang (hyunjeong.kang@samsung.com) |
| Ericsson | Antonino Orsino (antonino.orsino@ericsson.com) |
| Intel | Ansab Ali (ansab.ali@intel.com) |
| Spreadtrum | Xing Liu (xing.liu1@unisoc.com |
| LG | Giwon Park (giwon.park@lge.com) |
| Huawei | Li Zhao (zhaoli8@huawei.com) |
| CATT | Hao Xu(xuhao@catt.cn) |
| Xiaomi | Yang Xing(yangxing1@xiaomi.com) |
| Qualcomm | Dan Vassilovski (dvassilo@qti.qualcomm.com) |
| MediaTek | Nathan Tenny (nathan.tenny@mediatek.com) |

# 3	Discussion

## 3.1	T400 expiry in timer table (TS 38.331)

According to current specifcation TS 38.331 V16.3.1, in the informative table of section 7.1.1 for the timer T400, it is stated that when the timer expires, the sidelink reconfiguration failure procedure need to be performed, highlighed in yellow as below.

***********************************************From TS 38.331**********************************************************

### 7.1.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T400 | Upon transmission of RRCReconfigurationSidelink | Upon reception of RRCReconfigurationFailureSidelink or RRCReconfigurationCompleteSidelink | Perform the sidelink RRC reconfiguration failure procedure as specified in 5.8.9.1.8 |

***********************************************From TS 38.331**********************************************************

However, in the procedural text of section 5.8.9.3, the UE behaviour upon T400 expiry is treated as SL RLF and the Sidelink radio link failure related actions need to be performed, highlighted in yellow as below.

***********************************************From TS 38.331**********************************************************

#### 5.8.9.3	Sidelink radio link failure related actions

The UE shall:

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry for a specific destination; or

1>	upon indication from sidelink MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination:

2>	consider sidelink radio link failure to be detected for this destination;

2>	release the DRBs of this destination, in according to sub-clause 5.8.9.1a.1;

2>	release the SRBs of this destination, in according to sub-clause 5.8.9.1a.3;

2>	discard the NR sidelink communication related configuration of this destination;

2>	reset the sidelink specific MAC of this destination;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;

NOTE:	It is up to UE implementation on whether and how to indicate to upper layers to maintain the keep-alive procedure [55].

***********************************************From TS 38.331**********************************************************

As a consequence, the informative table for T400 expiry is inconsistent with the procedural text. It is noticeable that whether to consider T400 expiry as SL reconfiguration failure or SL RLF was discussed at RAN2#109-e meeting. At that meeting, RAN2 has made the following agreement to treat T400 expiry the same as SL RLF, highlighted in green as below.

RAN2#109-e Agreements on RRC:

Upon T400 expiry, TX UE detects PC5-RRC RLF and performs the same operations as RLF.

However, according to company CRs in [1][2][3], the UE behaviour upon T400 expiry may not be exactly the same operations as SL RLF. There are three Alternatives on the table as follows:

- Alt- 1:	Continue using the configuration used prior to corresponding RRCReconfigurationSidelink message and 	perform the sidelink UE information for NR sidelink communication procedure as specified in 5.8.3.3 if UE is in RRC_CONNECTED. (Huawei CR in R2-2101761).

- Alt- 2:	Perform the Sidelink radio link failure related actions as specified in 5.8.9.3. (vivo CR in R2-2100788).

- Alt- 3:	Continue using the configuration used prior to corresponding RRCReconfigurationSidelink message and perform the radio link failure related actions as specified in clause 5.8.9.3. (Ericsson CR in R2-2100978).

Rapporteur would like to invite interested companies to check which is the correct understanding of the UE behaviour upon T400 expiry and see if there is some additional/different UE behavior needed.

Q1-1: Do companies confirm that upon T400 expiry, at least the UE will perform the same operations as SL RLF?

- YES;

- NO.

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | Thanks rapporteur reminding the agreement @ #109, then better to align with it and keep the procedural text and thus update the text for T400 expiry in the table only. |
| Apple | Yes |  |
| Nokia | Yes with comment | We agree in principle, but also want to note that as the expiry of timer T400 is considered as a sidelink RLF, it will further lead to the release of that PC5 connection the expiry of T400 timer is associated with. In case the UE is in mode 1 and it can not obtain a NW-granted resource on time for transmitting the RRCReconfigurationSidelink message, it may lead to the expiry of T400 timer. In that case, the PC5 connection can be released, which is not due to PC5 radio link problem. Thus, if the UE cannot obtain a SL resource for transmitting the RRCReconfigurationSidelink message on time, e.g. there is a Uu RLF and no exceptional resource pool is configured, we think the UE may consider to hold T400 until the Uu problem is resolved. Furthermore as there are multiple T400 timers we suggest to modify the wording in the proposed/revised 5.8.9.3 to “destination ID” |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes |  |
| Spreadtrum | Yes |  |
| LG | Yes |  |
| HW | Yes | If the majority would like to keep the agreement, we are fine to follow. |
| CATT | Yes |  |
| Xiaomi | Yes |  |
| Qualcomm | Yes |  |
| vivo | Yes |  |
| MediaTek | Yes |  |
| ZTE | Yes | We shall follow the agreement in RAN2#109e that upon T400 expiry, UE shall consider it as SL RLF and perform the same operations as SL RLF. |
| Lenovo | Yes |  |

Summary:

16companies provide input to this question.

All companies agree that upon T400 expiry, at least the UE will perform the same operations as SL RLF

Q1-2: If the ANS to Q1-1 is YES, any additional UE behaviour is needed on top of the SL RLF related operations?

- None;

- Continue using the configuration used prior to corresponding RRCReconfigurationSidelink message;

- Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | 1 | As replied to Q1-1. |
| Apple | 2 | For this issue, we agree with the change in Ericsson CR R2-2100978 |
| Nokia | 2) and 3) | For option3: UE in RRC_CONNECTED can report the Sidelink Reconfiguration Failure to network |
| Samsung | 1 | As agreed in #109-e, UE can perform the procedures in 5.8.9.3 |
| Ericsson | 2 | Proponent |
| Intel | 2) and 3) | Agree with Nokia |
| Spreadtrum | 1 | We should stick with the prior agreement and we do not think any other action is needed. |
| LG | 2 | Agree with change in Ericsson CR R2-100978 |
| HW | 1 |  |
| CATT | 1 |  |
| Xiaomi | 1 |  |
| Qualcomm | 1 | Agree with Samsung’s comment |
| vivo | 1 | According to 5.8.9.3 Sidelink radio link failure related actions, the UE shall: 2> discard the NR sidelink communication related configuration of this destination; It is meaningless for the fallback using the configuration used prior to corresponding RRCReconfigurationSidelink message. |
| MediaTek | 1 |  |
| ZTE | 1 | None additional UE behaviour is need. Since T400 expiry regarded as SL RLF, RRC connection and all associated configuration will be released. It doesn’t make sense that UE continues using the released configuration used prior to corresponding RRCReconfigurationSidelink message. |
| Lenovo | 1 |  |

Summary:

16 companies provide input to this question.

11 companies think that upon T400 expiry, no additional UE behaviour is needed on top of the SL RLF related operations

4 companies think that upon T400 expiry, on top of the SL RLF related operations, UE can continue using the configuration used prior to corresponding RRCReconfigurationSidelink message. Among those companies, 2 companies also think that UE behaviour can also include: UE in RRC_CONNECTED can report the Sidelink Reconfiguration Failure to network

Q1-3: If the ANS to Q1-1 is NO, it means that the UE will perform some behaviour that can be different from the SL RLF related operations. Then what is the correct UE behaviour that needs to be specified?

- Continue using the configuration used prior to corresponding RRCReconfigurationSidelink message and 	perform the sidelink UE information for NR sidelink communication procedure as specified in 5.8.3.3 if UE is in RRC_CONNECTED;

- Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |

Proposal1: Upon T400 expiry, UE will perform the same operations as SL RLF and no additional behaviour is needed.

## 3.2	Protection of NR RRC messages

According to current specifcation TS 38.331 V16.3.1, in the Annex B.1 for protection of RRC messages, the security requirement for sending SidelinkUEInformationNR and ULInformationTransferIRAT messages are missing. This issue was discussed in [AT112-e][705][V2X], but was postponed as a left-over issue to this meeting.

## SidelinkUEInformationNR

Based on company CRs in [1][4][5], there is consensus that SidelinkUEInformationNR message shall not be sent unprotected after AS security activation. However, regarding whether SidelinkUEInformationNR message can be sent unprotected before AS security activation, company views diverge much.

In [1], the basic idea is that the security requirement defined for sending SidelinkUEInformation message in LTE V2X is reused in NR, i.e., SidelinkUEInformationNR message can be sent unprotected prior to AS security activation.

In [4], it is proposed not to follow the same principle in LTE V2X and apply the enhanced security requirement, i.e., SidelinkUEInformationNR message shall never be sent unprotected prior to AS security activation. The main motivation is that the only chance for sending SidelinkUEInformationNR message unprotected is in the initial phase of RRC connection establishment before security activation which may be a rare case. Furthermore, the security requirement in NR Uu for some other RRC messages over SRB1 has already been enhanced, e.g., RRCReestablishment message is sent over SRB1 with integrity protection instead of SRB0 in LTE. Another example is that RRCResume message during resuming from RRC_INACTIVE is sent over SRB1 with both integrity and ciphering protection. Frome these perpectives, it is better to apply the enhanced security requirement to SidelinkUEInformationNR message which is also sent over SRB1.

In [5],  some rules in between are considered, i.e., the SidelinkUEInformationNR message can be sent unprotected prior to AS security activation but with some conditions.

- Firstly, consider different  RRC states transition procedures. The SidelinkUEInformationNR message may be sent unprotectd before AS security activation if the UE transites from RRC_IDLE to RRC_CONNECTED, but shall not be sent unprotected before AS security activation if the UE transits from RRC_INACTIVE to RRC_CONNECTED or if the UE is in RRC_CONNECTED.

- Secondly, depend on the information that is carried in the SidelinkUEInformationNR message. One of the main differences between LTE and NR V2X is that SidelinkUEInformationNR message can be used to report the sidelink UE capability information of the associated peer UE for unicast communication. Since the UE capabilities are sensitve information, in order to protect privacy of the UE, the SidelinkUEInformationNR message shall not be sent unprotected before AS security activation if the sidelink UE capability information is included in the SidelinkUEInformationNR message.

Based on above observations, rapporteur would like to invite interested companies to check whether or not SidelinkUEInformationNR message can be sent unprotected prior to AS security activation and see if there is some condition that needs to be specified.

Q2-1: Do companies agree that SidelinkUEInformationNR message can be sent unprotected prior to AS security activation?

- YES;

- NO (i.e., SidelinkUEInformationNR message shall never be sent unprotected prior to AS security activation).

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Apple | No | There is no need to always inherit LTE D2D decisions. We think the protection of SUI message is needed to protect user privacy in V2X applications. |
| Nokia | No | We also support protection of SUI messags, but we would also support i.e. if compromise is to ask SA3 |
| Samsung | No | We think the SUI need to be protected. |
| Ericsson | No | We agree that, given the information included in the SUI during NR SL in Rel-16, the best option would be to always send it protected. |
| Intel |  | We are fine to agree to this if this is the majority view |
| Spreadtrum | No |  |
| LG | No |  |
| HW | Yes |  |
| CATT | Yes | In our understanding, the RRCReconfiguration including the SLRB configurations should be sent after AS security activation similar as Uu. But there is no need to restrict the sidelinkUEInformationNR can only be sent after AS security activation. |
| Xiaomi | No | We don’t see the need to send SUI early, since UE could use exceptional pool until reception of dedicated configuration. It’s safer to support message in protected way. |
| Qualcomm | No |  |
| vivo | No |  |
| MediaTek | Yes (with comment) | We tend to think we should have a security reason for changing security requirements. We would be fine to ask SA3 for guidance in understanding if there is a different security requirement here as compared to LTE. |
| ZTE | Yes | The SidelinkUEInformationNR message can be sent unprotected in the initial phase of RRC connection establishment before AS security activation. |
| Lenovo | Yes | we do not see much differences of content between SUI in LTE and SUINR, except UE capability information. Since UE capability information itself can be sent unprotected, we fail to see strong motivation to increase the security protection and must send SUINR protected. Follow LTE rules is enough. But we also agree to ask SA3 opinion on this issue if RAN2 is determined to do so |

Summary:

16 companies provide input to this question.

6 companies think that SidelinkUEInformationNR message can be sent unprotected prior to AS security activation

One company is fine to agree to this if it is the majority view

9 companies think that SidelinkUEInformationNR message shall never be sent unprotected prior to AS security activation

Q2-2: If the ANS to Q2-1 is YES, any condition that needs to be specified for the case when SidelinkUEInformationNR message shall not be sent unprotected prior to AS security activation?

- None (i.e., follow LTE V2X);

- If the UE transits from RRC_INACTIVE to RRC_CONNECTED or if the UE is in RRC_CONNECTED; and if the sidelink UE capability information is included in SidelinkUEInformationNR message;

- Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | 1 | For “if the sidelink UE capability information is included in SidelinkUEInformationNR message”: even UECapabilityInformation itself can be sent unprotected, so not sure why SUI has to be restricted. For “if the UE is in RRC_CONNECTED”, not sure what is the restriction, does it mean that after the reception of RRCResume or RRCSetup? But then it is contradictory to the intention as described by rapporteur that “The SidelinkUEInformationNR message may be sent unprotectd before AS security activation if the UE transites from RRC_IDLE to RRC_CONNECTED” which also include the reception of RRCSetup. For “transits from RRC_INACTIVE to RRC_CONNECTED”, we are fine with the intention, but our understanding that is the phase “after AS security activation”, while “P” is for “Messages that can be sent (unprotected) prior to AS security activation”, i.e., by not marking it as “A-I” or “A-C”, we already prevent the UE to send SUI during “transits from RRC_INACTIVE to RRC_CONNECTED”. |
| Ericsson | 2 (if SUI can be sent unprotected) | Our preference is to send the SUI message always protected. If this is not the common understanding, we prefer option 2. In reply to OPPO’s comment: For “transit from RRC_INACTIVE to RRC_CONNECTED” this is needed as the security is already provided by the network in the RRCRelease message and thus when the UE triggers the resume the AS security is already activated. For “UE in RRC_CONNECTED”, this is also needed as the UE may establish a PC5 connection while is Uu RRC status is already RRC_CONNECTED. In such a case, the AS security is already established. For “if the sidelink UE capability information is included in SidelinkUEInformationNR message”, this is also needed as we agreed that a UE may forward capabilities of a peer UE to the network. In such a case, disclosing capabilities of another UE may also cause privacy issue and thus it is essential to not send the SUI unprotected. |
| HW | 1 |  |
| CATT | 1 |  |
| vivo | 1 |  |
| MediaTek | 1 | Agree with OPPO’s analysis with the following comments: - For UE capability included: We aren’t sure why revealing the capabilities of a peer UE is worse than the UE revealing its own capabilities. This might be a point to clarify with SA3, since this seems to be the major difference from the LTE case. - For UE in RRC_CONNECTED: Ericsson’s response here is a bit confusing. We understand there is agreement that the message cannot be sent unprotected *after* AS security activation, so the case referred to by Ericsson, where AS security is already established, seems not in scope of the question. - For transit from RRC_INACTIVE to RRC_CONNECTED: We agree with OPPO’s reading, and Ericsson’s comment seems to be saying the same thing (“when the UE triggers the resume the AS security is already activated”). So we understand that there is no actual issue here as this is part of the “after security activation” case. |
| ZTE | 1 | We have the same concern as OPPO indicated about these conditions in bullet 2. |
| Lenovo | 1 |  |

Proposal 2: SidelinkUEInformationNR message shall never be sent unprotected prior to AS security activation

## ULInformationTransferIRAT

Based on company CRs in [1][4][5], there is also consensus that ULInformationTransferIRAT message shall not be sent unprotected after AS security activation. However, regarding whether ULInformationTransferIRAT message can be sent unprotected before AS security activation, there are mainly two kinds of views.

In [1] [5], although the two CRs are provided with different wording, rapporteur understands that they share a common view to consider the same security requirement according to the specific E-UTRA RRC messages carried in ULInformationTransferIRAT message. To be more specific, ULInformationTransferIRAT message is used for the cross-RAT control scenario (NR Uu control LTE SL) and this message carries V2X sidelink communication related RRC messages (MeasurementReport, UEAssistanceInformation and SidelinkUEInformation). Therefore, ULInformationTransferIRAT follows the same security requirement defined in TS 36.331 as E-UTRA RRC MeasurementReport message when it is used to transfer the MeasurementReport, the same security requirement as E-UTRA RRC UEAssistanceInformation message when transfer the UEAssistanceInformation and the same security requirement as E-UTRA RRC SidelinkUEInformation message when transfer the SidelinkUEInformation.

In [4], considering ULInformationTransferIRAT is sent on NR SRB1 it is proposed to simply follow the enhanced security requirement as defined for SidelinkUEInformationNR, i.e., make it not being able to be sent unprotected prior to AS security activation in any cases.

Based on above observations, rapporteur would like to invite interested companies to check whether or not ULInformationTransferIRAT message can be sent unprotected prior to AS security activation and see if there is some condition that needs to be specified.

Q3-1: Do companies agree that the NR ULInformationTransferIRAT message can be sent unprotected prior to AS security activation?

- YES;

- NO (i.e., the NR ULInformationTransferIRAT message shall never be sent unprotected prior to AS security activation).

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Apple | No | For the same reason as in Q2-1, we support the enhanced protection for this message. |
| Nokia | No | Same as in Q2-1 |
| Samsung | No | Same as in Q2-1 |
| Ericsson | No | We also think that this message should always be send protected. |
| Intel |  | Same view as for SUI, i.e. we can follow majority view on this |
| Spreadtrum | No |  |
| LG | No |  |
| HW | Yes |  |
| CATT | Yes |  |
| Xiaomi | No | Same as in Q2-1 |
| Qualcomm | No |  |
| vivo | No |  |
| MediaTek | Yes | We don’t see a strong reason to put a tighter restriction on this message than on the messages it contains. |
| ZTE | Yes | It shall apply the same security requirement as the specific E-UTRA RRC message included in it. Specifically, if the NR ULInformationTransferIRAT includes E-UTRA SidelinkUEInformation message, it can be sent unprotected prior to AS security activation. |
| Lenovo | Yes | Such message can be sent unprotected if carry LTE SUI message |

Summary:

16 companies provide input to this question.

6 companies agree that the NR ULInformationTransferIRAT message can be sent unprotected prior to AS security activation

9 companies think that the NR ULInformationTransferIRAT message shall never be sent unprotected prior to AS security activation

One company agree to follow majority view on this issue.

Q3-2: If the ANS to Q2-1 is YES, any condition that needs to be specified for the case when the NR ULInformationTransferIRAT message shall not be sent unprotected prior to AS security activation?

- Apply the same security requirement defined in TS 36.331 according to the specific E-UTRA RRC messages (MeasurementReport, UEAssistanceInformation and SidelinkUEInformation) carried in the NR ULInformationTransferIRAT message;

- Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | 1 |  |
| Ericsson | 1 (only if can be sent unprotected) | Our preference is to send the message always protected. If this is not the common understanding, we prefer option 1. |
| HW | 1 |  |
| CATT | 1 |  |
| MediaTek | 1 |  |
| ZTE | 1 with comment | For the detailed CR, we prefer to use a new “NOTE 2” similar as “NOTE 1” in the table for “DLDedicatedMessageSegment” to clarify the conditions rather than in the “comment” column. For example: NOTE 2: This message type carries E-UTRA RRC messages (MeasurementReport, SidelinkUEInformation and UEAssistanceInformation) as defined in TS 36.331 [10]. The protection of an instance of this message is the same as for the message which this message is carrying. |
| Lenovo | 1 |  |

Proposal 3: the NR ULInformationTransferIRAT message shall never be sent unprotected prior to AS security activation

## 3.3	Protection of E-UTRA RRC messages

According to current specifcation TS 36.331 V16.3.0, in the Annex A.6 for protection of RRC messages, the security requirement for sending ULInformationTransferIRAT message is missing and needs to be defined. The security requiremnt for sending SidelinkUEInformation may also need some clarificaiton case by case.This issue was discussed in [AT112-e][705][V2X], but was postponed as a left-over issue to this meeting.

## SidelinkUEInformation

In [6],  the CR proposes to make some clarification on the case when the SidelinkUEInformation message shall not be sent unprotected prior to AS security activation. Rapporteur  understands that the CR is trying to address different RRC states transition procedures, and further clarify that even though SidelinkUEInformation message can be sent unprotected before AS security activation, but shall not be sent unprotected before AS security activation if the UE transits from RRC_INACTIVE to RRC_CONNECTED or if the UE is in RRC_CONNECTED. Rapporteur would like to invite interested companies to share your view on the need of such clarification.

Q4: Any condition that needs to be specified for the case when SidelinkUEInformation message shall not be sent unprotected prior to AS security activation?

- None (i.e., without change to current specification)

- If the UE transits from RRC_INACTIVE to RRC_CONNECTED or if the UE is in RRC_CONNECTED

- UE shall not send this message unprotected. UE only send this message after AS security activation

- Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | 1 |  |
| Apple | 3 |  |
| Nokia | 3 |  |
| Samsung | 3 |  |
| Ericsson | 3 | Our preference is to send the SUI message always protected. If this is not the company understanding, we are okay to go for option 2. |
| Intel | 3 |  |
| Spreadtrum | 3 |  |
| LG | 3 |  |
| HW | 1 |  |
| CATT | 1 |  |
| Xiaomi | 3 |  |
| Qualcomm | 3 |  |
| vivo | 3 | The benefit is that we can have a unified security solution for Sidelink UE Information transfer in LTE and NR. |
| MediaTek | 1 |  |
| ZTE | 1 with comment | We think this Question is intended about E-UTRA SidelinkUEInformation not SidelinkUEInformationNR. |
| Lenovo | 1 |  |

Summary:

16 companies provide input to this question.

6 companies agree that no additional condition tneeds to be specified for the case when SidelinkUEInformation message shall not be sent unprotected prior to AS security activation

10 companies agree that UE shall not send SidelinkUEInformation message unprotected. UE only send this message after AS security activation

Proposal 4: UE shall not send SidelinkUEInformation message unprotected. UE only send this message after AS security activation

## ULInformationTransferIRAT

Similar to NR ULInformationTransferIRAT message, the E-UTRA ULInformationTransferIRAT message is used for the cross-RAT control scenario (LTE Uu control NR SL) and this message carries NR sidelink communication related RRC messages (MeasurementReport, UEAssistanceInformation and SidelinkUEInformationNR). Therefore, it is proposed to consider the same security requirement according to the specific NR RRC messages carried in the E-UTRA ULInformationTransferIRAT message in [6][7]. This security pripincle is similar as proposed in TS 38.331. Thus, similar quesitons are designed for the E-UTRA ULInformationTransferIRAT message.

Q5-1: Do companies agree that the E-UTRA ULInformationTransferIRAT message can be sent unprotected prior to AS security activation?

- YES;

- NO (i.e., the E-UTRA ULInformationTransferIRAT message shall never be sent unprotected prior to AS security activation).

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | 1 |  |
| Apple | NO |  |
| Nokia | No |  |
| Samsung | No |  |
| Ericsson | No |  |
| Intel | No |  |
| Spreadtrum | No |  |
| LG | No |  |
| HW | Yes |  |
| CATT | Yes |  |
| Xiaomi | No |  |
| Qualcomm | No |  |
| vivo | No |  |
| MediaTek | Yes | We think there isn’t a clear reason why the container message should have a more strict security requirement than the message inside it. |
| ZTE | Yes | It shall apply the same security requirement as the specific NR RRC message included in it. Specifically, if the E-UTRA ULInformationTransferIRAT includes SidelinkUEInformationNR message, it can be sent unprotected prior to AS security activation. |
| Lenovo | Yes |  |

Summary:

16 companies provide input to this question.

6 companies agree that that the E-UTRA ULInformationTransferIRAT message can be sent unprotected prior to AS security activation

10 companies think that the E-UTRA ULInformationTransferIRAT message shall never be sent unprotected prior to AS security activation

Q5-2: If the ANS to Q2-1 is YES, any condition that needs to be specified for the case when the E-URTA ULInformationTransferIRAT message shall not be sent unprotected prior to AS security activation?

- Apply the same security requirement defined in TS 38.331 according to the specific NR RRC messages (MeasurementReport, UEAssistanceInformation and SidelinkUEInformationNR) carried in the E-UTRA ULInformationTransferIRAT message;

- Others, please specify.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | 1 |  |
| Ericsson | 1 (only if can be sent unprotected) | Our preference is to send the SUI message always protected. If this is not the company understanding, we are okay to go for option 2. |
| HW | 1 |  |
| CATT | 1 |  |
| MediaTek | 1 |  |
| ZTE | 1 with comment | Same comment as Q3-2. For the detailed CR, we prefer to use a new “NOTE 2” similar as “NOTE 1” in the table for “DLDedicatedMessageSegment” to clarify the conditions rather than in the “comment” column. For example: NOTE 2: This message type carries NR RRC messages (MeasurementReport, SidelinkUEInformationNR and UEAssistanceInformation) as defined in TS 38.331 [82]. The protection of an instance of this message is the same as for the message which this message is carrying. |
| Lenovo | 1 |

Proposal 5: the E-UTRA ULInformationTransferIRAT message shall never be sent unprotected prior to AS security activation

# 4	Conclusion

This is email discussion is summarized with the following proposals:

Proposal1: Upon T400 expiry, UE will perform the same operations as SL RLF and no additional behaviour is needed.

Proposal 2: SidelinkUEInformationNR message shall never be sent unprotected prior to AS security activation

Proposal 3: the NR ULInformationTransferIRAT message shall never be sent unprotected prior to AS security activation

Proposal 4: UE shall not send SidelinkUEInformation message unprotected. UE only send this message after AS security activation

Proposal 5: the E-UTRA ULInformationTransferIRAT message shall never be sent unprotected prior to AS security activation

# 5	References

- R2-2101761	Miscellaneous corrections on TS 38.331 (Rapportuer CR)	Huawei, Hisilicon	CR	Rel-16	38.331	16.3.1	2437	-	F	5G_V2X_NRSL-Core

- R2-2100788	Correction on T400 expiry behavior		vivo	CR	Rel-16	38.331	16.3.1	2357	-	F	5G_V2X_NRSL-Core

- R2-2100978	Corrections regarding sidelink impacting NR	Ericsson	CR	Rel-16	38.331	16.3.1	2373	-	F	5G_V2X_NRSL-Core

- R2-2100790	Message protection for NR Sidelink	vivo	discussion

- R2-2100976	Protection of sidelinkUEInformation and ULInformationTrasferIRAT	Ericsson	CR	Rel-16	38.331	16.3.1	2372	-	F	5G_V2X_NRSL-Core

- R2-2100977	Protection of sidelinkUEInformation and ULInformationTrasferIRAT	Ericsson	CR	Rel-16	36.331	16.3.0	4558	-	F	5G_V2X_NRSL-Core

- R2-2101760	Miscellaneous corrections on TS 36.331 (Rapportuer CR)	Huawei, Hisilicon	CR	Rel-16	36.331	16.3.0	4591	-	F	5G_V2X_NRSL-Core