‎3GPP TSG-RAN WG2 Meeting #113 electronic											R2-210xxxx

Online, Jan 25 – Feb 5, 2021‎

# Agenda item:	6.10.3

# Source:	CATT

# Title:	Report of [AT113-e][803][NR/R17 SON/MDT]  Editorial corrections of 38.331and 36.331 CR (CATT)

# Document for:	Discussion and Agreement

1	Introduction

This document captures the discussions and conclusions of the following email discussions as per the session chair guidance.

* [AT113-e][803][NR/R17 SON/MDT]  Editorial corrections of 38.331and 36.331 CR (CATT)

-     The discussion including R2-2100186, R2-2100188, R2-2100190, R2-2101847, R2-2101848, R2-2101938,R2-2101939.

-     Every change in these documents should be addressed with clear conclusion (i.e., either agreed or not pursued)

-     All the agreed changes will be merged into one CR.

Intended outcome: Agreeable CR

Deadline: Thursday 28/01/2021

Two-phase discussions are planned, i.e.,

- Phase 1 (Kick off of 1st phase – Jan. 26th, UTC 23:59pm): Companies are invited to share their comments if any on the changes proposed in the listed CRs. After Phase 1 the moderator will provide a update to the draft report, as well as merged draft CRs for review.

- Phase 2 (Kick off of 2nd phase- Jan. 28th, UTC 23:59pm): Companies are invited to provide their comments if any on the updated report and the merged draft CRs.

The remainder of this document is organized as the following. The discussions are in Section 2 and the conclusions are summaried in Section 3.

The participants are invited to leave their contact in the table below for earlier discussions.

| Company name | Participtiant’s name / Email address |
| --- | --- |
| CATT | Erlin Zeng / erlin.zeng@catt.cn |
| ZTE | Zhihong Qiu /qiu.zhihong@zte.com.cn |

2	Discussion

2.1	R2-2100186‎ Miscellaneous  corrections to TS 36.331 on SON and MDT

Phase 1

In this document the following changes are proposed to TS 36.331

- Change “i.e. RLF” to “i.e. MCG-RLF” in 5.3.11.3. - Change “last radio link or handover failure” to “last radio link failure or handover failure” in 5.6.5.3. - Add the case of setting visitedCellId to the physical cell identity and carrier frequency of the current cell in 5.6.5.3. - Change “InDeviceCoexDetected” to “inDeviceCoexDetected” in 5.6.8.2. - Change “carrierFrequency” to “carrierFreq” in 5.6.8.2.

Companies are invited to share their comments in the following table to the changes that are NOT agreeable or require updates. If no any comment received for a change, it means the change is agreeable to all.

Table 1

| Company Name | Which changes are not agreeable or requires updates | Comments if a change is not agreeable or requires updates |
| --- | --- | --- |
| ZTE | 1,2 | Without change of 1 and 2, the text is still clear. |

Summary from Phase 1 discussions

tbd

2.2	R2-2100188‎ Miscellaneous Corrections for SON and MDT in 38.331‎

Phase 1

In this document the following changes are proposed to TS 38.331

1. Change “bearerg” to “beager”. 2. Delete the sentence “The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected” in clause 5.3.10.3. 3. Add the the missing description of sensor-LocationInfo in clause 5.10.3.5. 4. Change “sensor-MeasurementInformation” to italic. 5. Change “wlan-Namelist” and “sensor-Namelist” to “wlan-NameList” and “sensor-NameList”. 6. Add “failure” follow “last radio link”. 7. Add “or the physical cell identity and carrier frequency” to identify visitedCellId. 8. Change “ra-InformationCommon-r16” to “ra-InformationCommon”. 9. Delete the “(” before rsrp-ThresholdSSB in dlRSRPAboveThreshold description. 10. Delete the description of field ul-DelayValueConfig included in IE EventTriggerConfig field descriptions. 11. Change “msgA-DataScramblingIndex” to “msgA-dataScramblingIdentity”.

Companies are invited to share their comments in the following table to the changes that are NOT agreeable ‎or require updates. If no any comment received for a change, it means the change is agreeable to all.  ‎

Table 2

| Company Name | Which changes are not agreeable or requires updates | Comments if a change is not agreeable or requires updates |
| --- | --- | --- |
| ZTE | 3 and 11 | We prefer to reference the setting of locationInfo to 5.3.3.7 instead of duplicating the description in 5.10.3.5 11 shall be discussed in 2step RA WID |

Summary from Phase 1 discussions

tbd

2.3	R2-2100190‎ Correction on RLF Report for Re-connection

Phase 1

In this document the following changes are proposed to TS 38.331

1) Move the action of setting re-connected cell related information out of the branch of “set the content of RRCSetupComplete message as follows”; 2) Take “cross-RAT RLF reporting” out of the condition of “UE supports RLF report for inter-RAT MRO NR”; 3) The relationship of the two conditions (NR rlf available or LTE rlf available) to set the rlf-InfoAvailable should be “or”;

Companies are invited to share their comments in the following table to the changes that are NOT agreeable ‎or require updates. If no any comment received for a change, it means the change is agreeable to all.  ‎

Table 2

| Company Name | Which changes are not agreeable or requires updates | Comments if a change is not agreeable or requires updates |
| --- | --- | --- |

Summary from Phase 1 discussions

tbd

2.4	R2-2101847‎‎ Corrections for SON&MDT Logging Capability ‎

Phase 1

In this document the following changes are proposed to TS 38.331

Add capability reference along with the corresponding SON&MDT optional features. ‎

Companies are invited to share their comments in the following table to the changes that are NOT agreeable ‎or require updates. If no any comment received for a change, it means the change is agreeable to all.  ‎

Table 2

| Company Name | Which changes are not agreeable or requires updates | Comments if a change is not agreeable or requires updates |
| --- | --- | --- |
| ZTE | Seem unnecessary | Current specs is clear. |
| CATT | seems not needed |  |

Summary from Phase 1 discussions

tbd

2.5	R2-2101848‎‎‎ Miscellaneous Corrections for SON&MDT ‎

Phase 1

In this document the following changes are proposed to TS 38.331

- In subclause 5.3.3.7, change ‘if the RPLMN is not equal to plmn-identity’ to ‘if the RPLMN is not equal to plmn-identity’. - In subclause 5.3.5.9, use field name instead of the definition name for BT/WLAN/Sensor measurements. - In subclause 5.3.7.4, change ‘handover failure’ to ‘reconfiguration with sync failure’. - In subclause 5.3.10.5, refer to 5.3.3.7 when setting locationInfo. - In subclause 5.5a.3.2, delete ‘detail’. - In subclause 5.7.3.3, change ‘synchReconfigFailureSCG’ to ‘synchReconfigFailureSCG’ and change ‘.’ to ‘;’. - In subclause 5.7.3.5, the same changes with previous one. - In subclause 5.7.10.4, add reference when setting selected PLMN and remove the ‘-r16’ from ‘ra-InformationCommon-r16’. - In subclause 7.4, change ‘The UE variable VarConnEstFailReport includes the connection establishment failure and connection resume failure information’ to ‘The UE variable VarConnEstFailReport includes the connection establishment failure and/or connection resume failure information’.

Companies are invited to share their comments in the following table to the changes that are NOT agreeable ‎or require updates. If no any comment received for a change, it means the change is agreeable to all.  ‎

Table 2

| Company Name | Which changes are not agreeable or requires updates | Comments if a change is not agreeable or requires updates |
| --- | --- | --- |

Summary from Phase 1 discussions

tbd

2.6	R2-2101938‎‎‎‎ Corrections for Cross-RAT RLF Report ‎

Phase 1

In this document the following changes are proposed to TS 38.331

In sub-clause 5.7.10.3, add the description for failedPCellId-EUTRA when setting eutra-RLF-Report in RLF-Report.

Companies are invited to share their comments in the following table to the changes that are NOT agreeable ‎or require updates. If no any comment received for a change, it means the change is agreeable to all.  ‎

Table 2

| Company Name | Which changes are not agreeable or requires updates | Comments if a change is not agreeable or requires updates |
| --- | --- | --- |
| ZTE | The CR is not needed. | This is handled in first online session. |

Summary from Phase 1 discussions

tbd

2.7	R2-2101939 Corrections for Sensor

Phase 1

In this document the following changes are proposed to TS 38.331

In sub-clause 5.3.5.9, Sensor should be taken into accout for NOTE 2.‎

Companies are invited to share their comments in the following table to the changes that are NOT agreeable ‎or require updates. If no any comment received for a change, it means the change is agreeable to all.  ‎

Table 2

| Company Name | Which changes are not agreeable or requires updates | Comments if a change is not agreeable or requires updates |
| --- | --- | --- |

Summary from Phase 1 discussions

tbd

3	Conclusion

This document captures the discussions and conclusions from email discussion #803 in R2-113-e. The conclusions are summarized in the following.

tbd