3GPP TSG-RAN WG2 Meeting #113-e		R2-2100222

Online, Jan 25 – Feb 5, 2021

Source:	CATT

Title:	Analysis on IIoT in Unlicensed Spectrum

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

During GTW online RAN2#112-e session discussing R17 Uplink enhancements for URLLC in unlicensed controlled environments, various agreements were achieved [1].

Agreements:

From RAN2 perspective

1 	It is assumed that LBT failures only happen infrequently in UCE (unlicensed controlled environment).  A formal definition of UCE and its relationship to semi-static or dynamic access mode is not necessary in RAN2 specifications.

2	cg-RetransmissionTimer can be configured optionally for shared spectrum

3	When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection.

4	When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection.

5	As a baseline, HARQ processes sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured as in Rel-16 NR-U.

6	HARQ processes sharing between multiple CGs are not allowed when cg-RetransmissionTimer is not configured.

7	FFS if LCH based prioritization can be configured with cg-RetransmissionTimer

8	The assumption for Rel-16 is that the network will not configure autonomousTx and cg-RetransmissionTimer simultaneously per cell.  No optimizations will be pursued to allow the two features be configured together in Rel-16.  No CR is needed for this for now.

9	If a configured grant is deprioritized and/or gNB didn’t get it (e.g. LBT failure and/or tx failure) then we should be able to autonomously re-transmit it.  FFS how to achieve it (using existing mechanisms should be considered as baseline)

Meanwhile, some conclusions were also agreed in RAN1 [2].

In this document, we address the remaining FFSs in RAN2 as well as the options provided by RAN1. Our analysis and preferences are also presented.

Discussion

There are essentially two FFSs after last meeting, listed below, which we address in Sections 2.1 and 2.2 respectively.

7. FFS if LCH based prioritization can be configured with cg-RetransmissionTimer 9. If a configured grant is deprioritized and/or gNB didn’t get it (e.g. LBT failure and/or tx failure) then we should be able to autonomously re-transmit it. FFS how to achieve it (using existing mechanisms should be considered as baseline)

- Whether lch-basedPrioritization can be configured with cg-RetransmissionTimer

This issue was discussed in questions 6-7 of offline 501 in previous meeting [3]. Strictly speaking, all companies agreed that LCH-based prioritization should be supported for Rel-17 URLLC in unlicensed controlled environment (UCE) in question 7. At least that seems obvious for prioritization among initial transmissions. But there were mixed views on whether such lch-basedPrioritization should apply to re-transmissions, as in IIOT or whether for re-transmissions the NR-U rule should apply, (question 6). However, even there, some majority of companies expressed the opinion that the IIOT prioritization should prevail over the NR-U prioritization in that case. We also share this view and don’t see any problem to apply the IIOT rule to both initial and re-transmissions in UCE. Thus, we propose that:

Proposal 1: Support lch-based-Prioritization configured with cg-RetransmsisionTimer.

Proposal 2: When lch-basedPrioritization is configured with cg-RetransmsisionTimer initial transmissions can have higher priority than re-transmissions, considering LCH priorities.

- How to achieve autonomous (re-)transmission on the deprioritized/failed PDU

Essentially, this FFS is the main point of the discussion continuation on how to achieve the joint requirements of both IIOT and NR-U after last meeting agreements, namely:

- How to address pending PDUs due to LBT failures?

- How to address pending PDUs of failed transmissions?

- How to address pending PDUs of deprioritized grants?

In accordance to the preliminary agreements in RAN2#112-e meeting, it can be seen that there may be two available roads for IIoT on unlicensed spectrum:

- NR-U road, i.e. cg-RetransmissionTimer is configured and the HARQ process ID is selected by the UE.

- IIoT road, i.e. cg-RetransmissionTimer is not configured and the HARQ process ID may be determined based on timing.

3	When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection.

4	When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection.

We cover the 3 issues listed above for each above road i.e. NR-U and IIOT in Sections 2.2.1 and 2.2.2 respectively.

- 2.2.1	NR-U road: CGRT is configured and UE selects HP ID

In NR-U, pending PDUs due to LBT failures (issue #1) as well as, when cg-RetransmissionTimer is configured with CG-DFI, pending PDUs due to transmission failures (issue #2) are already addressed by the NR-U feature of autonomous retransmission on configured grant, available when cg-RetransmissionTimer is configured [4]-[5].

Observation 1: In NR-U, pending PDUs due to LBT failures or transmission failures are already addressed by the NR-U autonomous retransmission on configured grant, as in R16.

Therefore the only remaining issue is issue#3 of pending PDUs of deprioritized grants. The simplest approach is to re-use the NR-U autonomous retransmissions on configured grants, already used for issues #1 and #2. As elaborated in [5] in a typical configuration where cg-RetransmissionTimer < configuredGrantTimer, following a CG deprioritization, most likely the gNB does not decode the partial transmission, if any, of the deprioritized PUSCH so NR-U autonomous retransmission is triggered when cg-RetransmissionTimer expires since no DFI-ACK was received. This is illustrated in Figure 1.

Figure 1: Handling of deprioritized grant with NR-U protocol

It should be noted that in RAN2#112-e meeting, it was agreed in R16 IIOT session that, for a configured grant configured with autonomousTx, the configuredGrantTimer should be stopped if the corresponding CG-PUSCH is cancelled/deprioritized [4], [6] , which could be a departure from the above behavior. However, it was also agreed at the same meeting in the R17 URLLC session that “The assumption for Rel-16 is that the network will not configure autonomousTx and cg-RetransmissionTimer simultaneously per cell.  No optimizations will be pursued to allow the two features be configured together in Rel-16.  No CR is needed for this for now”. Therefore, the R16 IIOT CR has no impact on the above analysis and the conclusions in[5] remain valid.

Observation 2: In NR-U, pending PDUs due to deprioritized CG are already addressed by the NR-U autonomous retransmission on configured grant, as in R16.

However, a similar optimization to improve the latency of an NR-U autonomous retransmission of a deprioritized PDU can be achieved by letting the NR-U protocol treating a deprioritized grant similar to an LBT failure instead of a transmission failure. And this follows the same principle as the R16 IIOT CR: upon CG deprioritization, both the configuredGrantTimer and the cg-RetransmissionTimer are stopped and the HARQ process ID is switched to “pending”. As can be seen in Figure 2, compared with Figure 1, the NR-U autonomous retransmission does not need to wait for the cg-RetransmissionTimer expiry and can use the very next CGO.

Figure 2: CG deprioritization treated similar to LBT failure

Therefore it is proposed:

Proposal 3: When both lch-basedPrioritization and cg-RetransmissionTimer are configured, upon deprioritization of an UL grant, both the configuredGrantTimer and the cg-RetransmissionTimer are stopped for the corresponding HARQ process.

Proposal 4: When both lch-basedPrioritization and cg-RetransmissionTimer are configured, upon deprioritization of an UL grant, the corresponding HARQ process switches to “pending”.

Since we think both lch-basedPrioritization and cg-RetransmissionTimer can already be configured together in R16, we provided a corresponding R16 NR-U CR in [7] capturing the above proposals.

In last meeting it was agreed:

The assumption for Rel-16 is that the network will not configure autonomousTx and cg-RetransmissionTimer simultaneously per cell.  No optimizations will be pursued to allow the two features be configured together in Rel-16.  No CR is needed for this for now.

With the above proposals, there is no need for configuring autonomousTx and cg-RetransmissionTimer altogether in R17 either, which follows the above assumption for R16.

Observation 3: With proposals 3 and 4, autonomousTx and cg-RetransmissionTimer are fully redundant in R17, as in R16.

- 2.2.2	IIOT road: CGRT is not configured and HP ID is calculated by timing

As a baseline, as in R16, the IIOT protocol handles deprioritized grants (issue #3) by autonomous transmission on a next configured grant with same HARQ process from the same configured grant configuration via configuring autonomousTx. And similarly, as in R16, UL grants which transmission failed (issue #2) are handled by legacy gNB-scheduled dynamic grants mechanism.

Observation 4: In IIOT, pending PDUs due to deprioritized UL grants or transmission failures are already addressed by the IIOT autonomous transmission and gNB-scheduled dynamic grants mechanisms, respectively, as in R16.

Therefore the only remaining issue is issue#1 of pending PDUs due to LBT failure. Here again we prevail the simplest approach consisting in re-using the available IIOT autonomous transmission mechanism. This can be simply achieved by adding LBT failure to the conditions for considering an UL grant as deprioritized as shown in Figure 3, which can be captured in Section 5.4.2.1 of TS38.321 with, for example, the below TP:

3> if a MAC PDU to transmit has been obtained: 4> if the uplink grant is not a configured grant configured with autonomousTx; or 4> if the uplink grant is a prioritized uplink grant: 5> deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process; 5> instruct the identified HARQ process to trigger a new transmission; […] 5> if the uplink grant is a configured grant configured with autonomousTx; and 5> if the transmission is performed and LBT failure indication is received from lower layers: 6> consider the uplink grant as a deprioritized uplink grant;

Figure 3: Adding LBT failure to the conditions for considering an UL grant as deprioritized

Proposal 5: When autonomousTx is configured, LBT failure is a new condition for considering an UL grant as deprioritized.

- Analysis on RAN1 options

In RAN1#103-e meeting, RAN1 listed some design options from RAN1’s perspective, i.e. regarding CG-UCI and CG-DFI procedures.

Agreements: Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16. - Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

It is interesting to compare them with the RAN2 progress so far. So we first draw some quick basic principle which we think should be worth capturing for linking the RAN2 and RAN1 features.

- 2.3.1 CG-UCI and HARQ process ID selection

It is obvious the CG-UCI only needs be configured to inform the gNB which HPID and RV UE selected for a given CGO. On the contrary, when the HPID is determined based on CGO timing, the CG-UCI is totally useless. We can express it with the following proposals:

Proposal 6: When CG-UCI is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection, i.e UE selects the HARQ process ID.

Proposal 7: When CG-UCI is not configured, Rel-16 IIOT mechanism is used for HARQ process ID and RV selection, i.e. the HARQ process ID is calculated by timing.

- 2.3.2 CGRT and CG-DFI

As discussed in Section 2.2.1, the CG-DFI procedure must be enabled to allow the cg-RetransmissionTimer to handle autonomous retransmissions upon transmission failures (no “ACK” received at cg-RetransmissionTimer expiry). This can be captured as follows:

Proposal 8: When the cg-RetransmissionTimer is configured, CG-DFI must be enabled/configured.

Note that when the cg-RetransmissionTimer is not configured, the CG-DFI could still be enabled/configured, as an optimization of the IIOT procedure. But we can take it later and keep it FFS at the moment.

- 2.3.3 RAN1 Option 1

Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

It means cg-RetransmissionTimer is one enable flag to configure both CG-UCI and CG-DFI or not. So far, we have not considered in RAN2 discussions any new parameter on top of the existing cg-RetransmissionTimer to distinguish the NR-U and IIOT roads. So this option is consistent with RAN2 status so far.

Observation 5: Option 1 in RAN1 agreement is consistent with RAN2 status so far.

- 2.3.4 RAN1 Options 2-a/2-b/3

The main common principle of these options is that, unlike option 1, CG-UCI and CG-DFI are independently enabled or disabled using distinct RRC parameters, or one explicit and one implicit parameter (option 3). Our view is that Options 2-a and 3 are special cases of Option 2-b (most generic).

Specifically, Option 2-a ties the cg-RetransmissionTimer to the CG-UCI which is consistent with above proposal 8, hence from that perspective, Option 2-a also seems viable. However, following this linkage, Option 2-a also assumes CG-DFI could be configured independently of CG-UCI, hence cg-RetransmissionTimer. So, we could have:

- CGRT configured and CG-UCI not configured, which per proposals 6-7 means UE-selected HPID and RV, which is not aligned with RAN2 agreement #3

- CGRT not configured and CG-UCI configured, which per proposals 6-7 means time-based HPID determination, which is not aligned with RAN2 agreement #4, although “may” is used for this agreement.

We see that above case #2 could be seen attractive when one wishes to take profit of the UE-selected HPID/RV for its high flexibility and latency benefits, while not configuring the cg-RetransmissionTimer on top of configuredGrantTimer. Having a closer look at this, we see that cg-RetransmissionTimer addresses 2 issues:

- Issue 1: the case when gNB could only decode the CG-UCI, then wants to send a retransmission grant for the identified HARQ process (in CG-UCI), but is unable to send the retransmission grant because of DL LBT failure(s).

- Issue 2: gNB cannot decode the CG-UCI, hence cannot identify the UE-selected HARQ process hence cannot provide feedback.

The event of issue #1 can be considered negligible in controlled environment.

For issue #2, a possible solution is as follows: gNB indicates retransmission grants for CG in DCI referring to the time-based assigned HP (e.g. reused from IIOT). When receiving the retransmission grant, the UE interpret the indicated HP to identify which HARQ buffer it refers to. And it can send the correct CG-UCI again in the DG PUSCH. In this solution, UE maintains both UE-selected and time-based HPID for each CGO and uses the former for its transmissions and the latter for identifying the retransmission grant. See Figure 4. In this approach, only CG-UCI needs to be configured, cg-RetransmissionTimer and CG-DFI do not need to be configured.

Figure 4: Allowing gNB to provide feedback or schedule a re-transmission grant for a CG PUSCH even if it can’t decode CG-UCI.

Proposal 9: RAN2 sends an LS to RAN1 suggesting to narrow down the studied options to Option 1 and Option 2-a.

Conclusion

This contribution analyzed the various options for handling IIOT operation in shared spectrum with controlled environment and concluded the following observations and proposals:

Observation 1: In NR-U, pending PDUs due to LBT failures or transmission failures are already addressed by the NR-U autonomous retransmission on configured grant, as in R16.

Observation 2: In NR-U, pending PDUs due to deprioritized CG are already addressed by the NR-U autonomous retransmission on configured grant, as in R16.

Observation 3: With proposals 3 and 4, autonomousTx and cg-RetransmissionTimer are fully redundant in R17, as in R16.

Observation 4: In IIOT, pending PDUs due to deprioritized UL grants or transmission failures are already addressed by the IIOT autonomous transmission and gNB-scheduled dynamic grants mechanisms, respectively, as in R16.

Observation 5: Option 1 in RAN1 agreement is consistent with RAN2 status so far.

Proposal 1: Support lch-based-Prioritization configured with cg-RetransmsisionTimer.

Proposal 2: When lch-basedPrioritization is configured with cg-RetransmsisionTimer initial transmissions can have higher priority than re-transmissions, considering LCH priorities.

Proposal 3: When both lch-basedPrioritization and cg-RetransmissionTimer are configured, upon deprioritization of an UL grant, both the configuredGrantTimer and the cg-RetransmissionTimer are stopped for the corresponding HARQ process.

Proposal 4: When both lch-basedPrioritization and cg-RetransmissionTimer are configured, upon deprioritization of an UL grant, the corresponding HARQ process switches to “pending”.

Proposal 5: When autonomousTx is configured, LBT failure is a new condition for considering an UL grant as deprioritized.

Proposal 6: When CG-UCI is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection, i.e UE selects the HARQ process ID.

Proposal 7: When CG-UCI is not configured, Rel-16 IIOT mechanism is used for HARQ process ID and RV selection, i.e. the HARQ process ID is calculated by timing.

Proposal 8: When the cg-RetransmissionTimer is configured, CG-DFI must be enabled/configured.

Proposal 9: RAN2 sends an LS to RAN1 suggesting to narrow down the studied options to Option 1 and Option 2-a.

Reference

- RAN2#112-e meeting report;

- RAN1#103-e meeting report;

- R2-2010836, Summary of [AT112-e][501][IIoT] Summary of URLLC in unlicensed (Qualcomm)

- 38.321-g30, Medium Access Control (MAC) protocol specification;

- R2-2008859, Co-existence of NR-U and IIOT in R16, CATT;

- R2-2011075, Configured Grant related MAC CR for IIoT, Nokia, Nokia Shanghai Bell, Ericsson, Samsung;

- R2-2100217, 38 321_CR_(Rel-16)_R2-2100217 Handling of deprioritized CG PDU when both cg-RetransmissionTimer and lch-basedPrioritization are configured, CATT