3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2102680

Online, April 12 – 20, 2021

Agenda item:	8.9.2

Source:	Qualcomm Inc

Title:	UE subgroup for paging reception

WID/SID:	NR_UE_pow_sav_enh-Core

Document for:	Discussion and Decision

# Introduction

In RAN2 #103-e, the following agreement regarding UE grouping was made

- There is support to have UE ID based enhancement - There is still significant interest to have other additional methods (but also some concerns). The approach to have a single mechanism that can take several aspects into account can be a way forward. There are still questions on the details, e.g. whether CN or RAN would provide a parameter.

This paper proposes a network-based scheme for UE subgrouping within a paging occasion.

# Discussion

In the post-RAN2#112e email discussion #64 [1], a host of proposals for UE subgrouping (e.g. paging probability, power profile, etc) were discussed. However, none of them except legacy UE-ID based approach received much support. We do not consider that surprising, because no single paging-related attribute works well in all scenarios. And often time the pros of an attribute are diminished by its equally significant cons.

Observation 1.	There is no single paging-related attribute that would work well in all scenarios for determining UE’s paging subgroup.

The above issue can be alleviated to some extent if we include multiple attributes in the determination of UE’s paging subgroup. Some of such schemes were also discussed in [1]. What we have observed is that such schemes typically require complicated designs, especially if UEs use a predefined formula to derive its paging subgroup by itself. Since UE subgrouping does not produce significant power savings, we do not think it justifies any complicated designs.

Observation 2. It can lead complicated designs if UE has to use multiple attributes to derive its paging subgroup based on predefined formula.

We therefore think it is worth considering network based approach, e.g. network configures UE’s paging subgroup based on attributes provided by UEs (e.g. [2]). This approach gives network high flexibility in assigning UE’s page subgroup and keeps UE implementation simple.

Observation 3.	It is simpler to have network configure UE’s paging subgroup, based on attributes provided by UEs.

We think there can be two possible options to implement this scheme, depend on whether core network (CN) or RAN performs the subgroup assignment. Before we discuss the details of these two options below, we make the following assumptions:

- Although UE may have different periodicity in monitoring paging occasions in RRC Idle and RRC Inactive, we do not see any clear use cases in which UE can benefit from having different paging subgroups in these two states. In addition, having different paging subgroups for RRC Idle and RRC Inactive can lead to paging failure when state mismatch occurs between UE and network (e.g. UE is in RRC Inactive but network thinks it is in RRC Idle). Therefore, we think it makes sense to assume that UE has the same paging subgroup for RRC Idle and RRC Inactive.

- It is simpler that all attributes used in the determination of paging subgroup are predefined in the spec instead of being signalled, because that can help avoid the need to coordinate which attributes are used. We do not think this requirement is too restrictive, because network (either RAN or CN) still have the flexibility in which and how attributes are used in its decision.

- CN performs the subgroup assignment

- When UE performs Registration Request with CN (e.g. AMF), it includes a set of paging-related attributes in the request. The total set of attributes can be predefined in specs, but UE needs to report only a subset of them that are of significance to it. CN then returns UE’s paging subgroup assignment in Registration Accept message. In addition, CN will forward this assignment to UE’s serving gNB as well (e.g. in a UE Context Modification message). This step is necessary because gNB needs this information when it releases UE into RRC Inactive.

- When UE is in RRC Idle and has a new page, CN sends a paging notification message to RAN as in legacy, except that it additionally includes UE’s paging subgroup number in the message. When gNB sends paging (early) indication to UE, it includes UE’s paging subgroup number in the indication.

- When UE is in RRC Inactive and has a new page from RAN, the anchor gNB sends RAN Paging message to other gNBs, which also includes UE’s paging subgroup number. This allows a gNB to include UE’s paging subgroup number when sending paging (early) indication to the UE.

The above steps are illustrated in Figure 1 below.

Figure 1. CN performs UE subgroup assignment.

- RAN performs the subgroup assignment

- When UE is in RRC Connected, it is up to UE to decide when to provide its attributes to the serving gNB. When doing that, it should be allowed to choose any subset of the predefined attributes of relevance to itself. We think a good candidate for the message UE may use to provide its attribute is UE Assistance Information.

- After the serving gNB selects a paging subgroup for the UE, it can include that information in RRC Release message when it releases UE from RRC Connected. In addition, the gNB should include UE’s subgroup assignment in its RAN Configuration Transfer message to CN. This step is necessary because CN/AMF needs this information to page UE when UE is in RRC Idle.

- When UE is in RRC Idle or RRC Inactive, the same procedures as described in A2 and A3 above can be applied.

The above steps are illustrated in Figure 2 below.

Figure 2. RAN performs subgroup assignment.

Comparing the above two options, we can see that the procedures in these two options are exactly the same when UE is in RRC Idle or RRC Inactive. And both options require enhancements to both RAN and CN. The only difference is in how UE obtains its subgroup assignment: in Option A, UE uses NAS signaling (e.g. Registration Request); whereas in Option B, UE uses RRC message (e.g. UE Assistance Information). We prefer Option A for the following reasons:

- If paging subgroup is determined by CN, UEs’ subgroup assignments are consistent cross all cells in a registration area. On the contrary, if paging subgroup is determined by RAN, different gNBs may have different proprietary implementations for the assignment. An assignment which is optimal for one cell may not work well in another, e.g. due to different mix of UEs.

- It makes sense to have paging subgroup assignment decided by the same network entity that configures other paging related configurations such as eDRX, which currently is CN.

- When UE is RRC Idle/Inactive and wants to update its paging subgroup assignment, NAS signaling can have less overhead than RRC signaling (thus lower power consumption), as no UE context needs to be setup with RAN to complete the required NAS signaling.

Observation 4. Having CN assign UEs’ paging subgroup can ensure consistent subgrouping across cells and has less signaling overhead.

Several attributes were proposed in the last meeting as potential candidate for determining subgroups. Based on the outcome of the email discussion [1], we think the following attributes can be considered:

- Paging probability [3][4][5]

- UE’s power profile [6]

- UE’s mobility profile [5]

Although these attributes are predefined in specifications, UE should not be mandated to report them, as they may be of different relevance to different UEs. It should be up to UE implementation whether to report them or which attributes to report for their subgroup assignments.

Based on the above analysis, we have the following proposals:

Proposal 1. 	Core network assigns UE’s paging subgroup. UE can provide its attributes to assist network’s assignment.

Proposal 2. 	The set of attributes UE may provide include the following: UE’s paging probability, power profile and mobility profile.

Proposal 3.	It is up to UE implement whether to provide the attributes or which attributes to provide for its subgroup assignment.

Proposal 4.	UE has the same paging subgroup assignment for both RRC Idle and RRC Inactive.

We think UE subgroup can be included in both paging early indication (PEI) and paging DCI. The reason for the latter is that some UEs may not be capable of supporting PEI, or in some scenarios UE has to skip a PEI occasion and directly monitor paging DCI. UE subgroup can still save power in those cases, especially when cross-slot scheduling is used by network.

Proposal 5.	UE subgroup can be included in both paging early indication and paging DCI.

Given the fact that UE subgrouping does not produce significant power savings, it is not necessary to support many UE subgroups per PO. As more subgroups would require more signaling bits in paging (early) indications, it is a good idea to keep the maximum number of paging subgroups small. For example, we think a maximum of no more than 4 subgroups can be considered, as it would be sufficient to handle most scenarios and yet does only at most two bits to encode. The exact number can be discussed later, after RAN1’s choice for PEI and/or enhancements to paging DCI becomes clear.

Proposal 6.	Maximum number of paging subgroups per PO is no more than 4. Exact number can be FFS, depend on RAN1’s decision on PEI and/or paging DCI enhancements.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1.	There is no single paging-related attribute that would work well in all scenarios for determining UE’s paging subgroup.

Observation 2. It can lead complicated designs if UE has to use multiple attributes to derive its paging subgroup based on some predefined formula.

Observation 3.	It is simpler to have network configure UE’s paging subgroup, based on attributes provided by UEs.

Observation 4. Having CN assign UEs’ paging subgroup can ensure consistent subgrouping across cells and has less signaling overhead.

Proposal 1. 	Core network assigns UE’s paging subgroup. UE can provide its attributes to assist network’s assignment.

Proposal 2. 	The set of attributes UE may provide include the following: UE’s paging probability, power profile and mobility profile.

Proposal 3.	It is up to UE implement whether to provide the attributes or which attributes to provide for its subgroup assignment.

Proposal 4.	UE has the same paging subgroup assignment for both RRC Idle and RRC Inactive.

Proposal 5.	UE subgroup can be included in both paging early indication and paging DCI.

Proposal 6.	Maximum number of paging subgroups per PO is no more than 4. Exact number can be FFS, depend on RAN1’s decision on PEI and/or paging DCI enhancements.

# References

- R2-2100389, Report of [POST112-e][064][Pow17] Group Determination.

- R2-2101301, Network assigned subgrouping, Intel Corporation.

- R2-2008952, Discussion on paging enhancement, Xiaomi Communications.

- R2-2010244, Paging enhancements for idle/inactive-mode UE, Huawei, HiSilicon, British Telecom.

- R2-2009878, Consideration on Idle/inactive-mode UE power saving	Lenovo, Motorola Mobility.

- R2-2010397, UE Power profile based UE subgrouping, CMCC.