3GPP TSG-RAN2 Meeting # 113bis Electronic	R2-2102715

# 12 April - 20 April, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2476 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to initiation upon reception of RAN paging and T380 Expiry |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Scenario: - UE is in RRC Connected. UE receives RRCRelease with suspend config - UE enters RRC_INACTIVE - Reset MAC - Re-establish RLC entities of SRB1 - Suspend all SRBs and DRB(s) except SRB 0 - Indicate PDCP suspend to lower layer for all DRBs - RRC initiate resumption of RRC Connection in RRC_INACTIVE and - Apply the default SRB1 config - Apply the default MAC cell group config - Apply the CCCH config - Re-establish PDCP entity for SRB1 - Resume SRB1 - Generate RRCResumeRequest/RRCResumeRequest1 and submit it to lower layers for transmission - While the resumption is ongoing, T380 expires/RAN paging is received. - RRC initiates resumption of RRC Connection again according to section 5.3.13.8/5.3.2.3 respectively and - Apply the default SRB1 config - Apply the default MAC cell group config - Apply the CCCH config - Re-establish PDCP entity for SRB1 - Resume SRB1 - Generate RRCResumeRequest/RRCResumeRequest1 and submit it to lower layers for transmission In the above scenario, upon initiation of second connection resumption, following steps are missing in current specification. - MAC reset - Release RLC entity for SRB 0 (inorder to remove old CCCH message from RLC buffer). In the absence of these steps, even though RRC has initiated connection resume again the the transmission of RRC resume from first initiation of connection resume continues in lower layers (RLC/MAC), leading to unpredictable behaviour. - gNB may discard both the messages leading to failure of connection resume. - gNB may respond to the first one while the UE is expecting response as per the latest connection resume request. Upon receving second resume request while expecting resume complete, gNB may discard the second resume request and terminate the resume procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified in 5.3.13.8/5.3.2.3 that UE resets MAC and release RLC entity for SRB 0 before initiating RRC connection resume if RRC connection resume is ongoing. Impacted functionality: Connection resume Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no interoperability issues. - If the UE is implemented according to the CR and the network is not, there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Connection resume may fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.3, 5.3.13.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of Change>

#### 5.3.2.3	Reception of the Paging message by the UE

Upon receiving the Paging message, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	forward the ue-Identity and accessType (if present) to the upper layers;

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

3> if RRC connection resumption procedure is ongoing:

4> reset MAC;

4>  release RLC entity for SRB0;

3>	if the UE is configured by upper layers with Access Identity 1:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess;

3>	else if the UE is configured by upper layers with Access Identity 2:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess;

3>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess;

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

2>	else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	forward the ue-Identity to upper layers and accessType (if present) to the upper layers;

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

#### <End of Change>

#### <Start of Change>

#### 5.3.13.8	RNA update

In RRC_INACTIVE state, the UE shall:

1>	if T380 expires; or

1>	if RNA Update is triggered at reception of SIB1, as specified in 5.2.2.4.2:

2> if RRC connection resumption procedure is ongoing:

3> reset MAC;

3>  release RLC entity for SRB0;

2>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to rna-Update;

1>	if barring is alleviated for Access Category '8', as specified in 5.3.14.4:

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRNA-Update is set to true:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause value set to rna-Update.

If the UE in RRC_INACTIVE state fails to find a suitable cell and camps on the acceptable cell to obtain limited service as defined in TS 38.304 [20], the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

NOTE:	It is left to UE implementation how to behave when T380 expires while the UE is camped neither on a suitable nor on an acceptable cell.

#### <End of Change>